September 24, 1998
DRAFT

Insurance Institute for Highway Safety

Crashworthiness Evaluation

Crash Test Report

1999 Pontiac Grand Am (CF99026)
Vehicle identification number:
1G2NE52T7XC579674
Body style:
Midsize four-door sedan
Engine/transmission:
Transverse 2.4-liter 4-cylinder, 4-speed automatic,
front-wheel drive
Standard crashworthiness features: 

Driver and right front passenger airbags

Dual-locking shoulder belts (front and rear outboard seating positions)

Shoulder belt upper anchorage height adjusters (front seating positions only)

Seat belt force-limiting mechanisms (front seating positions only)

Other standard safety features:

Four-wheel antilock brakes

Daytime running lamps

Vehicle specifications (provided by manufacturer):

Wheelbase:
272 cm

Overall length:
474 cm

Overall width:
179 cm

Curb weight:
1,413 kg

Vehicle specifications (measured):

Front bumper to firewall:
128 cm
Curb weight:
1,397 kg
Test weight:
1,513 kg (61% front, 39% rear)
Overall width:
179 cm

Nominal test parameters:
40.0 mi/h (64.4 km/h), 40 percent overlap, deformable barrier face with slotted bumper

Crash test date:
October 26, 1999
Figure 1

Precrash and Postcrash Side Views — 1999 Pontiac Grand Am
[image: image1.png]



[image: image2.png]ST e
1999 GRAND AM

a8 INSURANCE INSTITUTE
g FOR HIGHWAY SAFETY

m—" A

CF9026 NN ] _d





Summary

A 1999 Pontiac Grand Am was crash tested on October 26, 1999 into a fixed deformable barrier at 39.9 mi/h (64.2 km/h) and a 41 percent overlap on the driver side.  A Hybrid III 50th percentile male dummy was positioned in the driver seat with the lap/shoulder belt fastened.

Measures of intrusion taken after the crash indicated the lower instrument panel in front of the dummy moved rearward 10-13 cm.  Resultant intrusion in the driver footwell measured 23 cm at the footrest and 18-29 cm at other places on the toepan.  All doors remained closed during the crash.  After the crash, the driver and left rear doors required tools to open, and the right front and right rear doors opened with ease.  The floorpan was buckled sharply upward just in front of the driver seat, and the welded seam joining the floorpan to the door sill was torn open.

The driver dummy was restrained by a three-point lap/shoulder belt and an airbag.  During the crash, 6 cm of webbing spooled off the retractor.  The airbag contacted the dummy’s face during deployment.  During rebound from the airbag, the dummy’s head leaned outward partway past the precrash plane of the open side window and moved downward toward the driver door, which had bowed outward.  The left rear part of the head contacted the window sill.  The head then moved rearward and upward, and the center rear of the head contacted the leading edge of the
B-pillar and shoulder belt upper anchorage D-ring.  After the crash, the upper end of the steering column had moved upward 14 cm and rearward 6 cm. 

The maximum resultant head acceleration was 88 g from the B-pillar and shoulder belt D-ring contact and 26 g from the window sill contact.  The right leg had a maximum tibia axial force of –4.0 kN and a maximum lower tibia L-M moment of –228 Nm, which contributed to a lower tibia index of 1.14.
Test Conditions

This vehicle had been tested previously in the Institute’s Low-Speed Crash Test Program and subjected to an impact on the front corner of the passenger side at 5 mi/h (8 km/h) into a 
30 degree angle barrier and a rear impact at 5 mi/h (8 km/h) into a flat barrier.  All structural damage on the front was repaired prior to this test (see Appendix, Low-Speed Crash Test Damage Repair Estimate).

This test was conducted according to the procedures specified in the IIHS Offset Barrier Crash Test Protocol (Version VII).  The Hybrid III dummy positioned in the driver seat was equipped with instrumented lower legs that included feet modified to include two accelerometers and to have a 45 degree dorsiflexion range with soft stops at all extremes of foot-ankle motion.  All dummy seating parameters were set according to the procedures specified for Federal Motor Vehicle Safety Standard 208 compliance testing (49 CFR Part 571.208 § 11).  The dummy’s left foot was placed on the footrest.

Seat back, shoulder belt upper anchorage, and steering column adjustments were set according to the manufacturer’s specifications for government crash testing.  Other adjustments were set according to the procedure specified for Federal Motor Vehicle Safety Standard 208 compliance testing (49 CFR Part 571.208 § 7 and 8).  After final positioning of the dummy, measurements from various parts of the dummy to a number of vehicle interior points were made.  These measurements and the seat back, shoulder belt upper anchorage, and steering column adjustments are described in the Appendix, Dummy Clearance Measurements. 

Vehicle acceleration measurements were made by a triaxial arrangement of accelerometers mounted on the vehicle’s longitudinal centerline and 63 cm behind its center of gravity (170 cm behind the front axle).  The vehicle speed recorded just prior to impact was 39.9 mi/h (64.2 km/h), and the actual overlap was 41 percent.

Structural Performance

All doors remained closed during the crash.  The driver door aperture shortened 6 cm, as measured at the lower edge of the window.  After the crash, the driver door required extensive prying with tools to open, the left rear door required light prying with tools to open, and the right front and right rear doors opened with ease.

The floorpan was buckled sharply upward just in front of the driver seat, and the welded seam joining the floorpan to the door sill was torn open.  The tear was about 32 cm long and had a maximum gap width of 4 cm.

No fuel system leaks were observed after the crash.  In addition, no fuel system leaks were observed when the vehicle was rotated onto its right side to allow postcrash photography.
Figure 2 shows the overhead view of the crash deformation.  Figure 3 illustrates the precrash and postcrash contour measures of the front bumper cover profile and the resulting permanent crush.  Figure 4 shows the precrash and postcrash views from below.  Figure 5 illustrates the deformation of the side rails, door sills, and engine cradle, which are visible in Figure 4.

Figure 2

Overhead View of Crash Deformation — 1999 Pontiac Grand Am
[image: image3.png]9206649

JIHIIIIILIIIIEEII I LI 1110)

ALIYS AYMHIIH 404
JUNLLLSNI DNVANSNI

IILI1LIILLILLIL LI e

NV aNvHD 6661





Figure 3

Front Bumper Cover Crush Contour — 1999 Pontiac Grand Am
[image: image4.wmf]C

11

C

10

C

9

C

8

C

7

C

6

C

5

C

4

C

3

C

2

C

1

-5

15

35

55

75

95

-80

-60

-40

-20

0

20

40

60

80

Lateral Distance from Vehicle Centerline (cm)

Distance Rearward from Front of Vehicle (cm)

Precrash

Postcrash



C1
C2
C3
C4
C5
C6
C7
C8
C9
C10
C11

Postcrash
Contour (cm)
92
83
75
71
61
53
45
31
20
12
–2

Precrash
Contour (cm)
15
7
3
1
0
0
0
1
3
7
15

Resulting
Crush (cm)
77
76
72
70
61
53
45
30
17
5
–17

The bumper cover was torn almost completely off the vehicle during the crash.  The postcrash contour represents the best attempt to fit the cover back onto the vehicle.  The length of the reference line was 140 cm precrash and 130 cm postcrash.

Figure 4

Precrash and Postcrash Views from Below — 1999 Pontiac Grand Am
[image: image5.png]



[image: image6.png]



Figure 5

Structural Deformation, Views from Below and Side — 1999 Pontiac Grand Am
[image: image7.wmf]-80

-60

-40

-20

0

20

40

60

80

0

20

40

60

80

100

120

140

160

180

200

220

240

260

280

300

320

340

Longitudinal Distance from Foremost Point on Vehicle (cm)

Lateral Distance from Centerline (cm)

Front Bumper Mount

Precrash Side Rail

A-Pillar Location

Postcrash Side Rail

B-Pillar Location

Precrash Door Sill

Precrash Engine Cradle

Postcrash Door Sill

Postcrash Engine Cradle

Left Side

Centerline

Right Side


[image: image8.wmf]-40

-20

0

20

0

20

40

60

80

100

120

140

160

180

200

220

240

260

280

300

320

340

Longitudinal Distance from Foremost Point on Vehicle (cm)

Vertical Position (cm)

Front Bumper Mount

Precrash Side Rails

A-Pillar Location

Postcrash Side Rail

B-Pillar Location

Precrash Door Sills

Postcrash Right Door Sill

Postcrash Left Door Sill

Right

Left

Both


Various measures of intrusion were made after the crash.  These residual measures of intrusion typically are less than the maximum deformation that occurs during the crash.  The coordinate reference system for these measures is described in the IIHS Offset Barrier Crash Test Protocol (Version VII).  The measures of deformation shown in Table 1 have been adjusted to better reflect the displacement of the various target locations relative to the driver, based on the locations of the four driver seat-attachment bolts.  The average displacement of the seat-attachment bolts relative to the reference system also is shown in Table 1.

Table 1
Residual Measurements of Intrusion Relative to Driver Seat — 
1999 Pontiac Grand Am

Selected Locations*
Longitudinal
Lateral
Vertical
Resultant

Steering column (cm)
–6
–3
14
16

Left lower instrument panel (cm)
–13
–4
10
17

Right lower instrument panel (cm)
–10
–4
10
14

Brake pedal (cm)
–19
–5
3
20

Left toepan (cm)
–27
–8
5
29

Center toepan (cm)
–26
–7
6
28

Right toepan (cm)
–18
–4
2
18

Footrest (cm)
–22
–6
3
23

Average displacement of the four seat-attachment bolts relative to reference system (cm)
0
2
–1
n/a

* All measurements taken on driver side.  From the driver’s position, positive is forward, left, and up.

Restraint System Performance

Airbags

Driver:  The uninflated driver airbag is approximately 70 cm in diameter and is untethered.  The airbag is vented by two holes located at positions corresponding to 10 and 2 o’clock on the forward-facing surface of the airbag.  Analysis of the high-speed film taken from camera position E indicated the airbag deployed at 48 ms into the crash and appeared to be fully inflated at 70 ms. 

Passenger:  The top-mounted passenger airbag deployed vertically, and the excursion of its center when inflated is limited by two tethers.  The cylinder-shaped airbag has no vent holes but is made of porous fabric.  The airbag did not contribute to windshield damage during deployment. 

Seat Belts

This vehicle is equipped with dual-locking lap/shoulder belts with sliding latch plates, adjustable upper anchorage points, and mechanical force-limiting mechanisms at both front seating positions.  The front inboard lower anchorage points are attached to and move with the seats.  The front outboard lower anchorage points are bolted to the sills at the base of the B-pillars.  During the crash, 6 cm of webbing was pulled from the retractor through the D-ring, as measured by a pull-string mounted between the retractor housing and the webbing beyond the retractor.  Postcrash investigation of the force-limiting sliding mount to which the retractor mechanism is attached indicated it was essentially unchanged from its precrash orientation.  Therefore, the force limiter did not contribute any webbing to the total amount pulled though the D-ring.

Seat

Postcrash examination of the driver seat rails indicated no discernible movement of the seat in its tracks during the crash.  Despite the sharp buckle in the floor ahead of the driver seat, there was little obvious pitching or tipping of the seat.

Steering Column

The upper end of the steering column moved upward 14 cm and rearward 6 cm relative to the driver seat.

Dummy Kinematics

Head, Neck, and Torso

Analysis of the high-speed film taken from camera position E indicated the deploying airbag contacted the dummy’s face at 54 ms into the crash.  The dummy’s face began to load the airbag at 86 ms (16 ms after the airbag was fully inflated).  Paint transferred from the dummy’s face indicated the nose contacted the fully inflated airbag 8 cm above and 11 cm to the left of its center.  During rebound from the airbag, the dummy’s head leaned outward partway past the precrash plane of the open side window and moved downward toward the driver door, which had bowed outward.  The left rear part of the head contacted the window sill 25 cm forward of the 
B-pillar at 153 ms.  The head then moved rearward and upward, and the center rear of the head contacted the leading edge of the B-pillar and shoulder belt upper anchorage D-ring at 235 ms.  Table 2 provides the timing of these events.

Table 2
Restraint System Performance and Dummy Kinematics —
1999 Pontiac Grand Am

Event
Time (ms)

Deployment of airbag
48

Airbag contacts face during deployment
54

Airbag fully inflated
70

Face begins to load airbag
86

Left rear of head contacts window sill
153

Center rear of head contacts B-pillar
235

Figure 6

Dummy and Vehicle Interior, Postcrash — 1999 Pontiac Grand Am
[image: image9.png]



Legs and Feet

Left leg and foot:  Paint transferred from the dummy’s left knee indicated the knee contacted the knee bolster 1 cm directly to the right of the left instrument panel intrusion reference point.  Paint transferred from the dummy’s left shin indicated the shin contacted the bolster directly below the knee impact location.  The left foot was found fully dorsiflexed and slightly inverted, with the sole of the forefoot and heel against the intruded toepan to the right of the footrest and the back of the heel against a sharp downward buckle in the floorpan.

Right leg and foot:  Paint transferred from the dummy’s right knee indicated the knee contacted the knee bolster about 4 cm below and 6 cm to the left of the right instrument panel intrusion reference point.  Paint transferred from the dummy’s right shin indicated the shin contacted the bolster directly below the knee impact location.  The right foot was found fully dorsiflexed and considerably everted, with the back of the heel against a sharp downward buckle in the floormat/carpeting on the floorpan.  The medial sole was pressed against the intruded toepan, and the lateral sole at the instep was pressed against the accelerator pedal, which in turn was almost fully depressed against the intruded toepan.  The lateral edge of the sole near the toe was against a steel brace to the right of the accelerator pedal.

Dummy Injury Measures

Head

The HICs were calculated during an interval that corresponds with the dummy’s head excursion into the airbag, and the maximum vector resultant head accelerations were recorded when the dummy’s head contacted the B-pillar and shoulder belt upper anchorage D-ring.  The peak resultant accelerations were 88 g at 238 ms from the B-pillar contact and 26 g at 160 ms from window sill contact.  Table 3 provides a summary of the maximum head injury measurements recorded during the crash.
Table 3
Head Injury Measurements — 1999 Pontiac Grand Am



Measure
Published Tolerance Threshold


Result


Time (ms)

Vector resultant acceleration (g)
80
88

238

Vector resultant acceleration — 3 ms clip (g)
80
64

237-240

Head Injury Criterion (HIC)
1000
514

77-113

Head Injury Criterion — 15 ms interval (HIC-15)*
700
331

88-103

* Canadian Motor Vehicle Safety Regulations (Standard 208) allow the resultant head acceleration to exceed 80 g in airbag-equipped vehicles if HIC-15 is less than 700 (Transport Canada, 1998).
Neck

Table 4 provides a summary of the maximum neck injury measurements recorded during the crash.  None of the recorded neck force measures exceeded the magnitude-duration injury criteria (Figures A-13 to A-16).

Table 4
Neck Injury Measurements — 1999 Pontiac Grand Am



Measure
Published Tolerance Threshold


Result


Time (ms)

A-P shear force (kN)
±3.1

–0.6

74


Axial compression force (kN)
4.0

0.1

241


Axial tension force (kN)
3.3

1.7

86


Flexion bending moment (Nm)
310

22

263


Extension bending moment (Nm)
122

26

117


Chest

Table 5 provides a summary of the maximum chest injury measurements recorded during the crash.

Table 5
Chest Injury Measurements — 1999 Pontiac Grand Am



Measure
Published Tolerance Threshold


Result


Time (ms)

Vector resultant spine acceleration — 3 ms clip (g)
60
38

105-108

Rib compression (mm)
–50
–27

100

Viscous criterion (m/s)
1.0
0.1

80

Legs and Feet

Left leg and foot:  None of the injury measures exceeded the published threshold values.

Right leg and foot:  The right leg had a maximum tibia axial force of –4.0 kN at 82 ms and a maximum lower tibia L-M moment of –228 Nm at 82 ms, which contributed to a lower tibia index of 1.14 at 82 ms. 

The upper tibia A-P bending moment had a time signature similar to the axial force (Figures 
A-40 and A-45).  The ratio of the A-P bending moment to the axial force (approximately 0.02 m) is slightly less than the distance (in meters) between a line through the knee and ankle joints and the upper load cell midline due to the bent shape of the dummy’s tibia.  Therefore, the upper tibia bending could have resulted from the foot force acting through the moment arm of the tibia bend rather than from transverse forces applied by contact with the knee bolster.  Consequently, the upper tibia resultant moment was not calculated, and the upper tibia index was calculated from only the upper tibia L-M moment and tibia axial force.

Table 6 provides a summary of the maximum leg and foot injury measurements recorded during the crash.

Table 6
Leg and Foot Injury Measurements — 1999 Pontiac Grand Am



Measure
Published Tolerance Threshold
Left
Right




Result
Time
(ms)

Result
Time
(ms)

Femur axial force (kN)
–9.1
*
–4.2
101
–2.8
89

Tibia-femur displacement (mm)
–15

–2
116
0
87

Upper Tibia







L-M moment (Nm)
±225

–34
48
–139
82

A-P moment (Nm)
±225

–92
84
**
n/a

Vector resultant moment (Nm)
225

92
84
**
n/a

Index
1.00

0.48
84
0.73
82

Lower Tibia







L-M moment (Nm)
±225
***
67
73
–228
82

A-P moment (Nm)
±225
***
34
98
47
84

Vector resultant moment (Nm)
225
***
69
73
232
82

Axial force (kN)
–8.0
***
–2.8
74
–4.0
82

Index
1.00

0.38
73
1.14
82

Foot







A-P acceleration (g)
±150

–80
71
–56
71

I-S acceleration (g)
±150

–72
82
–51
82

Vector resultant acceleration (g)
150

90
70
67
71

*** This critical value is for instantaneous loading.  Femur loads are compared with magnitude-duration injury criteria in Figures A-23 and A-34.

*** The upper tibia A-P bending could have resulted from the foot force acting through the moment arm of the tibia bend rather than from transverse forces applied by contact with the knee bolster; therefore the A-P and resultant moments are not indicated.  The listed upper tibia index was calculated from only the upper tibia L-M moment and tibia axial force.

*** These published thresholds are for fractures of the tibia.  Ankle and foot injuries have been associated with bending moments as low as 50-100 Nm, and heel fractures have been associated with axial forces as low as –6.0 kN.

References

Backaitis, S.H. and Mertz, H.J. (eds). 1994. Hybrid III: The First Human-Like Crash Test Dummy. Warrendale, PA: Society of Automotive Engineers.

Begeman, P.C. and Prasad, P. 1990. Human ankle impact response in dorsiflexion (SAE 902308). Thirty-fourth Stapp Car Crash Conference Proceedings, 39-53. Warrendale, PA: Society of Automotive Engineers.

Begeman, P.; Balakrishnan, P.; Levine, R.; and King, A. 1993. Dynamic human ankle response to inversion and eversion (SAE 933115). Thirty-seventh Stapp Car Crash Conference Proceedings, 83-93. Warrendale, PA: Society of Automotive Engineers.

Mertz, H.J. and Patrick, L.M. 1971. Strength and response of the human neck (SAE 710855). Biomechanics of Impact Injury and Injury Tolerances of the Head-Neck Complex, 821-46. Warrendale, PA: Society of Automobile Engineers.

Parenteau, C.S. 1995. Foot-ankle injury: epidemiology and method to investigate joint biomechanics. Gothenburg, Sweden: Chalmers University of Technology.

Prasad, P. and Mertz, H.J. 1985. The position of the United States delegation to the ISO Working Group 6 on the use of HIC in the automotive environment (SAE 851246). Biomechanics of Impact Injury and Injury Tolerances of the Head-Neck Complex, 373-83. Warrendale, PA: Society of Automotive Engineers.

Transport Canada. 1998. Motor Vehicle Safety Regulations – Canadian Motor Vehicle Safety Standards, Schedule IV Part III Standard 208, Occupant Restraint Systems in Frontal Impact. Ottawa, Ontario.

Welbourne, E.R. 1994. Vehicle performance requirements for head injury protection: a comparison of the head injury criterion with an 80 g limit on resultant acceleration. Technical Memorandum. Ottawa, Ontario: Transport Canada, Vehicle System Division.

Zeidler, F. 1984. The significance of lower limb injuries of belted drivers. Journal of Orthopedics [German].

Appendix

Low-Speed Crash Test Damage Repair Estimate
Dummy Clearance Measurements

Graph Index — index to graphs of time plots of dummy and vehicle data

Manufacturer’s window sticker

Low-Speed Crash Test Damage Repair Estimate
1999 Pontiac Grand Am Midsize Four-Door Sedan:  5 mi/h Front into Angle Barrier

Test Number:  LA99015

VIN:  1G2NE52T7XC579674
Mileage:  96

Features:  Driver and passenger airbags, four-wheel antilock brakes, air conditioning, automatic transmission, power mirrors, keyless entry system, power door locks, power windows, power driver seat height adjustment, heated back glass, tilt steering wheel, cruise control, front fog lamps, two-stage paint.

Part

Labor

Description
Mfg. No
Price

Operation
Hours








Unibody, front



Set up*
1.0

Bumper reinforcement, front 
22594042
$167.81

Replace*
2.8

Fog lamp assembly, left



Remove/reinstall


Fog lamp assembly, right



Remove/reinstall


Headlamp assembly, right
16526011
221.00

Replace
0.3

Headlamp mounting panel
22639967
151.53

Replace
0.8

Headlamps



Aim
0.5

Fender, right front
22605648
209.50

Replace**
1.9

Fender, right front



Refinish
3.4

Frame sidemember, right front



Repair/align*
1.0

Frame sidemember, right front



Refinish
0.5

End plate, frame sidemember, right front



Repair/align*
0.5

Horn



Remove/reinstall
0.2

Molding, right front fender



Remove/reinstall
0.1

Deflector, rocker panel, right



Remove/reinstall


Paint and materials

70.20











Total Parts

$820.04











Total Labor

442.00


13.0








Grand Total

$1,262.04











* This item was repaired or replaced as indicated before the 40 mi/h frontal offset test.
** This cosmetic part was repaired rather than replaced before the 40 mi/h frontal offset test.

Dummy Clearance Measurements

Test Number:
CF99026
Vehicle Make/Model:
Pontiac Grand Am
Vehicle Model Year:
1999
Seat Type:
Bucket seat with manual fore/aft and seat back angle adjustments and electric height adjustment

Manufacturer’s Specifications

Seat Back Information:
Reclined to 27 degrees from vertical, as measured on outboard rear frame of seat back at adjuster hinge

Upper Belt Anchorage:
Set to 2nd position from top

Steering Column Adjustment:
Set to midpoint of 5 positions

Location 
Code
Measure
Location 
Code
Measure








Head to header
HH
255
Neck angle, torso 90
NAT90
21.2(

Head to windshield
HW
546
Neck angle, seated*
NAS
4.4(

Nose to rim
NR
374
Torso angle (NAT90 – NAS)
TA
16.8(

Chest to dash
CD
588
Striker to knee**
SK
586

Rim to abdomen
RA
200
Striker to knee angle**
SKA
–1.7(

Knee to dash, left
KDL
225
Striker to H-point, horizontal
SHH
198

Knee to dash, right
KDR
175
Striker to H-point, vertical
SHV
126

Steering wheel to chest, horizontal
SCH
308
Ankle to ankle
AA
295

Steering wheel to chest, perpendicular
SCP
387
Knee to knee
KK
345

Steering wheel to chest, reference
SCR
370
Arm to door
AD
128

Hub to chest, minimum
HCM
251
H-point to door
HD
170

Pelvic angle
PA
24.6(
Head to A-pillar
HA
504

Seat back angle
SA
27(
Head to roof
HR
180




Head to side window
HS
237








** All distance measurements are in millimeters (mm).

** Dummy’s neck bracket was adjusted to –3.5 degrees to achieve a level instrumentation plane.
** These measurements were made in a vertical plane containing the striker and parallel to the driver door sill.

[image: image10.wmf]
Graph Index
A-1
Vehicle longitudinal acceleration (X)

A-2
Vehicle lateral acceleration (Y)

A-3
Vehicle vertical acceleration (Z)

A-4
Vehicle vector resultant acceleration

A-5
Integration of vehicle longitudinal acceleration (X)

A-6
Head A-P acceleration (X)

A-7
Head L-M acceleration (Y)

A-8
Head I-S acceleration (Z)

A-9
Head vector resultant acceleration

A-10
Neck A-P force (X)

A-11
Neck axial force

A-12
Neck occipital A-P bending moment

A-13
Neck tension — force by duration analysis

A-14
Neck compression — force by duration analysis

A-15
Neck shear (positive) — force by duration analysis

A-16
Neck shear (negative) — force by duration analysis

A-17
Chest compression 

A-18
Chest A-P acceleration (X)

A-19
Chest lateral acceleration (Y)

A-20
Chest I-S acceleration (Z)  

A-21
Chest vector resultant acceleration

A-22
Chest viscous criterion

A-23
Left femur axial force

A-24
Left femur — force by duration analysis

A-25
Left tibia-femur displacement

A-26
Left upper tibia L-M bending moment

A-27
Left upper tibia A-P bending moment

A-28
Left upper tibia vector resultant bending moment

A-29
Left lower tibia L-M bending moment

A-30
Left lower tibia A-P bending moment

A-31
Left lower tibia vector resultant bending moment

A-32
Left lower tibia axial force

A-33
Left foot vector resultant acceleration

A-34
Left foot A-P acceleration

A-35
Left foot I-S acceleration

A-36
Right femur axial force

A-37
Right femur — force by duration analysis

A-38
Right tibia-femur displacement

A-39
Right upper tibia L-M bending moment 

A-40
Right upper tibia A-P bending moment

A-41
Right upper tibia vector resultant bending moment

A-42
Right lower tibia L-M bending moment

A-43
Right lower tibia A-P bending moment

A-44
Right lower tibia vector resultant bending moment

A-45
Right lower tibia axial force

A-46
Right foot vector resultant acceleration

A-47
Right foot A-P acceleration

A-48
Right foot I-S acceleration

[image: image11.wmf]A- 1 CF99026 1999 Pontiac Grand Am Vehicle Longitudinal Acceleration

-40

-35

-30

-25

-20

-15

-10

-5

0

5

10

-0.05

0.00

0.05

0.10

0.15

0.20

0.25

0.30

0.35

Time (s)

Acceleration (g)


[image: image12.wmf]A- 2 CF99026 1999 Pontiac Grand Am Vehicle Lateral Acceleration

-10

-5

0

5

10

15

20

25

-0.05

0.00

0.05

0.10

0.15

0.20

0.25

0.30

0.35

Time (s)

Acceleration (g)


[image: image13.wmf]A- 3 CF99026 1999 Pontiac Grand Am Vehicle Vertical Acceleration

-15

-10

-5

0

5

10

15

20

-0.05

0.00

0.05

0.10

0.15

0.20

0.25

0.30

0.35

Time (s)

Acceleration (g)


[image: image14.wmf]A- 4 CF99026 1999 Pontiac Grand Am Vehicle Vector Resultant Acceleration

0

5

10

15

20

25

30

35

40

-0.05

0.00

0.05

0.10

0.15

0.20

0.25

0.30

0.35

Time (s)

Acceleration (g)


[image: image15.wmf]A- 5 CF99026 1999 Pontiac Grand Am Integration of Vehicle Longitudinal 

Acceleration

-10.0

0.0

10.0

20.0

30.0

40.0

50.0

60.0

70.0

-0.05

0.00

0.05

0.10

0.15

0.20

0.25

0.30

0.35

Time (s)

km/h


[image: image16.wmf]A- 6 CF99026 1999 Pontiac Grand Am Head A-P Acceleration

-80

-60

-40

-20

0

20

40

60

80

100

-0.05

0.00

0.05

0.10

0.15

0.20

0.25

0.30

0.35

Time (s)

Acceleration (g)


[image: image17.wmf]A- 7 CF99026 1999 Pontiac Grand Am Head L-M Acceleration

-20

-15

-10

-5

0

5

10

15

20

25

-0.05

0.00

0.05

0.10

0.15

0.20

0.25

0.30

0.35

Time (s)

Acceleration (g)


[image: image18.wmf]A- 8 CF99026 1999 Pontiac Grand Am Head I-S Acceleration

-5

0

5

10

15

20

25

-0.05

0.00

0.05

0.10

0.15

0.20

0.25

0.30

0.35

Time (s)

Acceleration (g)


[image: image19.wmf]A- 9 CF99026 1999 Pontiac Grand Am Head Vector Resultant Acceleration

0

10

20

30

40

50

60

70

80

90

100

-0.05

0.00

0.05

0.10

0.15

0.20

0.25

0.30

0.35

Time (s)

Acceleration (g)


[image: image20.wmf]A- 10 CF99026 1999 Pontiac Grand Am Neck A-P Shear Force

-600

-500

-400

-300

-200

-100

0

100

200

300

-0.05

0.00

0.05

0.10

0.15

0.20

0.25

0.30

0.35

Time (s)

Force (N)


[image: image21.wmf]A- 11 CF99026 1999 Pontiac Grand Am Neck Axial Force

-500

0

500

1000

1500

2000

-0.05

0.00

0.05

0.10

0.15

0.20

0.25

0.30

0.35

Time (s)

Force (N)


[image: image22.wmf]A- 12 CF99026 1999 Pontiac Grand Am Neck Occipital A-P Moment

-30

-20

-10

0

10

20

30

-0.05

0.00

0.05

0.10

0.15

0.20

0.25

0.30

0.35

Time (s)

Moment (Nm)


[image: image23.wmf]A- 13 CF99026 1999 Pontiac Grand Am Neck Tension Analysis

0

500

1000

1500

2000

2500

3000

3500

0.000

0.005

0.010

0.015

0.020

0.025

0.030

0.035

0.040

0.045

0.050

Duration (s)

Force (N)


[image: image24.wmf]A- 14 CF99026 1999 Pontiac Grand Am Neck Compression Analysis

0

500

1000

1500

2000

2500

3000

3500

4000

4500

0.000

0.005

0.010

0.015

0.020

0.025

0.030

0.035

0.040

Duration (s)

Force (N)


[image: image25.wmf]A- 15 CF99026 1999 Pontiac Grand Am Neck A-P Shear (Positive) Analysis

0

500

1000

1500

2000

2500

3000

3500

0.000

0.005

0.010

0.015

0.020

0.025

0.030

0.035

Duration (s)

Force (N)


[image: image26.wmf]A- 16 CF99026 1999 Pontiac Grand Am Neck A-P Shear (Negative) Analysis

0

500

1000

1500

2000

2500

3000

3500

0.000

0.005

0.010

0.015

0.020

0.025

0.030

0.035

Duration (s)

Force (N)


[image: image27.wmf]A- 17 CF99026 1999 Pontiac Grand Am Chest Compression

-30

-25

-20

-15

-10

-5

0

5

-0.05

0.00

0.05

0.10

0.15

0.20

0.25

0.30

0.35

Time (s)

Compression (mm)


[image: image28.wmf]A- 18 CF99026 1999 Pontiac Grand Am Chest A-P Acceleration

-40

-35

-30

-25

-20

-15

-10

-5

0

5

10

-0.05

0.00

0.05

0.10

0.15

0.20

0.25

0.30

0.35

Time (s)

Acceleration (g)


[image: image29.wmf]A- 19 CF99026 1999 Pontiac Grand Am Chest L-M Acceleration

-4

-2

0

2

4

6

8

10

12

14

-0.05

0.00

0.05

0.10

0.15

0.20

0.25

0.30

0.35

Time (s)

Acceleration (g)


[image: image30.wmf]A- 20 CF99026 1999 Pontiac Grand Am Chest I-S Acceleration

-6

-4

-2

0

2

4

6

8

-0.05

0.00

0.05

0.10

0.15

0.20

0.25

0.30

0.35

Time (s)

Acceleration (g)


[image: image31.wmf]A- 21 CF99026 1999 Pontiac Grand Am Chest Vector Resultant Acceleration

0

5

10

15

20

25

30

35

40

45

-0.05

0.00

0.05

0.10

0.15

0.20

0.25

0.30

0.35

Time (s)

Acceleration (g)


[image: image32.wmf]A- 22 CF99026 1999 Pontiac Grand Am Viscous Criterion

-0.2

-0.1

0.0

0.1

0.2

-0.05

0.00

0.05

0.10

0.15

0.20

0.25

0.30

0.35

Time (s)

m/s


[image: image33.wmf]A- 23 CF99026 1999 Pontiac Grand Am Left Femur Axial Force

-5000

-4000

-3000

-2000

-1000

0

1000

-0.05

0.00

0.05

0.10

0.15

0.20

0.25

0.30

0.35

Time (s)

Force (N)


[image: image34.wmf]A- 24 CF99026 1999 Pontiac Grand Am Left Femur Axial Force Analysis

0

2000

4000

6000

8000

10000

12000

0.000

0.002

0.004

0.006

0.008

0.010

0.012

0.014

0.016

0.018

0.020

Duration (s)

Force (N)

Good

Acceptable



Marginal

Poor


[image: image35.wmf]A- 25 CF99026 1999 Pontiac Grand Am Left Tibia-Femur Displacement

-2

-1

0

1

2

-0.05

0.00

0.05

0.10

0.15

0.20

0.25

0.30

0.35

Time (s)

Displacement (mm)


[image: image36.wmf]A- 26 CF99026 1999 Pontiac Grand Am Left Upper Tibia L-M Moment

-40

-30

-20

-10

0

10

20

30

40

-0.05

0.00

0.05

0.10

0.15

0.20

0.25

0.30

0.35

Time (s)

Moment (Nm)


[image: image37.wmf]A- 27 CF99026 1999 Pontiac Grand Am Left Upper Tibia A-P Moment

-100

-80

-60

-40

-20

0

20

40

-0.05

0.00

0.05

0.10

0.15

0.20

0.25

0.30

0.35

Time (s)

Moment (Nm)


[image: image38.wmf]A- 28 CF99026 1999 Pontiac Grand Am Left Upper Tibia Vector Resultant Moment

0

10

20

30

40

50

60

70

80

90

100

-0.05

0.00

0.05

0.10

0.15

0.20

0.25

0.30

0.35

Time (s)

Moment (Nm)


[image: image39.wmf]A- 29 CF99026 1999 Pontiac Grand Am Left Lower Tibia L-M Moment

-80

-60

-40

-20

0

20

40

60

80

-0.05

0.00

0.05

0.10

0.15

0.20

0.25

0.30

0.35

Time (s)

Moment (Nm)


[image: image40.wmf]A- 30 CF99026 1999 Pontiac Grand Am Left Lower Tibia A-P Moment

-40

-30

-20

-10

0

10

20

30

40

-0.05

0.00

0.05

0.10

0.15

0.20

0.25

0.30

0.35

Time (s)

Moment (Nm)


[image: image41.wmf]A- 31 CF99026 1999 Pontiac Grand Am Left Lower Tibia Vector Resultant Moment

0

10

20

30

40

50

60

70

80

-0.05

0.00

0.05

0.10

0.15

0.20

0.25

0.30

0.35

Time (s)

Moment (Nm)


[image: image42.wmf]A- 32 CF99026 1999 Pontiac Grand Am Left Lower Tibia Axial Force

-3000

-2500

-2000

-1500

-1000

-500

0

500

1000

-0.05

0.00

0.05

0.10

0.15

0.20

0.25

0.30

0.35

Time (s)

Force (N)


[image: image43.wmf]A- 33 CF99026 1999 Pontiac Grand Am Left Foot Vector Resultant Acceleration

0

10

20

30

40

50

60

70

80

90

100

-0.05

0.00

0.05

0.10

0.15

0.20

0.25

0.30

0.35

Time (s)

Acceleration (g)


[image: image44.wmf]A- 34 CF99026 1999 Pontiac Grand Am Left Foot A-P Acceleration

-100

-80

-60

-40

-20

0

20

40

60

-0.05

0.00

0.05

0.10

0.15

0.20

0.25

0.30

0.35

Time (s)

Acceleration (g)


[image: image45.wmf]A- 35 CF99026 1999 Pontiac Grand Am Left Foot I-S Acceleration

-80

-60

-40

-20

0

20

40

-0.05

0.00

0.05

0.10

0.15

0.20

0.25

0.30

0.35

Time (s)

Acceleration (g)


[image: image46.wmf]A- 36 CF99026 1999 Pontiac Grand Am Right Femur Axial Force

-3500

-3000

-2500

-2000

-1500

-1000

-500

0

500

1000

-0.05

0.00

0.05

0.10

0.15

0.20

0.25

0.30

0.35

Time (s)

Force (N)


[image: image47.wmf]A- 37 CF99026 1999 Pontiac Grand Am Right Femur Axial Force Analysis

0

2000

4000

6000

8000

10000

12000

0.000

0.002

0.004

0.006

0.008

0.010

0.012

0.014

0.016

0.018

0.020

Duration (s)

Force (N)

Acceptable

Good

Marginal

Poor


[image: image48.wmf]A- 38 CF99026 1999 Pontiac Grand Am Right Tibia-Femur Displacement

-1

0

1

-0.05

0.00

0.05

0.10

0.15

0.20

0.25

0.30

0.35

Time (s)

Displacement (mm)


[image: image49.wmf]A- 39 CF99026 1999 Pontiac Grand Am Right Upper Tibia L-M Moment

-160

-140

-120

-100

-80

-60

-40

-20

0

20

40

60

-0.05

0.00

0.05

0.10

0.15

0.20

0.25

0.30

0.35

Time (s)

Moment (Nm)


[image: image50.wmf]A- 40 CF99026 1999 Pontiac Grand Am Right Upper Tibia A-P Moment

-140

-120

-100

-80

-60

-40

-20

0

20

40

-0.05

0.00

0.05

0.10

0.15

0.20

0.25

0.30

0.35

Time (s)

Moment (Nm)

This data was ignored in the 

calculation of the upper tibia index 

because it was influenced by the tibia 

axial force.


[image: image51.wmf]A- 41 CF99026 1999 Pontiac Grand Am Right Upper Tibia Vector Resultant Moment

0

20

40

60

80

100

120

140

160

180

200

-0.05

0.00

0.05

0.10

0.15

0.20

0.25

0.30

0.35

Time (s)

Moment (Nm)

The resultant moment includes the 

contribution of the A-P upper tibia bending 

moment, which was artificially high due to the 

tibia axial force creating artifactual A-P 

bending moments.  This data was not use in 

the calculation of the tibia index. 


[image: image52.wmf]A- 42 CF99026 1999 Pontiac Grand Am Right Lower Tibia L-M Moment

-250

-200

-150

-100

-50

0

50

-0.05

0.00

0.05

0.10

0.15

0.20

0.25

0.30

0.35

Time (s)

Moment (Nm)


[image: image53.wmf]A- 43 CF99026 1999 Pontiac Grand Am Right Lower Tibia A-P Moment

-40

-30

-20

-10

0

10

20

30

40

50

60

-0.05

0.00

0.05

0.10

0.15

0.20

0.25

0.30

0.35

Time (s)

Moment (Nm)


[image: image54.wmf]A- 44 CF99026 1999 Pontiac Grand Am Right Lower Tibia Vector Resultant Moment

0

50

100

150

200

250

-0.05

0.00

0.05

0.10

0.15

0.20

0.25

0.30

0.35

Time (s)

Moment (Nm)


[image: image55.wmf]A- 45 CF99026 1999 Pontiac Grand Am Right Lower Tibia Axial Force

-4500

-4000

-3500

-3000

-2500

-2000

-1500

-1000

-500

0

500

-0.05

0.00

0.05

0.10

0.15

0.20

0.25

0.30

0.35

Time (s)

Force (N)


[image: image56.wmf]A- 46 CF99026 1999 Pontiac Grand Am Right Foot Vector Resultant Acceleration

0

10

20

30

40

50

60

70

-0.05

0.00

0.05

0.10

0.15

0.20

0.25

0.30

0.35

Time (s)

Acceleration (g)


[image: image57.wmf]A- 47 CF99026 1999 Pontiac Grand Am Right Foot A-P Acceleration

-60

-50

-40

-30

-20

-10

0

10

20

30

40

-0.05

0.00

0.05

0.10

0.15

0.20

0.25

0.30

0.35

Time (s)

Acceleration (g)


[image: image58.wmf]A- 48 CF99026 1999 Pontiac Grand Am Right Foot I-S Acceleration

-60

-50

-40

-30

-20

-10

0

10

-0.05

0.00

0.05

0.10

0.15

0.20

0.25

0.30

0.35

Time (s)

Acceleration (g)


[image: image59.png]1999 GRAND AM SE1 SEDAN
2.4L TWIN CAM 16V 4-CYL ENGINE exerion  PAINT, BRIGHT RED
®
®

TRANSMISSION, 4-SPEED AUTOMATIC nterior  TRIM, DARK PEWTER CLOTH

WE ARE DRIVING EXCITEMENT..

Division of General Motors Corporation

STANDARD EQUIPMENT MANUFACTURER'S SUGGESTED RETAIL PRICE

Items Featured Below are included at NO EXTRA CHARGE in the Standard Vehicle Price Shown at Right STANDARD VEHICLE PRICE $17 ’ 930.00
Options Installed by Manufacturer

POWERTRAINMEC H: e SOLAR HEAT REJECTING GLASS
P215/60R15 TOURING TIRES e REAR WINDOW DEFOGGER

OIL LIFE MONITOR e CONTROLLED CYCLE WIPERS CT/MA/NJ/NY/RI EMISSIONS -00
4-SPEED AUTO TRANSMISSION INTERIOR: SE1 CONTENT: NO CH
WITH ENHANCED TRACTION SYSTEM AIR CONDITIONING  CRUISE GONTROL CHARGE

STAINLESS STEEL EXHAUST ILLUMINATED ENTRY WITH o POWER WINDOWS
4-WHL INDEPENDENT SUSPENSION THEATER DIMMING e POWER MIRRORS
SAFETY/SECURITY: o INSTRUMENTATION W/TACHOMETER o DRIVER'S SEAT 4 WAY MANUAL
NEXT GENERATION DRIVER AND e CONSOLE W/FR & RR CUPHOLDERS W/POWER HEIGHT ADJUSTMENT
RIGHT FRT PASSENGER AIR BAGS e AM/FM STEREO WITH 4 SPEAKERS e SPLIT FOLDING REAR SEAT
4-WHEEL ANTI-LOCK BRAKES e CARPETED FLOOR MATS - FR & RR e AM/FM STEREO RADIO W/CLOCK
DAYTIME RUNNING LAMPS e ELECTRIC REMOTE TRUNK RELEASE & CASSETTE W/AUTO REV
PROGRAMMABLE AUTOMATIC POWER e TILT STEERING WHEEL o WHEELS. 15" ALUMINUM
DOOR LOCKS e BATTERY RUNDOWN PROTECTION ’
PASSLOCK THEFT-DETERRENT SYS. ¢ REAR SEAT HEAT DUCTS 'SOLID STYLE’ OPTION PACKAGE 895.00
EXTERIOR: . e AM/FM STEREO RADIO W/CD
WIDE BODY SIDE MOLDINGS g:‘l_‘,"es’g"%“ﬁg°’,§a"f“ (REPLACES RADIO INCLUDED IN
REFLECTOR OPTIC HEADLAMPS Z 'w,To:;I';g e Sartance OPTION PKG)
WITH AUTOMATIC LIGHT CONTROL e 16” ALUMINUM
FOG LAMPS — IMPACT RESISTANT ‘VPONTIACWNo-DeducthIe3Year/36,000 . 18 ALUMINUM WHEELSITIRES
REAR CORNERING LAMPS ©  (7s+ Mile Limited Warranty
REMOTE KEYLESS ENTRY 150.00
LIGHTER ELEMENT/ASHTRAY 15.00
TOTAL VEHICLE & OPTIONS 18,990.00
DESTINATION CHARGE 525.00
CITY MPG HIGHWAY MPG
SUBTOTAL 19,515.00
2 2 "VALUE PACKAGE” SAVINGS ~ -300.00
s s ron coupamson e
WILL VARY WITH OPTIONS, DRIVING VEH
¥ 1999 GRAND AM
CONDITIONS, DRiViNG HABITS AND
VEHICLE'S CONDITION. RESULTS 24 LITER L4 ENGINE COMPACT
REPORTED TO EPA INDICATE THAT THE FUEL INJECTION, AUTOMATIC HAVE BEEN ISSUED
MAJORITY OF VEHICLES WITH THESE 4 SPD ELECTRONIC TRANS ML AGE R TGS
ESTIMATES WILL ACHIEVE BETWEEN CATALYST, FEEDBACK FUEL SYSTEM n10 sawpeciy TOTAL FACTORY
18 AND 26 MPG IN THE CITY
AND BETWEEN 16 70 43 MPG SAVINGS OF -300.00
25 AND 35 MPG O . ESTIMATED ANNUAL FUEL COST: $719 HaHWAT: BASED ON PRICE OF OPTIONS AND
MODULES PURCHASED SEPARATELY
TOTAL VEHICLE PRICE* $19,215.00
DEALER TO WHOM DELIVERED CAPITAL PONTIAC-GMC TRUCK ORDER NO BPZT87 T D e M e maamOve Prior 10 dellver M M
5500 SOUTH LABURNUM AVENUE FINAL ASSEMBLY LANSING, Ml MODEL NO 2NE63 Does not Inciuds dealer installed options and accessories, local taxas or licanse 106,
RICHMOND VA 23231 SALES CODE E PART NO. 10283711 ©1995 General Moters Corporation

VIN 1G2NE5S2T7XC579674 M 1661949413










( 2000 Insurance Institute for Highway Safety
[MY Make Series] (CF9XXXX)
1005 N. Glebe Road, Arlington, VA  22201.  All rights reserved.
March 2000 — 1
( 2000 Insurance Institute for Highway Safety
1999 Pontiac Grand Am (CF99026)
1005 N. Glebe Road, Arlington, VA  22201.  All rights reserved.
April 2000 — 1

