REPORT NUMBER: 301-CAL-04-06
NATIONAL HIGHWAY TRAFFIC SAFETY ADMINISTRATION

MODERATE-TO-HIGH SPEED REAR IMPACT TEST

PERFORMED IN CONJUNCTION WITH

SAFETY COMPLIANCE TESTING FOR FMVSS 301R

FUEL SYSTEM INTEGRITY

DAIMLERCHRYSLER CORPORATION
2003 JEEP LIBERTY
SUV
NHTSA NUMBER: C30302
GDAIS TEST NUMBER: 8655-F301-23R
July 7, 2004
GENERAL DYNAMICS

ADVANCED INFORMATION ENGINEERING SERVICES

P.O. BOX 400

[image: image10.bmp]
BUFFALO, NEW YORK   14225

FINAL REPORT

PREPARED FOR:

U. S.  Department of Transportation

National Highway Traffic Safety Administration

Office of Vehicle Safety Research

Crashworthiness Research Division

400 Seventh Street,  S. W.

Room No.  6226

Washington,  DC   20590

This publication is distributed by the U.S. Department of Transportation, National Highway Traffic Safety Administration, in the interest of information exchange.  The opinions, findings and conclusions expressed in this publication are those of the author(s) and not necessarily those of the Department of Transportation or the National Highway Traffic Safety Administration.  The United States Government assumes no liability for its contents or use thereof.  If trade or manufacturers' names or products are mentioned, it is only because they are considered essential to the object of the publication and should not be construed as an endorsement.  The United States Government does not endorse products or manufacturers.

	

	
	
	
	
	
	
	
	Prepared By:
	

	
	
	
	
	
	
	
	
	
	Lawrence Q. Valvo, Project Engineer

	

	
	
	
	
	
	
	
	Approved By:
	

	
	
	
	
	
	
	
	
	
	David J. Travale, Program Manager

	
	
	
	
	
	
	
	
	Transportation Sciences Center



	
	
	
	
	
	
	
	Approval Date:
	

	
	
	
	
	
	
	
	
	
	

	

	
	
	
	
	
	
	
	FINAL REPORT ACCEPTANCE BY OCS:
	

	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	Accepted By:
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	Acceptance Date:
	


TECHNICAL REPORT STANDARD TITLE PAGE
	1.   Report No.
	2.   Government Accession No.
	3.  Recipient's Catalog No.

	       301-CAL-04-06
	
	

	4.  Title and Subtitle 
	5.  Report Date

	     Final Report, National Highway Traffic Safety Administration

Moderate-to-High Speed Rear Impact Test, Performed in Conjunction with Safety Compliance Testing for FMVSS 301R, Fuel System Integrity, of a 2003 Jeep Liberty SUV, 
	    July 7, 2004

	         NHTSA No. C30302
	6.  Performing Organization Code
     CAL

	7.  Author(s)
	8.  Performing Organization Report No.

	Lawrence Q. Valvo, Project Engineer

David J. Travale, Program Manager
	      8655-F301-23R

	9.  Performing Organization Name and Address
	10. Work Unit No.

	Advanced Information Engineering Services

4455 Genesee Street

Buffalo, New York 14225
	

	
	11. Contract or Grant No.
    DTNH22-01-C-01025

	12. Sponsoring Agency Name and Address 

       U.S. Department of Transportation

       National Highway Traffic Safety Administration

       Office of Vehicle Safety Research, Crashworthiness Research Division

       Mail Code: NVS-321

       400 Seventh St, S.W., Rm.  6226

       Washington, D.C.   20590
	13. Type of Report and Period Covered
  
Final Test Report

    July 2004

	
	14. Sponsoring Agency Code                                  NVS-321

	15. Supplementary Notes

	

	16. Abstract

	This test was performed in conjunction with Compliance Testing for FMVSS 301R, Fuel System Integrity.  The purpose of this test is to evaluate the seat and occupant performance in moderate-to-high speed rear impacts.  The base FMVSS 301R test was modified as follows:

- 2 fully instrumented Hybrid III ATDs were positioned in the front seats (P1 and P3) in place of ballast ATDs.

- 2 door mounted high speed cameras were installed.

- 4 seatback frame and 2 front sill MHDs were installed.

- 4 vehicle accelerometers were installed.

- VRTC seatback translation was installed.

The ATDs were positioned following TP208-12 with the following exceptions:

- Seatbacks for positions that contained a 50th percentile male were set to 25 degrees ± 1 degree as measured with the SAE J826 manikin.



	

	17. Key Words
   Seatback Rotation and Displacement Test    

   FMVSS 301R

    Rear Impact Test
	18. Distribution Statement 

Copies of this report are available from:

NHTSA Technical Reference Division  

Room 5108 (NPO-230), 400 Seventh , S.W., 

Washington, D.C.   20590

Telephone No. (202) 366-4946

	19. Security Classif. (of this report)

	20. Security Classif. (of this page)

	21. No. of Pages
	22. Price

	UNCLASSIFIED
	UNCLASSIFIED
	
185
	


TABLE OF CONTENTS
	Section
	
	Page No.

	
	
	

	1
	PURPOSE OF TEST
	1-1

	
	
	

	2
	SUMMARY OF TEST
	2-1

	
	
	

	3
	TEST DATA
	3-1

	
	Data Sheet 1 – Test Vehicle Specifications
	3-2

	
	Data Sheet 2 – Vehicle Data
	3-3

	
	Data Sheet 3 – Moving Barrier Data
	3-5

	
	Data Sheet 4 – Seat Data
	3-6

	
	Data Sheet 5 – ICBC Measurements
	3-8

	
	Data Sheet 6 – Dummy Measurements
	3-9

	
	Data Sheet 7 – Vehicle and Seat Instrumentation Measurements
	3-11

	
	Data Sheet 8 – Reference Target Measurements
	3-13

	
	Data Sheet 9 – Camera Positions For Rear Impacts
	3-14

	
	
	

	APPENDIX A
	PHOTOGRAPHS
	A-1

	
	
	

	APPENDIX B
	DATA
	B-1

	
	
	


SECTION 1

PURPOSE OF TEST


This moderate-to-high speed rear impact test was performed in conjunction with a Federal Motor Vehicle Safety Standard (FMVSS) 301R 30 mph rear moving barrier impact test of a 2003 Jeep Liberty SUV.  The purpose of this test is to evaluate the seat and occupant performance in moderate-to-high speed rear impacts.
SECTION 2

SUMMARY OF TEST


A 2035.5 kg 2003 Jeep Liberty SUV was impacted from the rear by a 1797 kg moving barrier at a velocity of 46.83 kph (29.1 mph).  The test was performed by Advanced Information Engineering Services on July 7, 2004.


The test vehicle was prepared for the Federal Motor Vehicle Safety Standard (FMVSS) 301R Compliance Test Program conducted for the National Highway Traffic Safety Administration (NHTSA) following the OVSC Laboratory Test Procedure No. TP-301-03, dated February 28, 2003.  For the purpose of acquiring information for Office of Vehicle Safety Research, the test was modified as follows:

- An instrumented 50th percentile male Part 572E Hybrid III ATD was positioned in the driver seat (P1) in place of a ballast ATD.

- An instrumented 50th percentile male Part 572E Hybrid III ATD was positioned in the right rear passenger seat (P3) in place of the right front passenger seat (P2) ballast ATD.

- The right front passenger seat (P2) was removed to improve film coverage and to help offset instrumentation weight.

- 2 high speed cameras were installed on the vehicle front doors (P2 and P4 doors).

- 2 seatback frame MHDs were installed on each occupied seat to measure rotation about the Y axis.

- 1 front sill MHD was installed to measure rotation about the Y-axis.

- 4 vehicle accelerometers were installed to measure acceleration along the X-axis.

- The VRTC seatback translation was installed on the inside of the roof to measure rearward displacement of the P1 and P3 seatbacks.


The crash event was recorded by ten high-speed cameras and one real-time camera.  Camera locations and other pertinent camera information are found on pages 3-13 and 3-14 of this report.    Pre- and post-test photographs of the ATDs can be found in Appendix A.  Electronic data plots are presented in Appendix B.


The occupant and seat data is summarized below.  

	
	Seatback Rotation 
	HIC (36ms)
	Nij
	Head -to-

Torso Rotation 
	Clip (g)
	Chest

Disp. (mm)
	Femur 

	
	Left

(deg)
	Right

(deg)
	
	Ntf
	Nte
	Ncf
	Nce
	Flexion

(deg)
	Extension

(deg)
	
	
	Left

(N)
	Right

(N)

	P1

ATD
	20.5
	21.2
	106.5
	0.16
	0.26
	0.00
	0.00
	20.3
	-47.9
	16.9
	-2.2
	-180.3
	-768.9

	P3

ATD
	11.4
	1.8
	132.3
	0.17
	0.23
	0.05
	0.01
	35.2
	-16.8
	28.9
	-4.7
	-1293.2
	-1160.6



The P1 seatback frame immediately above the adjustment mechanism deformed rearward and the left front seat track (fore/aft adjuster) slid rearward 66 mm during the impact.  The adjustment mechanism remained operable following the test.  The P1 seat back reclined 10.0 degrees statically from its pre-test position.  The P3 seatback frame deformed rearward 1.7 degrees statically from its pre-test position.  
SECTION 3

TEST DATA

DATA SHEET 1

TEST VEHICLE SPECIFICATIONS

	TEST VEHICLE INFORMATION:



	Year/Make/Model/Body Style:
	2003 Jeep Liberty SUV

	NHTSA No.:
	C30302
	;       Color:
	Red
	

	Engine Data:
	4
	Cylinders;
	-
	CID;
	2.4
	Liters;
	-
	cc

	Placement:
	X
	Longitudinal or In-Line;
	-
	Transverse or Lateral

	Transmission Data:
	5
	Speeds;
	X
	Manual;
	-
	Automatic;
	-
	Overdrive

	Final Drive:
	-
	Rear Wheel Drive;
	-
	Front Wheel Drive;
	X
	Four Wheel Drive

	
	Major Options:
	X
	A/C;
	X
	Power Steering;
	X
	Power Brakes

	
	
	X
	Power Windows;
 
	X
	Power Door Locks;
	X
	Tilt Wheel

	Date Received:
	1/19/2004
	;             Odometer Reading
	102
	km

	Selling Dealer:
	Ricart Chrysler, Plymouth, Jeep, Eagle

	& Address:
	465 South Hamilton Road,  Columbus OH  43213

	
	

	DATA FROM VEHICLE'S CERTIFICATION LABEL:

	
	Vehicle Manufactured by:
	DaimlerChrysler Corporation

	
	Date of Manufacture: 
	9-02

	
	VIN:
	1J4GL48183W539150

	
	GVWR:
	2359
	kg;           GAWR-FRONT:
	
	kg;         GAWR-REAR:
	
	 kg

	DATA FROM VEHICLE'S TIRE LABEL:

	
	Location of Placard on Vehicle:
	Driver Door

	
	Recommended Tire Size:
	P215/75R16

	*
	Recommended Cold Tire Pressure:                                      FRONT:
	227
	kPa;      REAR:
	227
	kPa

	DATA FROM TIRE SIDEWALL:

	
	Size of Tires on Test Vehicle:
	P215/75R16  101S
	Manufacturer:  
	Goodyear Wrangler

	
	Tire Pressure with Maximum Capacity Vehicle Load:        FRONT:
	300
	kPa;      REAR:
	300
	kPa

	
	Type of Spare Tire:
	P215/75R16 mounted on rear gate
	

	VEHICLE CAPACITY DATA:

	
	Type of Front Seats:
	X
	Bench;
	-
	Bucket;
	-
	Split Bench

	
	Number of Occupants:
	2
	Front;
	3
	Rear;
	5
	 Total

	
	Vehicle Capacity Weight  (VCW)     =
	601.5
	kg

	
	No. of Occupants x  68.04 kg            =
	340.2
	kg

	
	Rated Cargo/Luggage Weight  (RCLW) =

	261.3
	kg **

	*Tire pressure used for test

** Maximum of 136.1 kg used for target weight calculation.
	
	


DATA SHEET 2

VEHICLE DATA

	WEIGHT OF TEST VEHICLE AS RECEIVED FROM DEALER  (with maximum fluids)= UDW:

	
	Right Front        =
	444.5
	kg
	Right Rear =
	409.5
	kg

	
	Left Front        =
	470.0
	kg.
	Left Rear =
	433.5
	kg

	
	TOTAL FRONT  =
	914.5
	kg
	TOTAL REAR  =
	843
	kg

	
	TOTAL DELIVERED WEIGHT =
	1757.5
	kg

	
	% of Total  Front of Vehicle Weight  =
	52.0%
	of Total Rear Weight    =
	48.0%
	

	CALCULATION OF VEHICLE'S TARGET TEST WEIGHT:

	
	Total Delivered Weight                                                  =
	1757.5
	kg

	
	Rated Cargo/Luggage Weight (RCLW)                         =
	136.1
	kg

	
	Weight of 2 p.572 Dummies, 74.4 kg                            =
	148.8
	kg

	
	TARGET TEST WEIGHT                                             =
	2042.4
	kg

	WEIGHT OF TEST VEHICLE WITH TWO DUMMIES AND 
	129.2
	KG OF CARGO WEIGHT:

	
	Right Front        =
	520.0
	kg
	Right Rear =
	480.0
	kg

	
	Left Front        =
	532.5
	kg
	Left Rear =
	503.0
	kg

	
	TOTAL FRONT  =
	1052.5
	kg
	TOTAL REAR  =
	983
	kg

	
	TOTAL TEST WEIGHT =
	2035.5
	kg

	
	% of Total  Front of Vehicle Weight  =
	51.7%
	of Total Rear Weight    =
	48.3%
	

	*
	Weight of Ballast Secured in Vehicle Trunk Area   =
	73
	kg
	

	
	Type of Ballast:
	Lead shot
	

	
	Method of Securing Ballast:
	Left rear seat anchors, engine compartment firewall

	
	Vehicle Components Removed for Weight Reduction:
	None

	VEHICLE ATTITUDE (all dimension in millimeters):

	
	AS DELIVERED:
	    RF
	834
	      LF
	826
	       RR
	844
	      LR
	831
	

	
	AS TESTED:
	    RF
	803
	      LF
	796
	       RR
	816
	      LR
	810
	

	
	Vehicle's Wheel Base:
	2640
	mm

	
	Location of Vehicle's C.G.:
	1275
	millimeters rearward of front wheel center.


DATA SHEET 2 (continued)
VEHICLE DATA
	TYPE OF TEST:

	
	
	Type of Test:
	Rear Barrier
	Impact Angle:

	0º
	

	
	
	Test Date:
	July 7, 2004
	Time:
	10:30
	Temperature:
	21.1
	oC

	
	
	Vehicle NHTSA No.:
	C30302
	VIN:
	1J4GL48183W539150

	
	
	Required Impact Velocity Range:
	46.51
	to
	48.12
	kph

	BARRIER IMPACT VELOCITY:  (Speed traps within 5 feet of impact plane.)

	
	
	Trap No. 1   =
	46.83
	kph;     Trap No. 2  =
	46.83
	 kph

	
	
	Average Impact Speed   =
	46.83
	kph
	
	
	

	VEHICLE STATIC CRUSH:

	
	
	Vehicle Length:
	

	
	
	
	Pre-Test
	Left =
	4188
	;  C/L  =
	4444
	Right  =
	4188
	

	
	
	
	Post-Test
	Left =
	4090
	;  C/L  =
	4165
	Right  =
	4095
	

	
	
	
	Crush
	Left =
	98
	;  C/L  =
	279
	Right  =
	93
	

	
	
	
	AVERAGE
	=
	157
	millimeters


DATA SHEET 3

MOVING BARRIER DATA

	WEIGHT OF MOVING BARRIER:

	
	Right Front        =
	504.9
	kg
	Right Rear      =
	393.7
	kg.

	
	Left Front          =
	499.9
	kg
	Left Rear         =
	398.3
	kg

	
	TOTAL FRONT  =
	1004.8
	kg
	TOTAL REAR  =
	792.0
	kg

	
	TOTAL BARRIER WEIGHT =
	1796.8
	kg

	MOVING BARRIER DIMENSIONS:

	
	Barrier Face Height:
	1524
	mm

	
	Barrier Face Width:
	1981
	mm

	
	Barrier Face Ground Clearance:
	127
	mm

	
	Tread Width:
	1511
	mm

	
	Wheel Base:
	3048
	mm

	
	Location of C.G.:
	
	X:
	1344
	mm rearward of front wheel center.

	
	
	
	Y:
	0
	mm from longitudinal-vertical plane of symmetry.

	
	
	
	Z:
	414
	mm above ground.

	MOVING BARRIER TIRES:

	
	Manufacturer:
	Dunlop

	
	Model:
	AT Radial Rover

	
	Size:
	P205/75R15

	
	Recommended Max Pressure:
	240
	kPa:

	MOVING BARRIER ABORT SYSTEM:

	
	Type:
	Trailing cable

	[image: image2.wmf]


DATA SHEET 4

SEAT DATA

	P1 DATA:

	

	HEAD RESTRAINT
	YES
	
	NO
	DETAILS

	Fixed:
	X
	
	
	

	Vertical Adjustment:
	
	
	X
	

	Vertical Locks:
	
	
	X
	

	Fore-Aft Adjustment:
	
	
	X
	

	Fore-Aft Locks:
	
	
	X
	

	
	
	
	
	

	SEATBACK RECLINE TYPE
	YES
	
	NO
	DETAILS

	Fixed:
	-
	
	X
	

	Power:
	-
	
	X
	

	Lever (with detents):
	X
	
	-
	

	Lever (infinite)
	-
	
	X
	

	Lever (ratchet)
	-
	
	X
	

	Rotary Knob:
	-
	
	X
	

	Pivot locks on Both Sides:
	X
	
	-
	

	
	
	
	
	

	LUMBAR ADJUSTMENT
	YES
	
	NO
	DETAILS

	Fixed:
	X
	
	-
	

	Adjustable:
	-
	
	X
	Placed in retracted position for ICBC measurements and test.

	
	
	
	
	

	BOLSTER ADJUSTMENT
	YES
	
	NO
	DETAILS

	Fixed:
	X
	
	-
	

	Adjustable:
	-
	
	X
	Placed in full open for ICBC measurements and test.

	
	
	
	
	

	SEAT CUSHION TYPE
	YES
	
	NO
	DETAILS

	Cushion moves vertically independently of seat back:
	-
	
	X
	

	
	
	
	
	

	SEAT CUSHION FORE-AFT
	YES
	
	NO
	DETAILS

	Fixed:
	-
	
	X
	

	Power:
	-
	
	X
	Travel:
	-
	mm
	
	
	
	

	Manual-Detents:
	X
	
	-
	First Detent:
	0
	Last Detent:
	18
	Travel:
	215
	mm

	Manual-Infinite:
	-
	
	X
	Travel:
	-
	mm
	
	
	
	

	
	
	
	
	

	SEAT CUSHION VERTICAL
	YES
	
	NO
	DETAILS

	Fixed:
	
	
	
	

	Power:
	
	
	
	

	Manual-Detents
	
	
	
	

	Manual-Infinite
	
	
	
	

	
	
	
	
	

	SEAT TEST POSITION
	
	
	
	

	Seat Cushion Fore-aft:
	Mid-position (detent 9) OR 107.5 mm forward of rearward-most position

	Seat Cushion Vertical:
	Fixed

	Seat Back:
	25 degree torso angle as set with the H-point machine.

	Head Restraint:
	Fixed


DATA SHEET 4 (continued)

SEAT DATA

	P3 DATA:

	

	HEAD RESTRAINT
	YES
	
	NO
	DETAILS

	Fixed:
	-
	
	X
	

	Vertical Adjustment:
	X
	
	-
	

	Vertical Locks:
	X
	
	-
	Prevents head restraint from sliding downward. Button must be pushed to move restraint downward.

	Fore-Aft Adjustment:
	-
	
	X
	

	Fore-Aft Locks:
	-
	
	X
	

	
	
	
	
	

	SEATBACK RECLINE TYPE
	YES
	
	NO
	DETAILS

	Fixed:
	X
	
	-
	

	Power:
	-
	
	X
	

	Lever (with detents):
	-
	
	X
	

	Lever (infinite)
	-
	
	X
	

	Lever (ratchet)
	-
	
	X
	

	Rotary Knob:
	-
	
	X
	

	Pivot locks on Both Sides:
	-
	
	X
	Forward folding seatback latches in a single upright position for occupant use.  Latch is located at the upper outboard side of the seatback.

	
	
	
	
	

	LUMBAR ADJUSTMENT
	YES
	
	NO
	DETAILS

	Fixed:
	X
	
	-
	

	Adjustable:
	-
	
	X
	Placed in retracted position for ICBC measurements and test.

	
	
	
	
	

	BOLSTER ADJUSTMENT
	YES
	
	NO
	DETAILS

	Fixed:
	X
	
	-
	

	Adjustable:
	-
	
	X
	Placed in full open for ICBC measurements and test.

	
	
	
	
	

	SEAT CUSHION TYPE
	YES
	
	NO
	DETAILS

	Cushion moves vertically independently of seat back:
	-
	
	X
	

	
	
	
	
	

	SEAT CUSHION FORE-AFT
	YES
	
	NO
	DETAILS

	Fixed:
	X
	
	-
	

	Power:
	-
	
	X
	Travel:
	-
	mm
	
	
	
	

	Manual-Detents:
	-
	
	X
	First Detent:
	-
	Last Detent:
	-
	Travel:
	-
	mm

	Manual-Infinite:
	-
	
	X
	Travel:
	-
	mm
	
	
	
	

	
	
	
	
	

	SEAT CUSHION VERTICAL
	YES
	
	NO
	DETAILS

	Fixed:
	X
	
	-
	

	Power:
	-
	
	X
	

	Manual-Detents
	-
	
	X
	

	Manual-Infinite
	-
	
	X
	

	
	
	
	
	

	SEAT TEST POSITION
	
	
	
	

	Seat Cushion Fore-aft:
	Fixed

	Seat Cushion Vertical:
	Fixed

	Seat Back:
	Fixed

	Head Restraint:
	Full up


DATA SHEET 5

ICBC MEASUREMENTS

	
[image: image3.emf]H

B



	P1 DATA:

	

	ICBC MEASUREMENTS
	
	
	
	
	
	
	

	HEAD RESTRAINT POSITION

(vertical/fore-aft)
	B

(mm, backset)
	
	H

(mm, height)
	
	Torso Angle (deg)
	
	Height Along Torso Line From Pivot (mm)

	Fixed / Fixed:
	49
	
	63
	
	25.0
	
	789

	
	
	
	
	
	
	
	

	HEAD RESTRAINT POSITION – CONFIGURED FOR TEST

(vertical/fore-aft)
	B

(mm, backset)
	
	H

(mm, height)
	
	Torso Angle (deg)
	
	Height Along Torso Line From Pivot (mm)

	Fixed / Fixed:
	49
	
	63
	
	25.0
	
	789

	

	Head restraint positioning details:
	None


	P3 DATA:

	

	ICBC MEASUREMENTS
	
	
	
	
	
	
	

	HEAD RESTRAINT POSITION

(vertical/fore-aft)
	B

(mm, backset)
	
	H

(mm, height)
	
	Torso Angle (deg)
	
	Height Along Torso Line From Pivot (mm)

	Full Up / Fixed:
	88
	
	44
	
	24.1
	
	835

	Full Down / Fixed:
	81
	
	103
	
	24.1
	
	772

	
	
	
	
	
	
	
	

	HEAD RESTRAINT POSITION – CONFIGURED FOR TEST

(vertical/fore-aft)
	B

(mm, backset)
	
	H

(mm, height)
	
	Torso Angle (deg)
	
	Height Along Torso Line From Pivot (mm)

	Full Up / Fixed:
	88
	
	44
	
	24.1
	
	835

	

	Head restraint positioning details:
	Head restraint was positioned full up.


DATA SHEET 6
DUMMY MEASUREMENTS

[image: image4.png]DUMMY MEASUREMENT FOR FRONT SEAT PASSENGERS

CD - Chestto Dash
CS - Steering Wheel to Chest
HH - Head to Header

HW - Head to Windshield

HZ - Head to Raot

KDA - Kneeta DashAngle
KDL- Left Knee ta Dash

KDR - Right Knee to Dash
NA - Noseto Rim Angle

NR - Noseto Rim
PA - Pelic Angle
RA - Rim to Abdomen

SA - SeatBack Angle
SCA - Steering Column Angle
SH - Striker to H-Point

SK - Striker to Knee

ST - Striker to Head

SWA- Steering Wheel Angle

TA - Tibial Angle

WA - Windshield Angle

BS - Backset

AD - Ammn to Door
HD - H-Pointto

Door
HR - Headto

Side Header
HS - Headto

Side Window
KK - Knee to Knee
AA_ - Ankle to Ankle
SHY- Strikerto

H-Point

(¥ Direction)

VERTICAL LONGITUDINAL PLANES 1




DATA SHEET 6 (continued)
DUMMY MEASUREMENTS
	
	DRIVER (Serial #202)
	PASS. (Serial #206)

	WAO
	43.9
	 deg.
	N/A

	SWAO
	66.0
	 deg.
	N/A

	SCAO
	24.0
	 deg.
	N/A

	SAO
	25.0
	 deg. (H-point machine back pan)
	24.1
	 deg. (H-point machine back pan)

	HZ
	218
	294

	HH
	431
	1304

	HW
	558
	1458

	HR
	188
	245

	NR
	420
	Angle 
	17
	 deg.
	N/A

	CD
	519
	1432

	CS
	298
	N/A

	RA
	184
	N/A

	KDL
	136
	Angle (KDA)
	5
	 deg.
	1014

	KDR
	133
	1014
	Angle (KDA)
	3
	 deg.

	PAO
	24.8
	 deg.
	24.2
	 deg.

	TAO
	61.5
	 deg.
	62.0
	 deg.

	KK
	372
	306

	AA
	248
	180

	ST
	596
	Angle 
	5
	 deg.
	384
	Angle
	10
	 deg.

	SK
	594
	Angle
	83
	 deg.
	606
	Angle
	102
	 deg.

	SH
	207
	Angle
	102
	 deg.
	330
	Angle
	133
	 deg.

	SHY
	198
	222

	HS
	340
	358

	HD
	194
	160

	AD
	85
	93

	BS*
	68
	101


*Note: Measured with ATD in final seating position
Dimensions in millimeters

DATA SHEET 7
VEHICLE AND SEAT INSTRUMENTATION MEASUREMENTS
	VEHICLE INSTRUMENTATION

	LOCATION
	TRANSDUCER TYPE
	DIRECTION OF MEASURE
	PRE-TEST DIMENSIONS (mm) *

	
	
	
	X
	Y
	Z

	Left Rear Seat Cross Member
	Accelerometer
	X
	1905
	-681
	-465

	Left Rear Seat Cross Member Redundant
	Accelerometer
	X
	1905
	-681
	-465

	Right Rear Seat Cross Member Redundant
	Accelerometer
	X
	1905
	681
	-465

	Right Rear Seat Cross Member
	Accelerometer
	X
	1905
	681
	-465

	Left Front Sill
	MHD ARS-06
	About Y
	2665
	656
	-465

	Right Front Sill
	MHD ARS-06
	About Y
	-
	-
	-


* X: From rear bumper, positive forward; Y: From long. centerline, positive right; Z: From ground, positive down.
DATA SHEET 7
VEHICLE AND SEAT INSTRUMENTATION MEASUREMENTS
	
[image: image5.emf]a

c

b

d

e (400 mm target)

h-point (vertical 




	DIMENSION
	P1 LEFT
	P1 RIGHT
	P3 LEFT
	P3 RIGHT

	a (deg from horiz.)
	26.7
	29.5
	19.8
	20.0

	b (deg from vertical)
	23.6
	23.6
	21.5
	23.3

	c (mm)
	330
	328
	373
	381

	d (mm)
	375
	373
	369
	370

	e (mm, target=400)
	321
	316
	380
	386

	

	SEATBACK ANGLE
	P1
	P3

	
	PRE

(deg)
	POST

(deg)
	PRE

(deg)
	POST

(deg)

	Left MDH Bracket
	20.8
	31.1
	20.2
	21.6

	Right MHD Bracket
	20.8
	31.1
	22.0
	24.6

	Front Sill
	-0.8
	-0.5
	-0.8
	-0.5

	

	POST TEST DETAILS

	Seatback Deformation: The P1 seatback frame immediately above the adjustment mechanism deformed rearward.  The 

	P1 seat back reclined 10.0 degrees statically from its pre-test position.  The P3 seatback frame deformed (reclined) 

	rearward 1.7 degrees statically from its pre-test position.

	Seat/Floor Attachment Point Deformation:  No deformation of the seat/floor attachment points.  The P1 left front seat 

	track (fore/aft adjuster) slid rearward 66 mm during the impact.

	

	Recliner Mechanism Deformation: None. The adjustment mechanism remained operable following the test.

	

	

	Comments: None

	

	


DATA SHEET 8
REFERENCE TARGET MEASUREMENTS
	NHTSA No. :  C30302







 Vehicle :     2003 Jeep Liberty SUV




[image: image6.emf]200 mm

200 mm

Reference Target Locations

  
[image: image7.emf]Film Plane

Seatback Targets

Head Restraint Target

Head Target

Side Header Targets

Reference Target Planes


	Film Plane to Reference Plane Distance



	Plane
	Front Occupant View (mm)
	Rear Occupant View (mm)

	Seatback 
	1060
	-

	Head Restraint
	1220
	1182

	Head Target
	1250
	1220

	Side Header
	1457
	1404


  DATA SHEET 9
HIGH SPEED CAMERA LOCATIONS
[image: image1.png]




[image: image8]

DATA SHEET 9 (continued)
HIGH SPEED CAMERA LOCATIONS
	 NHTSA No. :  C30302

 Vehicle :     2003 Jeep Liberty SUV


	CAMERA NO.
	VIEW
	CAMERA POSITIONS (mm)*
	ANGLE** (degrees)
	LENS 

(mm)
	SPEED

(fps)

	
	
	X
	Y
	Z
	
	
	

	1
	Real-Time Camera
	-
	-
	-
	-
	-
	24

	2
	Left Side View
	17665
	3450
	1110
	0.0
	35
	View not available‡

	3
	Right Side View
	-17840
	405
	1108
	1.0
	35
	1000

	4
	Overhead Overall View
	0
	508
	9804
	-90
	13
	1000

	5
	Overhead Close View
	0
	508
	9804
	-105
	25
	Timing not available

	6†
	Onboard Driver View
	-964
	2355
	1208
	-10.0
	8
	1010

	7†
	Onboard Passenger View
	1015
	1590
	1205
	-10.0
	8
	1000

	8
	Vehicle Rear Underbody View
	0
	1080
	-1956
	90
	13
	1030

	9
	Vehicle Mid-Section Underbody View
	0
	2390
	-1956
	90
	13
	1000

	10
	Vehicle Front Underbody View
	0
	3790
	-1956
	90
	13
	1005


* X = film plane to monorail centerline (+ to left of rail) 

   Y = film plane to impact location (+ ahead of impact location)

   Z  = film plane to ground (+ above ground)

**    = referenced to horizontal plane

† Research cameras – X distance is measured to the reference target plane.

Appendix A

PHOTOGRAPHS

LIST OF PHOTOGRAPHS

	Figure
	Photograph Title
	Page  

	
	
	

	A-1
	DRIVER SEAT ICBC BACKSET VIEW
	A-3

	A-2
	PASSENGER SEAT ICBC BACKSET VIEW
	A-4

	A-3
	PRE-TEST DRIVER BACKSET VIEW
	A-5

	A-4
	PRE-TEST PASSENGER BACKSET VIEW
	A-6

	A-5
	PRE-TEST DRIVER SEAT CLOSE-UP VIEW
	A-7

	A-6
	POST-TEST DRIVER SEAT CLOSE-UP VIEW
	A-8

	A-7
	PRE-TEST DRIVER SEAT PERPENDICULAR VIEW
	A-9

	A-8
	POST-TEST DRIVER SEAT PERPENDICULAR VIEW
	A-10

	A-9
	PRE-TEST PASSENGER SEAT CLOSE-UP VIEW
	A-11

	A-10
	POST-TEST PASSENGER SEAT CLOSE-UP VIEW
	A-12

	A-11
	PRE-TEST PASSENGER SEAT PERPENDICULAR VIEW
	A-13

	A-12
	POST-TEST PASSENGER SEAT PERPENDICULAR VIEW
	A-14

	A-13
	PRE-TEST DRIVER LEFT SIDE  VIEW
	A-15

	A-14
	POST-TEST  DRIVER LEFT SIDE VIEW
	A-16

	A-15
	PRE-TEST DRIVER RIGHT SIDE  VIEW
	A-17

	A-16
	POST-TEST  DRIVER RIGHT SIDE VIEW
	A-18

	A-17
	PRE-TEST DRIVER ISOMETRICAL VIEW
	A-19

	A-18
	POST-TEST DRIVER ISOMETRICAL VIEW
	A-20

	A-19
	PRE-TEST PASSENGER LEFT SIDE VIEW
	A-21

	A-20
	POST-TEST PASSENGER LEFT SIDE VIEW
	A-22

	A-21
	PRE-TEST PASSENGER RIGHT SIDE VIEW
	A-23

	A-22
	POST-TEST PASSENGER RIGHT SIDE VIEW
	A-24

	A-23
	PRE-TEST PASSENGER ISOMETRICAL VIEW
	A-25

	A-24
	POST-TEST PASSENGER ISOMETRICAL VIEW
	A-26


Appendix B
VEHICLE AND DUMMY RESPONSE DATA 

TEST  NO.  C30302
DRIVER DUMMY (Pos. 1)

SAE  FILTER  CHANNEL  CLASS
Head Accelerations


1000

Head Angular Velocity


1000 
Neck Forces


1000

Neck Moments


  600

Chest Accelerations


  180

Chest Angular Velocity


  600

Chest Rotations


1000

Pelvic Accelerations


1000

Belt Forces


    60

Belt Spoolout



  180

Vehicle Accelerations


    60

Vehicle Angular Velocity


1000

Note:  Angular position is measured in degrees of rotation from the initial (design) position.

FACILITY: VERF                               DATE: July 07, 2004                 

TEST#: 42057-1009-2145                                 

TITLE: 2003  '301' Test 9  2004 Jeep Liberty  C30302

Version 5.00

	P1 Head
	

	P1 HIC(15 ms): 72.9

    t1: 115.7 msec

    t2: 130.7 msec

    Duration: 15.0 msec

    Average Acceleration: 29.8 g
	P1 HIC(36 ms): 106.6

    t1: 103.1 msec

    t2: 139.1 msec

    Duration: 36.0 msec

    Average Acceleration: 24.5 g

	
	

	P1 Upper Neck
	

	P1 UP NECK Fx:  Max:    386.5 N 126.6 msec  

                Min:   -204.4 N 361.7 msec
	P1 UP NECK Fz:  Max:   1135.5 N 115.5 msec  

                Min:    -14.2 N -22.2 msec

	P1 UP NECK Mocy (50th Male)

                Max:     16.5 N-m 373.4 msec  

                Min:    -18.1 N-m 146.7 msec
	

	
	

	P1 UP NECK Nij (50th Male)


	

	    Ntf: 0.16 Nij 123.0 msec   CVt: 6806; CVf: 310

              Fz: 990.3 N

              Fx: 319.6 N

              Mocy: 3.1 N-m
	    Nte: 0.26 Nij 115.1 msec   CVt: 6806; CVe: 135

              Fz: 1122.7 N

              Fx: 117.5 N

              Mocy: -12.3 N-m

	    Ncf: 0.00 Nij -28.9 msec   CVc: 6160; CVf: 310

              Fz: -1.6 N

              Fx: 0.1 N

              Mocy: 0.0 N-m
	    Nce: 0.00 Nij-100.0 msec   CVc: 6160; CVe: 135

              Fz: -9.6 N

              Fx: 1.1 N

              Mocy: -0.1 N-m

	
	

	P1 Chest
	

	P1 CLIP(3 ms): 16.9 g

    t1: 79.7 msec

    t2: 82.7 msec

    Duration: 3.0 msec

P1 CSI: 38.5
	P1 CHEST DISP:  Max:      0.3 mm 463.7 msec  

                Min:     -2.2 mm  98.1 msec  

	
	

	P1 Femur
	

	P1 LEFT FEMUR:  Max:   2490.5 N  72.3 msec  

                Min:   -180.3 N  64.3 msec
	P1 RIGHT FEMUR: Max:   1025.0 N  82.0 msec  

                Min:   -768.9 N  73.1 msec

	
	


FACILITY: VERF                               DATE: July 07, 2004                 

TEST#: 42057-1009-2145                                 

TITLE: 2003  '301' Test 9  2004 Jeep Liberty  C30302

Version 5.00

	P3 Head
	

	P3 HIC(15 ms): 107.2

    t1: 83.6 msec

    t2: 98.6 msec

    Duration: 15.0 msec

    Average Acceleration: 34.8 g
	P3 HIC(36 ms): 132.3

    t1: 80.3 msec

    t2: 105.0 msec

    Duration: 24.7 msec

    Average Acceleration: 31.0 g

	
	

	P3 Upper Neck
	

	P3 UP NECK Fx:  Max:    472.7 N  94.9 msec  

                Min:   -226.3 N 208.8 msec  
	P3 UP NECK Fz:  Max:    961.5 N  87.8 msec  

                Min:   -133.8 N  45.0 msec

	P3 UP NECK Mocy (50th Male)

                Max:     25.6 N-m  95.6 msec  

                Min:    -26.1 N-m 115.2 msec
	

	
	

	P3 UP NECK Nij (50th Male)


	

	    Ntf: 0.17 Nij  92.1 msec   CVt: 6806; CVf: 310

              Fz: 722.6 N

              Fx: 404.4 N

              Mocy: 18.7 N-m
	    Nte: 0.23 Nij 114.4 msec   CVt: 6806; CVe: 135

              Fz: 231.7 N

              Fx: -145.1 N

              Mocy: -25.9 N-m

	    Ncf: 0.05 Nij 251.1 msec   CVc: 6160; CVf: 310

              Fz: -30.2 N

              Fx: -176.5 N

              Mocy: 12.8 N-m
	    Nce: 0.01 Nij 497.6 msec   CVc: 6160; CVe: 135

              Fz: -5.4 N

              Fx: 23.6 N

              Mocy: -1.0 N-m

	
	

	P3 Chest
	

	P3 CLIP(3 ms): 28.9 g

    t1: 51.0 msec

    t2: 54.0 msec

    Duration: 3.0 msec

P3 CSI: 69.6
	P3 CHEST DISP:  Max:      0.2 mm 106.3 msec  

                Min:     -4.7 mm 206.6 msec

	
	

	P3 Femur
	

	P3 LEFT FEMUR:  Max:    272.7 N 159.8 msec  

                Min:  -1293.2 N  61.7 msec
	P3 RIGHT FEMUR: Max:    283.8 N 162.1 msec  

                Min:  -1160.6 N  57.6 msec

	
	


Maximum and Minimum Data

	NAME
	Unit
	Max
	msec
	Min
	msec
	Comment

	V2P1 Head x
	g
	28.9
	125.7
	-5.4
	365.5
	

	V2P1 Head y
	g
	3.0
	154.0
	-2.0
	122.3
	

	V2P1 Head z
	g
	17.4
	115.1
	-0.1
	-76.3
	

	V2P1 Head Resultant
	g
	31.2
	124.1
	0.0
	-87.0
	

	V2P1 Head AVy
	deg/s
	1503.7
	108.6
	-1021.4
	214.3
	

	V2P1 Head Comp AVy
	deg/s
	1588.7
	108.9
	-1030.6
	216.9
	f2=0.283 Hz; f4=0.001 Hz

	V2P1 Head Comp ADy
	deg
	64.6
	124.2
	-48.4
	379.6
	f2=0.283 Hz; f4=0.001 Hz

	V2P1 Upper Neck Fx
	N
	386.5
	126.6
	-204.4
	361.7
	

	V2P1 Upper Neck Fy
	N
	153.0
	143.8
	-48.8
	389.1
	

	V2P1 Upper Neck Fz
	N
	1135.5
	115.5
	-14.2
	-22.2
	

	V2P1 Upper Neck F Resultant
	N
	1143.6
	115.5
	0.1
	-47.2
	

	V2P1 Upper Neck Mx
	N-m
	10.4
	123.5
	-3.7
	380.1
	

	V2P1 Upper Neck My
	N-m
	16.0
	127.7
	-18.2
	146.7
	

	V2P1 Upper Neck Mz
	N-m
	4.3
	166.1
	-3.5
	102.8
	

	V2P1 Upper Neck M Resultant
	N-m
	18.8
	146.8
	0.0
	-5.8
	

	V2P1 Lower Neck Fx
	N
	993.1
	124.0
	-123.4
	356.3
	

	V2P1 Lower Neck Fy
	N
	245.8
	131.3
	-93.0
	81.1
	

	V2P1 Lower Neck Fz
	N
	840.2
	115.0
	-50.8
	390.8
	

	V2P1 Lower Neck F Resultant
	N
	1244.8
	115.3
	0.2
	-93.3
	

	V2P1 Lower Neck Mx
	N-m
	35.9
	132.8
	-10.9
	253.9
	

	V2P1 Lower Neck My
	N-m
	48.0
	378.8
	-186.0
	115.7
	

	V2P1 Lower Neck Mz
	N-m
	17.2
	378.4
	-80.4
	107.6
	

	V2P1 Lower Neck M Resultant
	N-m
	201.9
	115.7
	0.0
	-15.4
	

	V2P1 Lower Neck x
	g
	18.1
	56.1
	-4.3
	316.1
	

	V2P1 Chest x
	g
	17.1
	80.6
	-3.8
	316.5
	

	V2P1 Chest y
	g
	4.5
	83.6
	-2.1
	73.7
	

	V2P1 Chest z
	g
	7.8
	70.3
	-1.7
	84.6
	

	V2P1 Chest Resultant
	g
	17.5
	80.6
	0.0
	-42.5
	

	V2P1 Chest Displacement
	mm
	0.3
	463.7
	-2.2
	98.1
	

	V2P1 Chest AVy
	deg/s
	405.7
	89.3
	-289.5
	161.5
	

	V2P1 Chest Comp AVy
	deg/s
	420.0
	89.6
	-294.7
	266.5
	f2=0.298 Hz; f4=0.001 Hz

	V2P1 Chest Comp ADy
	deg
	17.7
	135.6
	-28.1
	380.3
	f2=0.298 Hz; f4=0.001 Hz

	V2P1 Pelvic x
	g
	32.1
	70.3
	-1.8
	141.4
	

	V2P1 Pelvic y
	g
	4.3
	113.7
	-2.9
	70.5
	

	V2P1 Pelvic z
	g
	12.2
	69.5
	-1.3
	300.0
	

	V2P1 Pelvic Resultant
	g
	33.0
	70.3
	0.0
	-12.7
	

	V2P1 Left Femur
	N
	2490.5
	72.3
	-180.3
	64.3
	

	V2P1 Right Femur
	N
	1025.0
	82.0
	-768.9
	73.1
	

	V2P1 Head to Chest
	deg
	20.3
	379.6
	-47.9
	122.8
	

	V2P1 Lap Belt
	N
	211.4
	332.7
	-25.0
	88.6
	

	V2P1 Belt Spoolout
	mm
	0.0
	12.9
	-83.3
	147.8
	


Maximum and Minimum Data (Continued)
	NAME
	Unit
	Max
	msec
	Min
	msec
	Comment

	V2P3 Head x
	g
	36.5
	95.6
	-5.4
	212.7
	

	V2P3 Head y
	g
	5.2
	119.9
	-4.6
	68.8
	

	V2P3 Head z
	g
	18.3
	87.8
	-3.2
	45.2
	

	V2P3 Head Resultant
	g
	37.9
	94.9
	0.0
	-32.9
	

	V2P3 Head AVy
	deg/s
	1352.5
	81.0
	-897.9
	129.0
	Questionable Data

	V2P3 Head Comp AVy
	deg/s
	1365.2
	81.0
	-896.7
	129.6
	f2=0.287 Hz; f4=0.001 Hz

	V2P3 Head Comp ADy
	deg
	13.3
	109.5
	-44.6
	229.9
	f2=0.287 Hz; f4=0.001 Hz

	V2P3 Upper Neck Fx
	N
	472.7
	94.9
	-226.3
	208.8
	

	V2P3 Upper Neck Fy
	N
	69.2
	138.6
	-163.7
	73.7
	

	V2P3 Upper Neck Fz
	N
	961.5
	87.8
	-133.8
	45.0
	

	V2P3 Upper Neck F Resultant
	N
	986.7
	87.9
	0.0
	-6.6
	

	V2P3 Upper Neck Mx
	N-m
	3.3
	54.4
	-17.0
	96.0
	

	V2P3 Upper Neck My
	N-m
	33.9
	95.6
	-28.7
	115.3
	

	V2P3 Upper Neck Mz
	N-m
	14.8
	132.6
	-9.0
	80.4
	

	V2P3 Upper Neck M Resultant
	N-m
	37.9
	95.6
	0.0
	-38.9
	

	V2P3 Lower Neck Fx
	N
	1006.1
	91.8
	-264.2
	212.1
	

	V2P3 Lower Neck Fy
	N
	254.0
	96.1
	-287.0
	63.4
	

	V2P3 Lower Neck Fz
	N
	526.8
	62.3
	-287.4
	97.2
	

	V2P3 Lower Neck F Resultant
	N
	1027.1
	91.7
	0.1
	-36.1
	

	V2P3 Lower Neck Mx
	N-m
	19.8
	122.5
	-35.8
	70.2
	

	V2P3 Lower Neck My
	N-m
	52.2
	215.5
	-135.4
	88.3
	

	V2P3 Lower Neck Mz
	N-m
	16.9
	96.2
	-17.4
	63.6
	

	V2P3 Lower Neck M Resultant
	N-m
	136.3
	88.3
	0.0
	-46.4
	

	V2P3 Lower Neck x
	g
	31.9
	51.8
	-2.8
	209.8
	

	V2P3 Chest x
	g
	29.6
	52.5
	-2.5
	191.2
	

	V2P3 Chest y
	g
	6.4
	89.1
	-4.8
	56.5
	

	V2P3 Chest z
	g
	8.0
	58.6
	-3.6
	70.4
	

	V2P3 Chest Resultant
	g
	30.7
	52.5
	0.0
	-31.6
	

	V2P3 Chest Compression
	mm
	0.2
	106.3
	-4.7
	206.6
	

	V2P3 Chest AVy
	deg/s
	296.2
	77.2
	-247.0
	186.8
	

	V2P3 Chest Comp AVy
	deg/s
	305.8
	77.3
	-253.1
	187.4
	f2=0.308 Hz; f4=0.001 Hz

	V2P3 Chest Comp ADy
	deg
	9.9
	109.2
	-13.2
	279.8
	f2=0.308 Hz; f4=0.001 Hz

	V2P3 Pelvic x
	g
	32.9
	59.2
	-4.2
	170.8
	

	V2P3 Pelvic y
	g
	3.0
	79.1
	-4.2
	57.8
	

	V2P3 Pelvic z
	g
	15.5
	55.5
	-1.5
	273.4
	

	V2P3 Pelvic Resultant
	g
	35.8
	58.4
	0.0
	-10.9
	

	V2P3 Left Femur
	N
	272.7
	159.8
	-1293.2
	61.7
	

	V2P3 Right Femur
	N
	283.8
	162.1
	-1160.6
	57.6
	

	V2P3 Head to Chest
	deg
	35.2
	214.6
	-16.8
	371.6
	

	V2P3 Lap Belt
	N
	1288.8
	168.1
	-7.8
	44.5
	


Maximum and Minimum Data (Continued)
	NAME
	Unit
	Max
	msec
	Min
	msec
	Comment

	V2 Left X Member x
	g
	35.4
	26.0
	-4.1
	123.6
	

	V2 Left X Member RED x
	g
	35.6
	25.9
	-2.5
	124.5
	

	V2 Right X Member x
	g
	41.9
	26.0
	-7.2
	31.8
	

	V2 Right X Member RED x
	g
	40.5
	26.1
	-9.2
	32.0
	

	V2 Left Front Sill AVy
	deg/s
	68.4
	80.1
	-188.6
	70.8
	

	V2 Left Front Sill Comp AVy
	deg/s
	66.1
	80.1
	-190.1
	70.8
	f2=0.283 Hz; f4=0.001 Hz

	V2 Left Front Sill Comp ADy
	deg
	0.2
	36.5
	-1.7
	174.7
	f2=0.283 Hz; f4=0.001 Hz

	V2P1 Left Seat AVy
	deg/s
	637.5
	60.8
	-277.3
	159.6
	

	V2P1 Left Seat Comp AVy
	deg/s
	655.1
	60.8
	-254.0
	159.6
	f2=0.374 Hz; f4=0.001 Hz

	V2P1 Left Seat Comp ADy
	deg
	18.9
	123.1
	0.0
	9.2
	f2=0.374 Hz; f4=0.001 Hz

	V2P1 Right Seat AVy
	deg/s
	549.6
	60.8
	-211.4
	116.4
	

	V2P1 Right Seat Comp AVy
	deg/s
	563.6
	60.9
	-179.9
	116.4
	f2=0.296 Hz; f4=0.001 Hz

	V2P1 Right Seat Comp ADy
	deg
	19.6
	101.8
	0.0
	7.1
	f2=0.296 Hz; f4=0.001 Hz

	V2P1 Left Link ADy
	deg
	24.7
	123.4
	-9.4
	49.3
	

	V2P1 Right Link ADy
	deg
	16.1
	106.9
	-5.1
	34.1
	

	V2P1 Left Span Bar D
	mm
	379.7
	122.6
	0.0
	-76.7
	

	V2P1 Right Span Bar D
	mm
	322.7
	110.6
	-0.1
	-76.7
	

	V2P3 Left Seat AVy
	deg/s
	1046.1
	67.1
	-756.8
	48.6
	

	V2P3 Left Seat Comp AVy
	deg/s
	1051.0
	67.1
	-755.6
	48.7
	f2=0.3 Hz; f4=0.001 Hz

	V2P3 Left Seat Comp ADy
	deg
	9.9
	103.5
	-3.6
	58.2
	f2=0.3 Hz; f4=0.001 Hz

	V2P3 Right Seat AVy
	deg/s
	396.2
	110.4
	-381.7
	77.8
	

	V2P3 Right Seat Comp AVy
	deg/s
	383.8
	110.4
	-394.5
	77.8
	f2=0.396 Hz; f4=0.001 Hz

	V2P3 Right Seat Comp ADy
	deg
	0.1
	161.9
	-7.4
	103.5
	f2=0.396 Hz; f4=0.001 Hz

	V2P3 Left Link ADy
	deg
	12.8
	128.0
	-51.2
	115.0
	

	V2P3 Right Link ADy
	deg
	23.2
	109.7
	-13.2
	156.0
	

	V2P3 Left Span Bar D
	mm
	189.9
	106.0
	0.0
	-76.7
	

	V2P3 Right Span Bar D
	mm
	126.2
	84.7
	0.0
	-76.7
	

	P1 Left Change in Seat Back Angle WR
	deg
	20.5
	123.0
	0.0
	11.3
	

	P1 Left Change in String Angle
	deg
	5.6
	203.0
	-12.7
	49.3
	

	P1 Left Seat X Displacement
	mm
	249.4
	123.4
	-6.6
	31.6
	

	P1 Right Change in Seat Back Angle W
	deg
	21.2
	99.8
	0.0
	9.1
	

	P1 Right Change in String Angle
	deg
	2.8
	37.2
	-10.3
	100.1
	

	P1 Right Seat X Displacement
	mm
	133.5
	107.0
	-12.0
	33.7
	

	P3 Left Change in Seat Back Angle WR
	deg
	11.4
	103.4
	-3.8
	58.1
	

	P3 Left Change in String Angle
	deg
	10.5
	128.0
	-57.3
	114.5
	

	P3 Left Seat X Displacement
	mm
	106.1
	92.6
	-273.5
	114.3
	

	P3 Right Change in Seat Back Angle W
	deg
	1.8
	161.9
	-5.9
	103.6
	

	P3 Right Change in String Angle
	deg
	27.8
	109.2
	-14.7
	156.1
	

	P3 Right Seat X Displacement
	mm
	184.7
	103.6
	-26.8
	156.3
	


 Table of Data Plots
	PLOT
	PLOT  NAME[UNITS, CHANNEL FILTER CLASS]
	PAGE

	
	
	

	1
	V2P1 Head x [g, CFC_1000]
	B-11

	2
	V2P1 Head y [g, CFC_1000]
	B-12

	3
	V2P1 Head z [g, CFC_1000]
	B-13

	4
	V2P1 Head Resultant [g, CFC_1000]
	B-14

	5
	V2P1 Head AVy
	B-15

	6
	V2P1 Head Comp AVy
	B-16

	7
	V2P1 Head Comp ADy
	B-17

	8
	V2P1 Upper Neck Fx [N, CFC_1000]
	B-18

	9
	V2P1 Upper Neck Fy [N, CFC_1000]
	B-19

	10
	V2P1 Upper Neck Fz [N, CFC_1000]
	B-20

	11
	V2P1 Upper Neck F Resultant [N, CFC_1000]
	B-21

	12
	V2P1 Upper Neck Mx [N-m, CFC_600]
	B-22

	13
	V2P1 Upper Neck My [N-m, CFC_600]
	B-23

	14
	V2P1 Upper Neck Mz [N-m, CFC_600]
	B-24

	15
	V2P1 Upper Neck M Resultant [N-m, CFC_600]
	B-25

	16
	V2P1 Lower Neck Fx [N, CFC_1000]
	B-26

	17
	V2P1 Lower Neck Fy [N, CFC_1000]
	B-27

	18
	V2P1 Lower Neck Fz [N, CFC_1000]
	B-28

	19
	V2P1 Lower Neck F Resultant [N, CFC_1000]
	B-29

	20
	V2P1 Lower Neck Mx [N-m, CFC_600]
	B-30

	21
	V2P1 Lower Neck My [N-m, CFC_600]
	B-31

	22
	V2P1 Lower Neck Mz [N-m, CFC_600]
	B-32

	23
	V2P1 Lower Neck M Resultant [N-m, CFC_600]
	B-33

	24
	V2P1 Lower Neck x [g, CFC_1000]
	B-34

	25
	V2P1 Chest x [g, CFC_180]
	B-35

	26
	V2P1 Chest y [g, CFC_180]
	B-36

	27
	V2P1 Chest z [g, CFC_180]
	B-37

	28
	V2P1 Chest Resultant [g, CFC_180]
	B-38

	29
	V2P1 Chest Displacement [mm, CFC_600]
	B-39

	30
	V2P1 Chest AVy
	B-40

	31
	V2P1 Chest Comp AVy
	B-41

	32
	V2P1 Chest Comp ADy
	B-42

	33
	V2P1 Pelvic x [g, CFC_1000]
	B-43

	34
	V2P1 Pelvic y [g, CFC_1000]
	B-44

	35
	V2P1 Pelvic z [g, CFC_1000]
	B-45

	36
	V2P1 Pelvic Resultant [g, CFC_1000]
	B-46

	37
	V2P1 Left Femur [N, CFC_600]
	B-47

	38
	V2P1 Right Femur [N, CFC_600]
	B-48

	39
	V2P1 Head to Chest
	B-49

	40
	V2P1 Lap Belt [N, CFC_60]
	B-50

	41
	V2P1 Belt Spoolout [mm, CFC_180]
	B-51

	42
	V2P3 Head x [g, CFC_1000]
	B-52

	43
	V2P3 Head y [g, CFC_1000]
	B-53

	44
	V2P3 Head z [g, CFC_1000]
	B-54

	45
	V2P3 Head Resultant [g, CFC_1000]
	B-55

	46
	V2P3 Head AVy
	B-56

	47
	V2P3 Head Comp AVy
	B-57

	48
	V2P3 Head Comp ADy
	B-58


Table of Data Plots (continued)
	PLOT
	PLOT  NAME[UNITS, CHANNEL FILTER CLASS]
	PAGE

	
	
	

	49
	V2P3 Upper Neck Fx [N, CFC_1000]
	B-59

	50
	V2P3 Upper Neck Fy [N, CFC_1000]
	B-60

	51
	V2P3 Upper Neck Fz [N, CFC_1000]
	B-61

	52
	V2P3 Upper Neck F Resultant [N, CFC_1000]
	B-62

	53
	V2P3 Upper Neck Mx [N-m, CFC_600]
	B-63

	54
	V2P3 Upper Neck My [N-m, CFC_600]
	B-64

	55
	V2P3 Upper Neck Mz [N-m, CFC_600]
	B-65

	56
	V2P3 Upper Neck M Resultant [N-m, CFC_600]
	B-66

	57
	V2P3 Lower Neck Fx [N, CFC_1000]
	B-67

	58
	V2P3 Lower Neck Fy [N, CFC_1000]
	B-68

	59
	V2P3 Lower Neck Fz [N, CFC_1000]
	B-69

	60
	V2P3 Lower Neck F Resultant [N, CFC_1000]
	B-70

	61
	V2P3 Lower Neck Mx [N-m, CFC_600]
	B-71

	62
	V2P3 Lower Neck My [N-m, CFC_600]
	B-72

	63
	V2P3 Lower Neck Mz [N-m, CFC_600]
	B-73

	64
	V2P3 Lower Neck M Resultant [N-m, CFC_600]
	B-74

	65
	V2P3 Lower Neck x [g, CFC_1000]
	B-75

	66
	V2P3 Chest x [g, CFC_180]
	B-76

	67
	V2P3 Chest y [g, CFC_180]
	B-77

	68
	V2P3 Chest z [g, CFC_180]
	B-78

	69
	V2P3 Chest Resultant [g, CFC_180]
	B-79

	70
	V2P3 Chest Compression [mm, CFC_600]
	B-80

	71
	V2P3 Chest AVy
	B-81

	72
	V2P3 Chest Comp AVy
	B-82

	73
	V2P3 Chest Comp ADy
	B-83

	74
	V2P3 Pelvic x [g, CFC_1000]
	B-84

	75
	V2P3 Pelvic y [g, CFC_1000]
	B-85

	76
	V2P3 Pelvic z [g, CFC_1000]
	B-86

	77
	V2P3 Pelvic Resultant [g, CFC_1000]
	B-87

	78
	V2P3 Left Femur [N, CFC_600]
	B-88

	79
	V2P3 Right Femur [N, CFC_600]
	B-89

	80
	V2P3 Head to Chest
	B-90

	81
	V2P3 Lap Belt [N, CFC_60]
	B-91

	82
	V2 Left X Member x [g, CFC_60]
	B-92

	83
	V2 Left X Member x Velocity [kph, CFC_180]
	B-93

	84
	V2 Left X Member x Displacement [mm, CFC_180]
	B-94

	85
	V2 Left X Member RED x [g, CFC_60]
	B-95

	86
	V2 Left X Member RED x Velocity [kph, CFC_180]
	B-96

	87
	V2 Left X Member RED x Displacement [mm, CFC_180]
	B-97

	88
	V2 Right X Member x [g, CFC_60]
	B-98

	89
	V2 Right X Member x Velocity [kph, CFC_180]
	B-99

	90
	V2 Right X Member x Displacement [mm, CFC_180]
	B-100

	91
	V2 Right X Member RED x [g, CFC_60]
	B-101

	92
	V2 Right X Member RED x Velocity [kph, CFC_180]
	B-102

	93
	V2 Right X Member RED x Displacement [mm, CFC_180]
	B-103

	94
	V2 Left Front Sill AVy
	B-104

	95
	V2 Left Front Sill Comp AVy
	B-105

	96
	V2 Left Front Sill Comp ADy
	B-106


Table of Data Plots (continued)
	PLOT
	PLOT  NAME[UNITS, CHANNEL FILTER CLASS]
	PAGE

	
	
	

	97
	V2P1 Left Seat AVy
	B-107

	98
	V2P1 Left Seat Comp AVy
	B-108

	99
	V2P1 Left Seat Comp ADy
	B-109

	100
	V2P1 Right Seat AVy
	B-110

	101
	V2P1 Right Seat Comp AVy
	B-111

	102
	V2P1 Right Seat Comp ADy
	B-112

	103
	V2P1 Left Link ADy [deg, CFC_1000]
	B-113

	104
	V2P1 Right Link ADy [deg, CFC_1000]
	B-114

	105
	V2P1 Left Span Bar D [mm, CFC_1000]
	B-115

	106
	V2P1 Right Span Bar D [mm, CFC_1000]
	B-116

	107
	V2P3 Left Seat AVy
	B-117

	108
	V2P3 Left Seat Comp AVy
	B-118

	109
	V2P3 Left Seat Comp ADy
	B-119

	110
	V2P3 Right Seat AVy
	B-120

	111
	V2P3 Right Seat Comp AVy
	B-121

	112
	V2P3 Right Seat Comp ADy
	B-122

	113
	V2P3 Left Link ADy [deg, CFC_1000]
	B-123

	114
	V2P3 Right Link ADy [deg, CFC_1000]
	B-124

	115
	V2P3 Left Span Bar D [mm, CFC_1000]
	B-125

	116
	V2P3 Right Span Bar D [mm, CFC_1000]
	B-126

	117
	P1 Left Change in Seat Back Angle WRT Sill
	B-127

	118
	P1 Left Change in String Angle
	B-128

	119
	P1 Left Seat X Displacement
	B-129

	120
	P1 Right Change in Seat Back Angle WRT Sill
	B-130

	121
	P1 Right Change in String Angle
	B-131

	122
	P1 Right Seat X Displacement
	B-132

	123
	P3 Left Change in Seat Back Angle WRT Sill
	B-133

	124
	P3 Left Change in String Angle
	B-134

	125
	P3 Left Seat X Displacement
	B-135

	126
	P3 Right Change in Seat Back Angle WRT Sill
	B-136

	127
	P3 Right Change in String Angle
	B-137

	128
	P3 Right Seat X Displacement
	B-138


TOP VIEW





TOW ROAD





REAL TIME CAMERA





2





3





1





7





CONCRETE PAD





TEST





VEHICLE





NO STEEL GRATING ALLOWED OVER PHOTO PIT





PHOTO PIT





10





MOVING BARRIER





9





8





LEFT SIDE VIEW





6





7





4, 5








1

[image: image9.bmp]_1123077791.bin

_1149416077.ppt






Film Plane

Seatback Targets

Head Restraint Target

Head Target

Side Header Targets

Reference Target Planes












_1149416327.ppt






Reference Target Locations

























































200 mm





200 mm














_1123501163.ppt








H



B












_1121600695.ppt






a

c

b

d

e (400 mm target)

h-point (vertical 








