REPORT NUMBER: TO2-MGA-2004-011
VEHICLE TO VEHICLE CRASH TESTS 

IN SUPPORT OF COMPATIBILITY RESEARCH

FULL FRONTAL COLLINEAR

Test Date: June 29, 2004
Task Order #T0002 / RFP #0003

1999 DODGE GRAND CARAVAN SE (RX0322)
 TO 2004 HONDA ACCORD EX (R45344)
PREPARED BY:

MGA RESEARCH CORPORATION

5000 WARREN ROAD

BURLINGTON, WI 53105


[image: image41.png]



FINAL REPORT SUBMITTED:  

PREPARED FOR:

U.S. DEPARTMENT OF TRANSPORTATION

VOLPE NATIONAL TRANSPORTATION SYSTEM CENTER

55 BROADWAY, KENDALL SQUARE

CAMBRIDGE, MA 02142

This final test report was prepared for the U.S. Department of Transportation, Volpe National Transportation System Center, in response to Contract Number DTRS57-04-D-30001.
[image: image1.png]



This document is disseminated under the sponsorship of the U.S. Department of Transportation in the interest of information exchange. The United States Government assumes no liability for its content or use thereof.

Prepared by:                                                                     

 


Shefalika Naik, Project Engineer
[image: image13.wmf] 

A

 

D

 

E

 

B

 

F

 

C

 

Lower Edge of Protected Zone

 

FRONT VIEW OF WINDSHIELD

 

Zero Point

 

Y

 

X

 


Reviewed by:                                                                   

 



David Winkelbauer, Project Engineer

Approval Date:  July 14, 2004___________________ 

Technicians:

Wayne Dahlke




Jordan Haynes




Erik Nelson




Chris Novak





Eric Peschman




Clark Subrt




Rick Dodge




Daniel Sienko Jr.

Engineers:

Gary Strassburg



Jeff Lewandowski



Photographic:

Tammy Brausch




Matt Jacobson




Chris Kulis

Instrumentation/
Jamie Aide

   Calibration:

Jim Carney




Jessica Gall





Tim Michnay




David Wilcox

Secretary:

Cyndy Kline

FINAL REPORT ACCEPTED BY:

Accepted By: __________________________________



Contract Technical Manager

Acceptance Date: _______________________________

MGA does not endorse or certify products.  The manufacturer’s name appears solely for identification purposes.
TABLE OF CONTENTS

	Section
	
	Page No

	    1
	Purpose and Summary of the Test
	      1

	    2
	Bullet Vehicle Information / Data Sheets
	      3

	
	
	

	Data Sheet No.
	
	Page No.

	    1
	Bullet Vehicle Test Summary 
	      4

	    2
	Bullet Vehicle Data
	      5

	    3
	Bullet Vehicle Tire Information
	      6

	    4
	Bullet Vehicle Setup Information
	      7

	    5
	Dummy Positioning in Bullet Vehicle
	      9

	    6
	Bullet Vehicle Seat Belt Positioning Data
	     11

	    7
	Bullet Vehicle Accelerometer Location and Data Summary
	     12

	    8
	Bullet Vehicle Hybrid III ATD Injury Criteria and Sensor Data
	     14

	    9
	Bullet Vehicle Seat Belt Assessment and Post Test Airbag Data
	     18

	   10
	Bullet Vehicle FMVSS 212 Data
	     19

	   11
	Bullet Vehicle FMVSS 219 (partial) Data
	     20

	   12
	Bullet Vehicle Measurements
	     21

	   13
	Bullet Vehicle Photographic Reference Target Locations
	     25

	   14
	Bullet Vehicle Intrusion Measurements
	     26

	   15
	Bullet Vehicle IIHS Measurements
	     31

	
	
	

	Section
	
	

	    3
	Target Vehicle Information / Data Sheets
	     33

	
	
	

	Data Sheet No.
	
	

	    16
	Target Vehicle Test Summary 
	     34

	    17
	Target Vehicle Data
	     35

	    18
	Target Vehicle Tire Information
	     36

	    19
	Target Vehicle Setup Information
	     37

	    20
	Dummy Positioning in Target Vehicle
	     39

	    21
	Target Vehicle Seat Belt Positioning Data
	     41

	    22
	Target Vehicle Accelerometer Location and Data Summary
	     42

	    23
	Target Vehicle Hybrid III ATD Injury Criteria and Sensor Data
	     44

	    24
	Target Vehicle Seat Belt Assessment and Post Test Airbag Data
	     49

	    25
	Target Vehicle FMVSS 212 Data
	     50


TABLE OF CONTENTS (Continued)
	Data Sheet No.
	
	Page No.

	   26
	Target Vehicle FMVSS 219 (partial) Data
	     51

	   27
	Target Vehicle Measurements
	     52

	   28
	Target Vehicle Photographic Reference Target Locations
	     56

	   29
	Target Vehicle Intrusion Measurements
	     57

	   30
	Target Vehicle IIHS Measurements
	     62

	   31
	Camera Locations

	     64

	
	
	

	Appendix
	
	

	    A
	Photographs
	      A

	    B
	Bullet Vehicle Data Traces
	      B

	    C
	Target Vehicle Data Traces
	      C

	    D
	Dummy Calibration Data Traces and Tables
	      D

	    E
	Test Equipment and Instrumentation Calibration
	      E

	
	
	


SECTION 1

PURPOSE AND SUMMARY OF TEST

PURPOSE


This vehicle to vehicle crash test was conducted as part of Contract No.: DTRS57-04-D-30001, RFP 3, task order 2, sponsored by the U.S. Department of Transportation, Volpe National Transportation System Center.  The purpose of this testing is to evaluate and compare vehicle and occupant responses in vehicle to vehicle crash tests against full frontal rigid barrier crash tests conducted using a high resolution load cell barrier.  This task order plans for eighteen vehicle to vehicle crash tests in support of the vehicle compatibility research plan.  There will be six tests each in three test configurations; full frontal collinear, fifty percent offset collinear, and FMVSS 214 configuration side impact.  For all tests the target vehicle will be stationary.
SUMMARY


A 1999 Dodge Grand Caravan SE impacted a 2004 Honda Accord EX in the full frontal collinear configuration.  The test was performed at MGA Research Corporation on June 29, 2004.

The bullet vehicle (1999 Dodge Grand Caravan SE) contained one Part 572E, 50th percentile male anthropomorphic test device (ATD) placed in the driver seating position and one Part 572O, 5th percentile female ATD placed in the right front seating position.  These dummies were positioned according to dummy placement instructions specified in the FMVSS 208 TP-12 Laboratory Indicant Test Procedure.

The target vehicle (2004 Honda Accord EX) contained one Part 572E, 50th percentile male anthropomorphic test device (ATD) with THOR Lx lower leg instrumentation placed in the driver seating position and one Part 572O 5th percentile female ATD placed in the right front seating position.  These dummies were positioned according to dummy placement instructions specified in the FMVSS 208 TP-12 Laboratory Indicant Test Procedure.


Appendix A contains the test photos.  Appendices B and C contain the data plots for the bullet and target vehicles respectively.  The dummy calibration data is located in Appendix D and the dummy instrumentation is summarized in Appendix E.

The remainder of this report contains data sheets that document the test conditions.
TEST NOTES
The driver airbag deployed but had a large tear in the fabric. 

No valid data was collected for the following channels:

Bullet:  Driver Left Knee Shear

Right Brake Caliper X after 40 msec.


Bottom of Engine X after 40 msec.

Target: Driver Left Upper Tibia Moment X

 Bottom of Engine X

 
 Driver Left Mid-Foot Z after 55 msec.

 Right Rear Seat Crossmember X after 30 msec
SECTION 2

BULLET VEHICLE INFORMATION / DATA SHEETS

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


CONVERSION FACTORS USED IN THIS REPORT*

	Quantity
	Typical Application
	English Units
	Metric Unit
	Multiply By

	Mass
	Vehicle Weight
	lb
	kg
	0.4536

	Linear Velocity
	Impact Velocity
	mile/h
	km/h
	1.609

	Length or Distance
	Measurements 
	in
	mm
	25.4

	Volume
	Fuel Systems
	gal
	liter
	3.785

	Volume
	Small Fluids
	oz
	mL
	29.573

	Pressure
	Tire Pressure
	lbf/in2
	kPa
	7.0

	Volume
	Liquid
	gal
	liter
	3.785

	Temperature
	General Use
	°F
	°C
	=(tf -32)/1.8

	Force
	Dynamic Forces
	lbf
	N
	4.448

	Moment
	Torque
	lbf/ft
	Nm
	1.355


*Based on the Recommended Practice in SAE J916, May 85

DATA SHEET NO. 1

BULLET VEHICLE TEST SUMMARY

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


DOOR OPENING AND SEAT TRACK INFORMATION

	Description
	Driver
	Passenger

	Locked/Unlocked Doors
	Door was unlocked
	Door was unlocked

	Front Door Opening
	Door remained closed and latched; Door opened without tools
	Door remained closed and latched; Door opened without tools

	Rear Door Opening
	Door remained closed and latched; Door opened without tools
	Door remained closed and latched; Door opened without tools

	Seat Track Shift
	0
	0


TEST DUMMY INFORMATION

	Description
	Driver
	Passenger

	Dummy Type / Serial No.
	HIII 50th/ 110
	HIII 5th/ 324

	Head Contact
	Airbag, Head Rest
	Airbag, Head Rest

	Chest Contact
	Airbag
	Airbag

	Abdomen Contact
	Airbag
	Airbag

	Left Knee Contact
	Knee Bolster, Steering Column
	Glovebox

	Right Knee Contact
	Knee Bolster, Steering Column
	Glovebox


INSTRUMENTATION
	Driver ATD Sensors
	57

	Passenger ATD Sensors
	36

	Belt Assessment Sensors
	4

	Vehicle Structure Accelerometers
	13

	Timing Channels
	1

	Total
	111


TEST SPEED

	Measured Parameter
	Units
	Requirement
	Value

	Trap No. 1 Velocity (Primary)
	km/h
	101.7
	101.2


	SpeedBullet  = 
	(WeightBullet + WeightTarget)
	* 56 kph
	=
	(1992.6 + 1626.7)
	* 56 kph

	
	WeightBullet
	
	
	1992.6
	


DATA SHEET NO. 2

BULLET VEHICLE DATA

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


  VEHICLE INFORMATION 

                                      VEHICLE OPTIONS

	Make
	Chrysler
	
	Driver Airbag
	Yes

	Model
	Dodge Grand Caravan SE
	
	Passenger Airbag
	Yes

	Body Style
	Van
	
	Power Windows
	Yes

	NHTSA No.
	RX0322
	
	Power Steering
	Yes

	VIN
	1B4GP44G2XB574877
	
	Power Door Locks
	Yes

	Color
	Maroon
	
	Tilt Wheel
	Yes

	Odometer Reading (mile)
	76,002
	
	Air Conditioning
	Yes

	Transmission
	Automatic
	
	Power Brakes
	Yes

	Final Drive
	Front
	
	Disc Brakes, Front
	Yes

	Number of Cylinders
	6
	
	Disc Brakes, Rear
	No

	Engine Displacement (L)
	3.3
	
	Anti-lock Brakes
	Yes

	Engine Placement
	Lateral
	
	AM/FM/CD
	Yes


DATA FROM CERTIFICATION LABEL

	Manufactured By
	DaimlerChrysler Corporation

	Date of Manufacture
	09/98


1

	GVWR (kg)
	2432

	GAWR Front (kg)
	1246

	GAWR Rear (kg)
	1246


DATA FROM TIRE PLACARD

	Measured Parameter
	Front
	Rear

	Cold Pressure (kPa)
	241
	241

	Recommended Tire Size
	P215/65R15
	P215/65R15

	Tire Size on Vehicle
	P215/65R15
	P215/65R15

	Tire Manufacturer
	GoodYear
	GoodYear


	Measured Parameter
	Front
	Rear
	Third
	Total

	Type of Seats
	Bucket
	Bench
	Bench
	

	Number Of Occupants
	2
	2
	3
	7


DATA SHEET NO. 3
BULLET VEHICLE TIRE INFORMATION

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


	Vehicle Year
	1999
	Vehicle Make
	Chrysler

	VIN
	1B4GP44G2XB574877
	Vehicle Model
	Dodge Grand Caravan SE


[image: image14.wmf] 

I

 

J

 

K

 

C

 

F

 

D

 

E

 

A

 

B

 

B

 

H

 

M

 

G

 

L

 

N

 

O

 

P

 

Q

 


	
	Front
	Rear

	Tire Manufacturer
	GoodYear
	GoodYear

	Tire Name
	Weather Handler CS
	Weather Handler CS

	Tire Type
	P
	P

	Tire Width (mm)
	215
	215

	Ratio of Height to Width (aspect ratio)
	65
	65

	Radial
	R
	R

	Wheel Diameter
	15
	15

	Load Index & Speed Symbol
	95T
	95T

	Treadwear
	560
	560

	Traction Grade
	A
	A

	Temperature Grade
	B
	B


DATA SHEET NO. 4
BULLET VEHICLE SETUP INFORMATION

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX


VEHICLE WEIGHTS

	
	Units
	As Delivered (UVW) (Axle)
	As Tested (ATW) (Axle)

	
	
	Front
	Rear 
	Total
	Front 
	Rear
	Total

	Left
	kg
	545.2
	392.8
	
	593.8
	431.8
	

	Right
	kg
	505.8
	384.2
	
	545.2
	421.8
	

	Ratio
	%
	57.5
	42.5
	
	57.2
	42.8
	

	Totals
	kg
	1051.0
	777.0
	1828.0
	1139.0
	853.6
	1992.6


TARGET TEST WEIGHT CALCULATION

	Measured Parameter
	Units
	Value

	Total Delivered Weight (UVW)
	kg
	1828.0

	Weight of 1 P572E & 1 P572O ATDs
	kg
	127.0

	Rated Cargo/Luggage Weight (RCLW)
	kg
	45.4

	Calculated Vehicle Target Weight (TVTW)
	kg
	2004.4


VEHICLE ATTITUDES AND CG

	
	Units
	LF
	RF
	LR
	RR
	CG(aft of front axle)

	As Delivered
	mm
	748
	751
	768
	770
	1291

	As Tested
	mm
	737
	742
	756
	758
	1301

	Post Test
	mm
	800
	865
	835
	818
	


Vehicle Wheelbase (mm): 






   
3036
Weight of Ballast secured in cargo area (kg): 





  0

Vehicle Components Removed: Third Seat, Rear Carpeting, Hub Covers, jack
Ballast weight does not include instrumentation, data acquisition system, and onboard cameras.
DATA SHEET NO. 4... (Continued)

BULLET VEHICLE SETUP INFORMATION

SEAT BACK POSITION

Driver seat back angle:  24.9°  on seat back
Passenger seat back angle:  21.2°  on seat back
SEAT FORE/AFT POSITIONS

The driver and passenger seat travel are manually adjustable.
Driver seat fore/aft total travel: 
   23 positions 

Passenger seat fore/aft total travel: 
   23 positions 

Driver seat fore/aft position: 

    12th position of 23 total
Passenger seat fore/aft position: 
   full forward
SEAT BELT UPPER ANCHORAGE

The driver anchorage was placed in the middle position. The passenger anchorage was placed full down.
FUEL TANK CAPACITY DATA

[image: image15.wmf] 

A

 

B

 

C

 

D

 

5

 

4

 

3

 

2

 

1

 

The fuel tank was filled with 26.5 liters of water for ballast.
STEERING COLUMN ADJUSTMENT

Lowermost. Position 1:   18.7° 

Geometric Center, Position 2:   27.1°
Uppermost, Position 3:   36.7°
DATA SHEET NO. 5
DUMMY POSITIONING IN BULLET VEHICLE

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


DUMMY MEASUREMENTS FOR FRONT SEAT OCCUPANTS


[image: image2.wmf] 

PASS

 

DRVR

 

Vertical Longitudinal Planes

 

Vertical

 

Transverse

 

Plane

 

HS

 

KK

 

AD

 

HD

 

HR

 

SHY

 

STRIKER

 

HZ

 

CD

 

KD

 

CS

 

Seat Back Angle Line

 

SH

 

NR

 

SK

 

AD

 

Arm to Door

 

HD

 

H

-

Point to Door

 

HR

 

Head to Side Header

 

HS

 

Head to Side Window

 

KK

 

Knee to Knee

 

SHY

 

St

riker to H

-

Point

 

          (Y Axis)

 

CD

 

Chest to Dash

 

CS

 

Chest to Steering Wheel Hub

 

HH

 

Head to Header

 

HW

 

Head to Windshield

 

HZ

 

Head to Roof

 

KDA

 

Knee to Dash Angle

 

KDL

 

Left Knee to Dash

 

KDR

 

Right Knee to Dash

 

NA

 

Nose to Rim Angle

 

NR

 

Nose to Rim

 

PA

 

Pelvic Angle

 

RA

 

Rim to Abdomen

 

SA

 

Seat Back Angle

 

SCA

 

Steering Column Angle

 

SH

 

Striker to H

-

Point

 

SK

 

Striker t

o Knee

 

ST

 

Striker to Head

 

SWA

 

Steering Wheel Angle

 

TA

 

Tibial Angle

 

WA

 

Windshield Angle

 

 SWA

°

 

  HH

 

 WA

°

 

 HW

 

 NA

°

 

 SCA

°

 

 KDA

°

 

 TA

°

 

 SA

°

 

ST

 

 PA

°

 


DATA SHEET NO. 5... (continued)

DUMMY POSITIONING IN BULLET VEHICLE

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


TEST DUMMY POSITION MEASUREMENTS

	Code
	Measurement Description
	Driver
	Passenger

	
	
	Length (mm)
	Angle (°)
	Length (mm)
	Angle (°)

	WA
	Windshield Angle
	
	28.6
	
	

	SWA
	Steering Wheel Angle
	
	62.5
	
	

	SCA
	Steering Column Angle
	
	27.1
	
	

	SA
	Seat Back Angle
	
	24.9
	
	21.2

	HZ
	Head to Roof (Z)
	206
	90.0
	240
	90.0

	HH
	Head to Header
	349
	23.6
	316
	46.8

	HW
	Head to Windshield
	634
	0.0
	680
	0.0

	HR
	Head to Side Header (Y)
	232
	
	257
	

	NR
	Nose to Rim
	399
	12.8
	
	

	CD
	Chest to Dash
	558
	
	452
	

	CS
	Chest to Steering Hub
	301
	13.3
	
	

	RA
	Rim to Abdomen
	153
	0.0
	
	

	KDL
	Left Knee to Dash
	172
	35.3
	133
	

	KDR
	Right Knee to Dash
	193
	
	119
	17.4

	PA
	Pelvic Angle
	
	22.5
	
	27.8

	TA
	Tibia Angle
	
	59.1
	
	62.1

	KK
	Knee to Knee (Y)
	306
	
	223
	

	SK
	Striker to Knee
	635
	80.2
	701
	84.2

	ST
	Striker to Head
	648
	9.6
	602
	23.4

	SH
	Striker to H-Point
	239
	91.9
	356
	90.4

	SHY
	Striker to H-Point (Y)
	271
	
	314
	

	HS
	Head to Side Window
	341
	
	319
	

	HD
	H-Point to Door (Y)
	146
	
	171
	

	AD
	Arm to Door (Y)
	138
	
	81
	

	AA
	Ankle to Ankle
	306
	
	142
	


DATA SHEET NO. 6
BULLET VEHICLE SEAT BELT POSITIONING DATA

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344



[image: image3.wmf] 

 

Dummy’s Centerline

 

TBI

 

'D' Ring

 

Shoulder Belt 

Portion

 

Lap Belt 

Portion

 

1/8" Thick 

Aluminum 

Plate

 

Inboard 

Anchorage

 

Flo

orpan

 

Buckle 

Assembly

 

PBL

 

PBU

 

Outboard 

Anchor

 

Locking 

Retractor

 


SEAT BELT POSITIONING MEASUREMENTS

	Measurement Description
	Units
	Driver
	Passenger

	PBU - Top surface of reference to belt upper edge
	mm
	329
	275

	PBL - To surface of reference to belt lower edge
	mm
	260
	198


DATA SHEET NO. 7

BULLET VEHICLE ACCELEROMETER LOCATION AND DATA SUMMARY

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


ACCELEROMETER PEAK DATA (CFC 60)
	 Accelerometer Location 
	Peak Values

	
	Units
	Max
	Time
	Min
	Time

	 Left Rear Seat Crossmember X
	G's
	3.3
	129
	-34.2
	39

	 Right Rear Seat Crossmember X
	G's
	3.0
	129
	-32.4
	37

	 Front Brake Caliper Left X
	G's
	44.0
	55
	-105.6
	40

	 Front Brake Caliper Right X
	G's
	*
	*
	*
	*

	 Dash Panel Center X
	G's
	16.3
	63
	-64.8
	50

	 Top Engine X
	G’s
	48.3
	46
	-138.8
	34

	 Bottom Engine X
	G’s
	*
	*
	*
	*

	 Toe Pan (Accelerator) X
	G’s
	12.0
	64
	-65.8
	42

	 Toe Pan (Accelerator) Z
	G’s
	28.4
	58
	-20.7
	47

	 Toe Pan (Footrest) X
	G’s
	19.8
	48
	-99.2
	41

	 Toe Pan (Footrest) Z
	G’s
	27.1
	52
	-48.6
	47

	 Rear Deck Z
	G’s
	20.1
	62
	-17.1
	51

	 Rear Compartment Center X
	G’s
	3.2
	130
	-32.4
	38


* No Valid Data Collected after 40 msec

DATA SHEET NO. 7… (Continued)

BULLET VEHICLE ACCELEROMETER LOCATION AND DATA SUMMARY

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


ACCELEROMETER LOCATIONS
	 Accelerometer Location 
	Pre Test 

Measurements (mm)
	Post Test Measurements (mm)

	
	X
	Y
	Z
	X
	Y
	Z

	 Left Rear Seat Crossmember 
	2322
	-305
	465
	2322
	-305
	510

	 Right Rear Seat Crossmember
	2322
	305
	464
	2322
	305
	527

	 Front Brake Caliper Left 
	4196
	-680
	256
	4098
	-663
	277

	 Front Brake Caliper Right
	4196
	680
	256
	4122
	700
	264

	 Dash Panel Center 
	3758
	0
	1150
	3754
	0
	1244

	 Top Engine 
	4292
	80
	882
	4170
	106
	925

	 Bottom Engine 
	4283
	0
	214
	4054
	15
	183

	 Toe Pan (Accelerator) 
	3778
	-212
	472
	3667
	-283
	538

	 Toe Pan (Footrest) 
	3898
	-480
	592
	3786
	-513
	647

	 Rear Deck 
	385
	-70
	560
	385
	-70
	628

	 Rear Compartment Center
	2790
	0
	432
	2792
	0
	493


Reference Points:
X - Rear Surface of Vehicle (+ forward)




Y - Vehicle Centerline
(+ to right)





Z - Ground Plane (+ up)

DATA SHEET NO. 8

BULLET VEHICLE HYBRID III ATD INJURY CRITERIA AND SENSOR DATA

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


HEAD PRIMARY PEAK ACCELERATIONS

	Location
	Axis
	Units
	Driver
	Passenger

	
	
	
	Max
	Time
	Min
	Time
	Max
	Time
	Min
	Time

	Head CG
	X
	G's
	28.2
	195
	-63.7
	85
	34.6
	165
	-74.8
	79

	Head CG
	Y
	G's
	6.3
	197
	-5.6
	96
	31.1
	162
	-28.5
	89

	Head CG
	Z
	G's
	15.5
	75
	-8.0
	86
	24.2
	78
	-4.8
	91

	Head CG Resultant
	N/A
	G's
	64.0
	85
	
	
	78.5
	79
	
	


HEAD INJURY CRITERIA (HIC)

	Location
	Driver
	Passenger

	
	HIC
	Avg. G's
	T1
	T2
	HIC
	Avg. G's
	T1
	T2

	HIC 36
	513.3
	50.7
	70.6
	98.6
	777.7
	59.7
	72.0
	100.3

	HIC 15
	381.2
	57.8
	76.0
	91.0
	577.6
	68.3
	74.4
	89.4


HEAD PEAK ACCELERATIONS

	Location
	Axis
	Units
	Driver
	Passenger

	
	
	
	Max
	Time
	Min
	Time
	Max
	Time
	Min
	Time

	Head Front
	X
	G's
	27.8
	195
	-63.4
	84
	33.4
	166
	-77.0
	79

	Head Front
	Y
	G's
	2.5
	253
	-6.7
	68
	22.5
	45
	-36.6
	88

	Head Front
	Z
	G's
	22.1
	62
	-4.4
	86
	38.2
	78
	-16.0
	48

	Head Front Res
	N/A
	G's
	63.6
	85
	
	
	84.9
	78
	
	


HEAD PEAK ACCELERATIONS

	Location
	Axis
	Units
	Driver
	Passenger

	
	
	
	Max
	Time
	Min
	Time
	Max
	Time
	Min
	Time

	Head Left
	X
	G's
	24.2
	195
	-63.8
	84
	19.4
	169
	-78.4
	81

	Head Left
	Y
	G's
	5.2
	198
	-5.5
	89
	31.4
	162
	-23.7
	88

	Head Left
	Z
	G's
	16.3
	65
	-13.4
	87
	24.2
	72
	-7.0
	91

	Head Left Res
	N/A
	G's
	64.8
	85
	
	
	80.6
	81
	
	


HEAD PEAK ACCELERATIONS

	Location
	Axis
	Units
	Driver
	Passenger

	
	
	
	Max
	Time
	Min
	Time
	Max
	Time
	Min
	Time

	Head Upper
	X
	G's
	42.9
	195
	-63.2
	81
	51.0
	162
	-84.0
	83

	Head Upper
	Y
	G's
	8.3
	198
	-9.4
	100
	52.8
	162
	-35.4
	84

	Head Upper
	Z
	G's
	16.3
	65
	-7.8
	87
	27.5
	78
	-4.7
	91

	Head Upper Res
	N/A
	G's
	63.4
	76
	
	
	91.1
	85
	
	


DATA SHEET NO. 8... (Continued)

BULLET VEHICLE HYBRID III ATD INJURY CRITERIA AND SENSOR DATA

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


UPPER NECK PEAK FORCES AND MOMENTS

	 Location
	Axis
	Units
	Driver
	Passenger

	
	
	
	Max
	Time
	Min
	Time
	Max
	Time
	Min
	Time

	 Neck Force
	X
	Newtons
	442
	70
	-108
	59
	1102
	84
	-203
	183

	 Neck Force
	Y
	Newtons
	127
	78
	-72
	133
	338
	105
	-218
	181

	 Neck Force
	Z
	Newtons
	2013
	81
	-58
	265
	1158
	73
	-580
	95

	 Neck Moment
	X
	N•m
	4.4
	199
	-12.9
	100
	9.6
	89
	-19.8
	71

	 Neck Moment
	Y
	N•m
	39.9
	70
	-13.1
	225
	83.3
	86
	-18.0
	184

	 Neck Moment
	Z
	N•m
	9.6
	111
	-5.1
	160
	27.0
	108
	-4.8
	300


LOWER NECK PEAK FORCES AND MOMENTS

	 Location
	Axis
	Units
	Driver
	Passenger

	
	
	
	Max
	Time
	Min
	Time
	Max
	Time
	Min
	Time

	 Neck Force
	X
	Newtons
	592
	71
	-94
	131
	661
	86
	-269
	185

	 Neck Force
	Y
	Newtons
	274
	75
	-118
	114
	92
	126
	-344
	90

	 Neck Force
	Z
	Newtons
	2365
	78
	-72
	264
	1623
	73
	-575
	185

	 Neck Moment
	X
	N•m
	22.7
	77
	17.5
	114
	20.2
	120
	-19.3
	60

	 Neck Moment
	Y
	N•m
	19.1
	136
	-147.7
	78
	42.9
	184
	-91.3
	72

	 Neck Moment
	Z
	N•m
	20.9
	92
	-4.0
	159
	31.0
	112
	-15.7
	60


CHEST PRIMARY PEAK ACCELERATIONS

	Location
	Axis
	Units
	Driver
	Passenger

	
	
	
	Max
	Time

12

	Min
	Time
	Max
	Time
	Min
	Time

	Chest CG
	X
	G's
	5.6
	197
	-46.5
	80
	5.5
	176
	-48.9
	61

	Chest CG
	Y
	G's
	1.4
	108
	-5.0
	86
	4.9
	51
	-7.4
	73

	Chest CG
	Z
	G's
	8.2
	58
	-4.2
	84
	5.7
	78
	-4.6
	116

	Chest CG Resultant
	N/A
	G's
	46.7
	75
	
	
	49.0
	61
	
	


CHEST PEAK DISPLACEMENTS

	 Location
	Axis
	Units
	Driver
	
	Passenger

	
	
	
	Max
	Time
	Min
	Time
	Max
	Time
	Min
	Time

	 Chest 
	X
	mm
	
	
	-35.4
	92
	
	
	-12.7
	74


CHEST CLIP (3MSEC)

	Location
	Driver
	Passenger

	
	CLIP
	T1
	T2
	CLIP
	T1
	T2

	Chest CG Primary
	46.4
	77.1
	80.4
	43.1
	59.3
	62.3


SEAT BELT SENSOR PEAK VALUES
	 Location
	Axis
	Units
	Driver
	Passenger

	
	
	
	Max
	Time
	Min
	Time
	Max
	Time
	Min
	Time

	 Lap Belt Force
	N/A
	Newtons
	5334
	67
	
	
	2670
	58
	
	

	 Shoulder Belt Force
	N/A
	Newtons
	2968
	77
	
	
	3034
	59
	
	


DATA SHEET NO. 8... (Continued)

BULLET VEHICLE HYBRID III ATD INJURY CRITERIA AND SENSOR DATA

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


FEMUR PEAK FORCES

	 Location
	Axis
	Units
	Driver
	Passenger

	
	
	
	Max
	Time
	Min
	Time
	Max
	Time
	Min
	Time

	 Left Femur
	Z
	Newtons
	814
	49
	-5397
	62
	348
	46
	-5976
	54

	 Right Femur
	Z
	Newtons
	251
	49
	-5290
	65
	530
	47
	-4401
	61


UPPER AND LOWER TIBIA PEAK FORCES AND MOMENTS

	 Location
	Axis
	Units
	Driver
	Passenger

	
	
	
	Max
	Time
	Min
	Time
	Max
	Time
	Min
	Time

	Left Lower Moment
	X
	N•m
	15.0
	48
	-77.7
	57
	
	
	
	

	Left Lower Moment
	Y
	N•m
	152.6
	65
	-48.9
	48
	
	
	
	

	Left Lower Force
	X
	Newtons
	1079
	55
	-82
	147
	
	
	
	

	Left Lower Force 
	Y
	Newtons
	61
	128
	-414
	63
	
	
	
	

	Left Lower Force

Left Lower Force
	Z
	Newtons
	260
	48
	-1877
	58
	
	
	
	

	Left Upper Moment
	X
	N•m
	49.5
	49
	-16.5
	59
	
	
	
	

	Left Upper Moment
	Y
	N•m
	24.8
	145
	-107.3
	69
	
	
	
	

	Left Upper Force 
	X
	Newtons
	171
	48
	-716
	65
	
	
	
	

	Left Upper Force
	Y
	Newtons
	470
	57
	-71
	102
	
	
	
	

	Left Upper Force
	Z
	Newtons
	204
	46
	-1615
	59
	
	
	
	

	Right Lower Moment
	X
	N•m
	82.4
	68
	-17.3
	50
	
	
	
	

	Right Lower Moment
	Y
	N•m
	101.0
	47
	-482.3
	65
	
	
	
	

	Right Lower Force 
	X
	Newtons
	1933
	69
	-75
	163
	
	
	
	

	Right Lower Force 
	Y
	Newtons
	192
	67
	-148
	56
	
	
	
	

	Right Lower Force
	Z
	Newtons
	132
	153
	-2860
	68
	
	
	
	

	Right Upper Moment
	X
	N•m
	36.6
	90
	-22.1
	51
	
	
	
	

	Right Upper Moment
	Y
	N•m
	66.1
	62
	-168.2
	49
	
	
	
	

	Right Upper Force
	X
	Newtons
	1841
	66
	-509
	50
	
	
	
	

	Right Upper Force
	Y
	Newtons
	133
	47
	-456
	55
	
	
	
	

	Right Upper Force
	Z
	Newtons
	65
	174
	-3327
	72
	
	
	
	


KNEE SHEAR DISPLACEMENT

	Location
	Axis
	Units
	Driver
	Passenger

	
	
	
	Max
	Time
	Min
	Time
	Max
	Time
	Min
	Time

	Left
	Z
	mm
	
	
	
	
	
	
	
	

	Right
	Z
	mm
	
	
	-8.3
	63
	
	
	
	


DATA SHEET NO. 8... (Continued)

BULLET VEHICLE HYBRID III ATD INJURY CRITERIA AND SENSOR DATA

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


HEAD REDUNDANT PEAK ACCELERATIONS

	Location
	Axis
	Units
	Driver
	Passenger

	
	
	
	Max
	Time
	Min
	Time
	Max
	Time
	Min
	Time

	Head CG
	X
	G's
	28.4
	195
	-63.3
	85
	34.9
	165
	-73.6
	79

	Head CG
	Y
	G's
	7.1
	230
	-11.7
	84
	31.3
	162
	-27.9
	89

	Head CG
	Z
	G's
	15.9
	75
	-7.4
	87
	25.4
	78
	-4.0
	48

	Head CG Resultant
	N/A
	G's
	64.2
	84
	
	
	77.7
	79
	
	


CHEST REDUNDANT PEAK ACCELERATIONS

	Location
	Axis
	Units
	Driver
	Passenger

	
	
	
	Max
	Time

12

	Min
	Time
	Max
	Time
	Min
	Time

	Chest CG
	X
	G's
	5.7
	197
	-46.8
	79
	6.7
	176
	-48.0
	61

	Chest CG
	Y
	G's
	1.8
	101
	-3.7
	87
	4.7
	51
	-7.3
	73

	Chest CG
	Z
	G's
	7.2
	57
	-6.7
	84
	5.6
	78
	-4.6
	116

	Chest CG Resultant
	N/A
	G's
	46.9
	80
	
	
	48.2
	61
	
	


REDUNDANT HEAD INJURY CRITERIA (HIC)

	Location
	Driver
	Passenger

	
	HIC
	Avg. G's 
	T1
	T2
HIC
	HIC
	Avg. G's
	T1
	T2

	HIC 36
	506.5
	50.7
	70.6
	98.3
	726.2
	58.5
	72.0
	99.7

	HIC 15
	375.0
	57.4
	76.1
	91.1
	555.3
	67.2
	74.2
	89.2


REDUNDANT CHEST CLIP (3MSEC)

	Location
	Driver
	Passenger

	
	CLIP
	T1
	T2
	CLIP
	T1
	T2

	Chest CG Redundant
	46.4
	77.5
	80.5
	41.9
	59.3
	62.3


NECK INJURY CRITERIA

	Location
	Driver
	Passenger

	Nte (Tension – Extension)
	0.4
	0.1

	Ntf  (Tension – Flexion)
	0.3
	0.3

	Nce (Compression – Extension)
	0.1
	0.4

	Ncf  (Compression – Flexion)
	0.0
	0.5


DATA SHEET NO. 9
BULLET VEHICLE SEAT BELT ASSESSMENT AND POST TEST AIRBAG DATA

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


BELT LENGTH DATA

	Measurement Description
	Units
	Driver
	Passenger

	Shoulder belt length as measured on ATD
	mm
	862
	885

	Lap belt length as measured on ATD
	mm
	890
	925

	Remainder of belt 
	mm
	3955
	3875

	Total belt length for continuous webbing systems
	mm
	5707
	5685


BELT SPOOL-OUT DATA

	Measurement Description
	Units
	Driver
	Passenger

	Pretensioner
	mm
	
	

	At the Retractor
	mm
	169
	315

	At the Shoulder
	mm
	197
	320


POST TEST AIRBAG DATA
	Number of Vent Holes

	Driver
	0
	Passenger
	0

	Size of Vent Holes (Note whether round or square)

	Driver
	N/A
	Passenger
	N/A

	Total Vent Area

	Driver
	N/A
	Passenger
	N/A

	Deflated Airbag Length and Width Dimensions or, if Round, Diameter

	Driver
	Length:  
	Width: 
	Diameter: 665 mm

	Passenger
	Length:  625 mm
	Width:  463 mm
	Diameter:

	Is the Airbag Tethered?

	Driver
	Yes
	
	Length: 275 mm

	Passenger
	
	No
	Length:


DATA SHEET NO. 10
BULLET VEHICLE FMVSS 212 DATA

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


Windshield Mounting Details:

Windshield glass is secured to the vehicle frame with a rubber trim and glue.

The standard requires that the post-test retention measurement be a minimum of 75 percent of the pretest total periphery measurement for vehicles not equipped with occupant passive restraints and 50 percent for each side of the windshield for vehicles, which are equipped with occupant passive restraints.

WINDSHIELD PERIPHERY MEASUREMENTS

	Measurement
	Pre-Test (mm)
	Post-Test (mm)
	% of Retention

	Left Side
	2331
	2331
	100

	Right Side
	2331
	2331
	100

	Total
	4662
	4662
	100


[image: image16.wmf] 

A

 

B

 

C

 

D

 

5

 

4

 

3

 

2

 

1

 


[image: image17.wmf] 

X

 

Y

 

10

 

4

 

5

 

6

 

2

 

1

 

3

 

8

 

9

 

7

 

16

 

15

 

14

 

13

 

12

 

11

 

7

 

[image: image18.wmf] 

A

 

C

 

B

 

FRONT VIEW OF WINDSHIELD

 

Zero Point

 

Y

 

X

 

[image: image19.wmf][image: image20.png]


[image: image21.wmf] 

I

 

J

 

K

 

C

 

F

 

D

 

E

 

A

 

B

 

B

 

H

 

M

 

G

 

L

 

N

 

O

 

P

 

Q

 

[image: image22.wmf] 

A

 

C

 

B

 

FRONT VIEW OF WINDSHIELD

 

Zero Point

 

Y

 

X

 


	Item
	Units
	Segment Length
	Molding Width

	A
	mm
	1240
	11

	B
	mm
	1648
	11

	C
	mm
	887
	12


DATA SHEET NO. 11
	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


BULLET VEHICLE FMVSS 219 (Partial) DATA

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX


	Item
	Units 
	Value

	A
	mm
	1240

	B
	mm
	521

	C
	mm
	1648

	D
	mm
	887

	E
	mm
	601

	F
	mm
	675


[image: image23.wmf] 

A

 

D

 

E

 

B

 

F

 

C

 

Lower Edge of Protected Zone

 

FRONT VIEW OF WINDSHIELD

 

Zero Point

 

Y

 

X

 


AREA OF PROTECTED ZONE FAILURES - NONE

A. Provide coordinates of the area that the protected zone was penetrated more than 0.25 inches by a vehicle component other than one that is normally in contact with the windshield. None
	X
	Y

	
	

	
	

	
	

	
	


B. 
Provide coordinates of the area beneath the protected zone that the inner surface of the windshield was penetrated by a vehicle component. None

	X
	Y

	
	

	
	

	
	

	
	


DATA SHEET NO. 12
BULLET VEHICLE MEASUREMENTS

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


Note:  The body shifted forward on the frame during impact.
	No.
	Measurement Description
	Units
	Pre-Test
	Post-Test
	Difference

	X1
	Total length of vehicle at centerline
	mm
	5024
	4594
	430

	X2
	RSOV to front of engine
	mm
	4502
	4449
	53

	X3
	RSOV to firewall centerline
	mm
	4291
	4283
	8

	X4
	RSOV to leading edge of right door
	mm
	3644
	3653
	-9

	X5
	RSOV to leading edge of left door
	mm
	3645
	3656
	-11

	X6
	RSOV to lower leading edge of right door
	mm
	3608
	3615
	-7

	X7
	RSOV to lower leading edge of left door
	mm
	3602
	3612
	-10

	X8
	RSOV to upper leading edge of right door
	mm
	2622
	2630
	-8

	X9
	RSOV to upper leading edge of left door
	mm
	2620
	2633
	-13

	X10
	RSOV to lower trailing edge of right door
	mm
	2612
	2620
	-8

	X11
	RSOV to lower trailing edge of left door
	mm
	2613
	2616
	-3

	X12
	RSOV to bottom of right 'A' pillar
	mm
	3601
	3611
	-10

	X13
	RSOV to bottom of left 'A' pillar
	mm
	3596
	3603
	-7

	X14
	RSOV to firewall on right side
	mm
	4129
	4121
	8

	X15
	RSOV to firewall on left side
	mm
	4138
	4246
	-108

	X16
	RSOV to steering column
	mm
	3196
	3179
	17

	X17
	Center of steering column to left 'A' pillar
	mm
	401
	357
	44

	X18
	Center of steering column to headlining
	mm
	450
	397
	53

	X19
	RSOV to right side of front bumper
	mm
	4856
	4460
	396

	X20
	RSOV to left side of front bumper
	mm
	4851
	4517
	334

	X21
	Width of engine block
	mm
	408
	408
	0

	Z22
	Right Front Sill to Ground Plane
	mm
	260
	301
	-41

	Z23
	Left Front Sill to Ground Plane
	mm
	251
	335
	-84

	Z24
	Right Rear Sill to Ground Plane
	mm
	290
	330
	-40

	Z25
	Left Rear Sill to Ground Plane
	mm
	290
	370
	-80

	X26
	Firewall to Engine or Transaxle
	mm
	173
	166
	7

	Z27
	Vertical Dim. From Door Sill to Centerline of St. Col.
	mm
	754
	814
	-60

	X28
	Wheelbase of Vehicle (left side)
	mm
	3036
	2948
	88

	Y29
	Width of Vehicle at Maximum Width Point
	mm
	1930
	1930
	0

	X30
	RSOV to Engine Target
	mm
	4301
	4068
	233

	X31
	RSOV to Compartment Target
	mm
	3025
	3039
	-14

	X32
	RSOV to Bumper Target
	mm
	4983
	4481
	502

	X33
	RSOV to Frame Crossmember
	mm
	3817
	3670
	147


DATA SHEET NO. 12... (continued)

BULLET VEHICLE MEASUREMENTS

[image: image24.wmf] 

LEFT SIDE VIEW

 

STEERING COLUMN ASSEMBLY

 

1

 

2

 

3

 


	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


[image: image25.wmf] 

A

 

B

 

C

 

D

 

5

 

4

 

3

 

2

 

1

 

[image: image26.wmf] 

A

 

B

 

C

 

D

 

5

 

4

 

3

 

2

 

1

 



DATA SHEET NO. 12... (continued)

BULLET VEHICLE MEASUREMENTS

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


TOP OF BUMPER

	No.
	Measurement Description
	Units
	Pre-Test
	Post-Test
	Difference

	C1
	Crush Zone 1 on left side
	mm
	249
	583
	334

	C2
	Crush Zone 2 on left side
	mm
	132
	663
	531

	C3
	Crush Zone 3 on left side
	mm
	85
	595
	510

	C4
	Crush Zone 4 on right side
	mm
	85
	574
	489

	C5
	Crush Zone 5 on right side
	mm
	130
	595
	465

	C6
	Crush Zone 6 on right side
	mm
	247
	640
	393


BOTTOM OF BUMPER

	No.
	Measurement Description
	Units
	Pre-Test
	Post-Test
	Difference

	C1
	Crush Zone 1 on left side
	mm
	270
	575
	305

	C2
	Crush Zone 2 on left side
	mm
	160
	620
	460

	C3
	Crush Zone 3 on left side
	mm
	112
	590
	478

	C4
	Crush Zone 4 on right side
	mm
	112
	572
	460

	C5
	Crush Zone 5 on right side
	mm
	160
	610
	450

	C6
	Crush Zone 6 on right side
	mm
	272
	654
	382


CENTER GRILL

	No.
	Measurement Description
	Units
	Pre-Test
	Post-Test
	Difference

	C1
	Crush Zone 1 on left side
	mm
	177
	573
	396

	C2
	Crush Zone 2 on left side
	mm
	161
	616
	455

	C3
	Crush Zone 3 on left side
	mm
	149
	628
	479

	C4
	Crush Zone 4 on right side
	mm
	150
	633
	483

	C5
	Crush Zone 5 on right side
	mm
	162
	680
	518

	C6
	Crush Zone 6 on right side
	mm
	180
	680
	500


FRONT HOOD

	No.
	Measurement Description
	Units
	Pre-Test
	Post-Test
	Difference

	C1
	Crush Zone 1 on left side
	mm
	368
	583
	215

	C2
	Crush Zone 2 on left side
	mm
	232
	540
	308

	C3
	Crush Zone 3 on left side
	mm
	196
	583
	387

	C4
	Crush Zone 4 on right side
	mm
	197
	608
	411

	C5
	Crush Zone 5 on right side
	mm
	234
	621
	387

	C6
	Crush Zone 6 on right side
	mm
	373
	673
	300


	No.
	Measurement Description
	Units
	Pre-Test
	Post-Test
	Difference

	L
	Length of Region
	mm
	1525
	1168
	-357




Note:  Crush is positive.  Measurements are taken from a reference plane in front of the vehicle.  The body shifted forward on the frame.

DATA SHEET NO. 12... (continued)

BULLET VEHICLE MEASUREMENTS

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


BULLET VEHICLE STRUCTURAL MEASUREMENT

	
	Elements
	Pre-Test (mm)

	1
	Total Length
	5024

	2
	Total Width
	1930

	3
	Bumper Top Height
	500

	4
	Bumper Bottom Height
	390

	5
	Longitudinal Member Top Height
	570

	6
	Distance between Longitudinal Members
	1055

	7
	Longitudinal Member Width
	80

	8
	Engine Top Height
	882

	9
	Engine Bottom Height
	170

	10
	Engine and gearbox width
	408

	11
	Front bumper-engine distance
	300

	12
	Front shock absorber fixing height
	955

	13
	Bonnet leading edge height
	740

	14
	Front shock absorber fixing width
	1190

	15
	Front bumper – front axle distance
	917

	16
	Front axle – a pillar distance
	485

	17
	A-pillar – B-pillar distance
	1010

	18
	B-Pillar – rear axle distance
	1530

	19
	B-pillar – C-pillar distance
	860

	20
	Roof sill bottom height
	1558

	21
	Roof sill top height
	1616

	22
	Floor sill bottom height
	268

	23
	Floor sill top height
	368


DATA SHEET NO. 13
BULLET VEHICLE PHOTOGRAPHIC REFERENCE TARGET LOCATIONS
	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


	Item
	[image: image27.wmf] 

A

 

D

 

E

 

B

 

F

 

C

 

Lower Edge of Protected Zone

 

FRONT VIEW OF WINDSHIELD

 

Zero Point

 

Y

 

X

 

Value

	A
	392

	B
	965

	C
	2264

	D
	611

	E
	2149

	F
	

	G
	1613

	H
	1558

	I
	1448

	J
	1020

	K
	1024

	L
	1532

	M
	1555

	N
	1529

	O
	1026

	P
	1018

	Q
	1451


DATA SHEET NO. 14
BULLET VEHICLE INTRUSION MEASUREMENTS

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


DOOR OPENING WIDTH

	Item
	Description
	Units
	Pre-Test
	Post-Test
	Difference

	A
	Left Side Upper
	mm
	1000
	995
	5

	B
	Left Side Lower
	mm
	980
	978
	2

	D
	Right Side Upper
	mm
	1000
	995
	5

	E
	Right Side Lower
	mm
	980
	976
	4


WHEELBASE MEASUREMENTS

	Item
	Description
	Units
	Pre-Test
	Post-Test
	Difference

	C
	Left Side Wheelbase
	mm
	3036
	2948
	88

	F
	Right Side Wheelbase
	mm
	3036
	2975
	61



[image: image4.wmf] 

C

 

B

 

A

 

 

F

 

E

 

D

 


DATA SHEET NO. 14... (Continued)

BULLET VEHICLE INTRUSION MEASUREMENTS

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


DRIVER COMPARTMENT INTRUSION

	Item
	Description
	Units
	Pre-Test
	Post-Test
	Difference

	AB
	Door Opening (Inside window jam)
	mm
	947
	942
	5

	CX
	Left Knee Bolster to X
	mm
	366
	369
	-3

	DX
	Right Knee Bolster to X
	mm
	370
	392
	-22

	EX
	Brake Pedal to X
	mm
	520
	438
	82

	FX
	Foot Rest to X
	mm
	582
	557
	25

	GX
	Center of Steering Column Wheel Hub to X
	mm
	52
	80
	-28


X = Front of Seat Track (stationary)

[image: image5.wmf] 

 A

 

 B

 

 X

 

G

 

 FX

 

 AB

 

 CX

 

 DX

 

 EX

 

GX

 


DRIVER COMPARTMENT  

DATA SHEET NO. 14... (Continued)
BULLET VEHICLE INTRUSION MEASUREMENTS

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


[image: image28.wmf]Measurement reference point for X and Z axis is the forward outboard seat mounting bolt.

Columns A through D are evenly spaced.

Rows 1 and 2 are on the toe kick portion of the floor pan. Rows 3, 4, and 5 are located on the most level portion of the floor pan.

Row 3 will be at the intersection of the toe kick and the level sections of the floor pan.

DRIVER FLOOR PAN X-AXIS

	
	Pre-Test
	Post-Test
	Difference

	
	A
	B
	C
	D
	A
	B
	C
	D
	A
	B
	C
	D

	1
	
	654
	654
	647
	
	547
	528
	512
	
	107
	126
	135

	2
	
	603
	603
	601
	
	497
	492
	464
	
	106
	111
	137

	3
	545
	545
	545
	545
	468
	467
	458
	454
	77
	78
	87
	91

	4
	375
	375
	375
	375
	354
	354
	351
	336
	21
	21
	24
	39

	5
	205
	205
	205
	205
	186
	185
	185
	186
	19
	20
	20
	19


DRIVER FLOOR PAN Z-AXIS

	
	Pre-Test
	Post-Test
	Difference

	
	A
	B
	C
	D
	A
	B
	C
	D
	A
	B
	C
	D

	1
	
	-2
	-8
	-15
	
	63
	75
	86
	
	-65
	-83
	-101

	2
	
	-62
	-70
	-62
	
	0
	2
	25
	
	-62
	-72
	-87

	3
	-117
	-114
	-125
	-123
	-90
	-84
	-71
	-43
	-27
	-30
	-54
	-80

	4
	-116
	-107
	-126
	-112
	-122
	-111
	-118
	-119
	6
	4
	-8
	7

	5
	-100
	-102
	-108
	-112
	-107
	-112
	-117
	-114
	7
	10
	9
	2


DATA SHEET NO. 14... (Continued)

BULLET VEHICLE INTRUSION MEASUREMENTS

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


[image: image29.png]Spuguits Mo




Measurement reference point for X and Z axis is the forward outboard seat mounting bolt.

Columns A through D are evenly spaced.

Rows 1 and 2 are on the toe kick portion of the floor pan. Rows 3, 4, and 5 are located on the most level portion of the floor pan.

Row 3 will be at the intersection of the toe kick and the level sections of the floor pan.

PASSENGER FLOOR PAN X-AXIS

	
	Pre-Test
	Post-Test
	Difference

	
	A
	B
	C
	D
	A
	B
	C
	D
	A
	B
	C
	D

	1
	
	661
	658
	
	
	523
	520
	
	
	138
	138
	

	2
	
	605
	605
	
	
	480
	501
	
	
	125
	104
	

	3
	
	542
	545
	
	
	485
	486
	
	
	57
	59
	

	4
	372
	372
	375
	375
	356
	360
	368
	368
	16
	12
	7
	7

	5
	205
	205
	205
	205
	201
	201
	203
	203
	4
	4
	2
	2


PASSENGER FLOOR PAN Z-AXIS

	
	Pre-Test
	Post-Test
	Difference

	
	A
	B
	C
	D
	A
	B
	C
	D
	A
	B
	C
	D

	1
	
	0
	0
	
	
	90
	104
	
	
	-90
	-104
	

	2
	
	-54
	-56
	
	
	25
	27
	
	
	-79
	-83
	

	3
	
	-104
	-104
	
	
	-48
	-52
	
	
	-56
	-52
	

	4
	-104
	-104
	-104
	-118
	-103
	-95
	-92
	-113
	-1
	-9
	-12
	-5

	5
	-100
	-97
	-97
	-109
	-111
	-105
	-99
	-114
	11
	8
	2
	5


DATA SHEET NO. 14... (Continued)

BULLET VEHICLE INTRUSION MEASUREMENTS

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


[image: image30.wmf] 

LEFT SIDE VIEW

 

STEERING COLUMN ASSEMBLY

 

1

 

2

 

3

 


UNDERBODY FLOORBOARD DEFORMATION

	Measurement
	Pre-Test
	Post-Test
	Difference

	A
	280
	282
	-2

	B
	234
	232
	2

	C
	239
	239
	0

	D
	265
	265
	0

	E
	280
	280
	0

	F
	230
	231
	-1

	G
	232
	232
	0

	H
	270
	269
	1


DATA SHEET NO. 15
BULLET VEHICLE IIHS MEASUREMENTS

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


	Location
	
	Difference

	A Pillar
	X
	-2

	
	Y
	13

	
	Z
	4

	B Pillar
	X
	-2

	
	Y
	4

	
	Z
	3

	Brake Pedal
	X
	-83

	
	Y
	34

	
	Z
	66

	Footrest
	X
	-29

	
	Y
	17

	
	Z
	26

	Knee Bolster Left
	X
	-1

	
	Y
	9

	
	Z
	21

	Knee Bolster Right
	X
	4

	
	Y
	8

	
	Z
	16

	Toepan Left
	X
	-51

	
	Y
	20

	
	Z
	19

	Toepan Center
	X
	-103

	
	Y
	9

	
	Z
	48

	Toepan Right
	X
	-108

	
	Y
	12

	
	Z
	25

	Outboard Front Seat Bolt
	X
	1

	
	Y
	9

	
	Z
	-1

	Inboard Front Seat Bolt
	X
	2

	
	Y
	6

	
	Z
	-17

	Inboard Aft Seat Bolt
	X
	2

	
	Y
	8

	
	Z
	-8

	Outboard Aft Seat Bolt
	X
	1

	
	Y
	6

	
	Z
	0


DATA SHEET NO. 15… (Continued)

BULLET VEHICLE IIHS MEASUREMENTS

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


	Location
	
	Difference

	Bumper 1 (Extreme LH)
	X
	-326

	
	Y
	-70

	
	Z
	-170

	Bumper 2
	X
	-457

	
	Y
	-76

	
	Z
	-205

	Bumper 3
	X
	-535

	
	Y
	-84

	
	Z
	-221

	Bumper 4
	X
	-519

	
	Y
	-76

	
	Z
	-193

	Bumper 5
	X
	-499

	
	Y
	-70

	
	Z
	-167

	Bumper 6 (Mid)
	X
	-499

	
	Y
	-67

	
	Z
	-157

	Bumper 7
	X
	-479

	
	Y
	-69

	
	Z
	-149

	Bumper 8
	X
	-482

	
	Y
	-65

	
	Z
	-145

	Bumper 9
	X
	-480

	
	Y
	-60

	
	Z
	-146

	Bumper 10
	X
	-488

	
	Y
	-61

	
	Z
	-143

	Bumper 11 (Extreme RH)
	X
	-467

	
	Y
	-34

	
	Z
	-159


X – Positive forward, Y – Positive to the left, Z – Positive up

SECTION 3

TARGET VEHICLE INFORMATION / DATA SHEETS

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


CONVERSION FACTORS USED IN THIS REPORT*

	Quantity
	Typical Application
	English Units
	Metric Unit
	Multiply By

	Mass
	Vehicle Weight
	lb
	kg
	0.4536

	Linear Velocity
	Impact Velocity
	mile/h
	km/h
	1.609

	Length or Distance
	Measurements 
	in
	mm
	25.4

	Volume
	Fuel Systems
	gal
	liter
	3.785

	Volume
	Small Fluids
	oz
	mL
	29.573

	Pressure
	Tire Pressure
	lbf/in2
	kPa
	7.0

	Volume
	Liquid
	gal
	liter
	3.785

	Temperature
	General Use
	°F
	°C
	=(tf -32)/1.8

	Force
	Dynamic Forces
	lbf
	N
	4.448

	Moment
	Torque
	lbf/ft
	Nm
	1.355


*Based on the Recommended Practice in SAE J916, May 85

DATA SHEET NO. 16
TARGET VEHICLE TEST SUMMARY

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


DOOR OPENING AND SEAT TRACK INFORMATION

	Description
	Driver
	Passenger

	Locked/Unlocked Doors
	Door was unlocked
	Door was unlocked

	Front Door Opening
	Door remained closed and latched; Door opened without tools
	Door remained closed and latched; Door opened without tools

	Rear Door Opening
	Door remained closed and latched; Door opened without tools
	Door remained closed and latched; Door opened without tools

	Seat Track Shift
	0 mm
	0 mm


TEST DUMMY INFORMATION

	Description
	Driver
	Passenger

	Dummy Type / Serial No.
	HIII 50th/ 90
	HIII 5th/ 547

	Head Contact
	Front Airbag, Side Airbag, Curtain Airbag, Head Rest
	Front Airbag, Side Airbag, Curtain Airbag, Head Rest

	Chest Contact
	Airbag
	Airbag

	Abdomen Contact
	Airbag
	Airbag

	Left Knee Contact
	Knee Bolster, Steering Column
	Glovebox

	Right Knee Contact
	Knee Bolster, Steering Column
	Glovebox


INSTRUMENTATION
	Driver ATD Sensors
	72

	Passenger ATD Sensors
	37

	Belt Assessment Sensors
	4

	Vehicle Structure Accelerometers
	13

	Total
	126


DATA SHEET NO. 17
TARGET VEHICLE DATA

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


VEHICLE INFORMATION 

                                    VEHICLE OPTIONS

	Make
	Honda
	
	Driver Airbag
	Yes

	Model
	Accord EX
	
	Passenger Airbag
	Yes

	Body Style
	Sedan
	
	Power Windows
	Yes

	NHTSA No.
	R45344
	
	Power Steering
	Yes

	VIN
	1HGCM56754A108943
	
	Power Door Locks
	Yes

	Color
	Silver
	
	Tilt Wheel
	Yes

	Odometer Reading (mile)
	42
	
	Air Conditioning
	Yes

	Transmission
	Automatic
	
	Power Brakes
	Yes

	Final Drive
	Front
	
	Disc Brakes, Front
	Yes

	Number of Cylinders
	4
	
	Disc Brakes, Rear
	Yes

	Engine Displacement (L)
	2.4
	
	Anti-lock Brakes
	Yes

	Engine Placement
	Lateral
	
	AM/FM/CD
	Yes


DATA FROM CERTIFICATION LABEL

	Manufactured By
	Honda of America Mfg., Inc.

	Date of Manufacture
	04/04


1

	GVWR (kg)
	1871

	GAWR Front (kg)
	1009

	GAWR Rear (kg)
	900


DATA FROM TIRE PLACARD

	Measured Parameter
	Front
	Rear

	Cold Pressure (kPa)
	220
	210

	Recommended Tire Size
	P205/60R16
	P205/60R16

	Tire Size on Vehicle
	P205/60R16
	P205/60R16

	Tire Manufacturer
	Michelin
	Michelin


	Measured Parameter
	Front
	Rear
	Third
	Total

	Type of Seats
	Bucket
	Bench
	
	

	Number Of Occupants
	2
	3
	
	5

	Vehicle Capacity Weight (kg)
	
	
	
	385.6

	Rated Cargo Weight (kg)
	
	
	
	45.4


DATA SHEET NO. 18
TARGET VEHICLE TIRE INFORMATION

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


	Vehicle Year
	2004
	Vehicle Make
	Honda

	VIN
	1HGCM56754A108943
	Vehicle Model
	Accord EX


[image: image31.wmf] 

A

 

D

 

E

 

B

 

F

 

C

 

Lower Edge of Protected Zone

 

FRONT VIEW OF WINDSHIELD

 

Zero Point

 

Y

 

X

 


	
	Front
	Rear

	Tire Manufacturer
	Michelin
	Michelin

	Tire Name
	Energy MXV4
	Energy MXV4

	Tire Type
	P
	P

	Tire Width (mm)
	205
	205

	Ratio of Height to Width (aspect ratio)
	60
	60

	Radial
	R
	R

	Wheel Diameter
	16
	16

	Load Index & Speed Symbol
	91V
	91V

	Treadwear
	440
	440

	Traction Grade
	A
	A

	Temperature Grade
	A
	A


DATA SHEET NO. 19
TARGET VEHICLE SETUP INFORMATION

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX


VEHICLE WEIGHTS

	
	Units
	As Delivered (UVW) (Axle)
	As Tested (ATW) (Axle)

	
	
	Front
	Rear 
	Total
	Front 
	Rear
	Total

	Left
	kg
	445.9
	291.2
	
	484.0
	345.2
	

	Right
	kg
	446.3
	279.0
	
	475.4
	322.1
	

	Ratio
	%
	61.0
	39.0
	
	59.0
	41.0
	

	Totals
	kg
	892.2
	570.2
	1464.4
	959.4
	667.3
	1626.7


TARGET TEST WEIGHT CALCULATION

	Measured Parameter
	Units
	Value

	Total Delivered Weight (UVW)
	kg
	1464.4

	Weight of 1 P572E & 1 P572O ATDs
	kg
	127.0

	Rated Cargo/Luggage Weight (RCLW)
	kg
	45.4

	Calculated Vehicle Target Weight (TVTW)
	kg
	1636.8


VEHICLE ATTITUDES AND CG

	
	Units
	LF
	RF
	LR
	RR
	CG(aft of front axle)

	As Delivered
	mm
	704
	709
	687
	694
	1067

	As Tested
	mm
	691
	698
	672
	679
	1124

	Post Test
	mm
	765
	651
	798
	702
	


Vehicle Wheelbase (mm): 






   
2740
Weight of Ballast secured in cargo area (kg): 





   4.5

Vehicle Components Removed: Rear Seat, Trunk Interior, Spare and Jack.
Ballast weight does not include instrumentation, data acquisition system and cameras.
DATA SHEET NO. 19… (Continued)

TARGET VEHICLE SETUP INFORMATION

SEAT BACK POSITION

Driver seat back angle:  
   5th notch, 1st as 0 (14.3 deg. on headrest post)

Passenger seat back angle: 
   6.6 deg. on headrest post
  
SEAT FORE/AFT POSITIONS

The driver and passenger seat were manually operated.

Driver seat fore/aft total travel: 
   25 positions 

Passenger seat fore/aft total travel: 
   25 positions

Driver seat fore/aft position: 

    13th position of 25
   

Passenger seat fore/aft position: 
    Full forward 
 

SEAT BELT UPPER ANCHORAGE

There were four adjustment positions.  The driver side was placed in the second position with the top as one and the passenger side was placed in the lowest position.
FUEL TANK CAPACITY DATA

The fuel tank was filled with 60.6 liters of water for ballast.
STEERING COLUMN ADJUSTMENT

The steering column was placed in the mid position of travel.
DATA SHEET NO. 20
DUMMY POSITIONING IN TARGET VEHICLE

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


DUMMY MEASUREMENTS FOR FRONT SEAT OCCUPANTS


[image: image6.wmf] 

PASS

 

DRVR

 

Vertical Longitudinal Planes

 

Vertical

 

Transverse

 

Plane

 

HS

 

KK

 

AD

 

HD

 

HR

 

SHY

 

STRIKER

 

HZ

 

CD

 

KD

 

CS

 

Seat Back Angle Line

 

SH

 

NR

 

SK

 

AD

 

Arm to Door

 

HD

 

H

-

Point to Door

 

HR

 

Head to Side Header

 

HS

 

Head to Side Window

 

KK

 

Knee to Knee

 

SHY

 

St

riker to H

-

Point

 

          (Y Axis)

 

CD

 

Chest to Dash

 

CS

 

Chest to Steering Wheel Hub

 

HH

 

Head to Header

 

HW

 

Head to Windshield

 

HZ

 

Head to Roof

 

KDA

 

Knee to Dash Angle

 

KDL

 

Left Knee to Dash

 

KDR

 

Right Knee to Dash

 

NA

 

Nose to Rim Angle

 

NR

 

Nose to Rim

 

PA

 

Pelvic Angle

 

RA

 

Rim to Abdomen

 

SA

 

Seat Back Angle

 

SCA

 

Steering Column Angle

 

SH

 

Striker to H

-

Point

 

SK

 

Striker t

o Knee

 

ST

 

Striker to Head

 

SWA

 

Steering Wheel Angle

 

TA

 

Tibial Angle

 

WA

 

Windshield Angle

 

 SWA

°

 

  HH

 

 WA

°

 

 HW

 

 NA

°

 

 SCA

°

 

 KDA

°

 

 TA

°

 

 SA

°

 

ST

 

 PA

°

 


DATA SHEET NO. 20... (Continued)

DUMMY POSITIONING IN TARGET VEHICLE

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


TEST DUMMY POSITION MEASUREMENTS

	Code
	Measurement Description
	Driver
	Passenger

	
	
	Length (mm)
	Angle (°)
	Length (mm)
	Angle (°)

	WA
	Windshield Angle
	
	27.0
	
	

	SWA
	Steering Wheel Angle
	
	67.3
	
	

	SCA
	Steering Column Angle
	
	22.6
	
	

	SA
	Seat Back Angle (on headrest post)
	
	14.3
	
	6.6

	HZ
	Head to Roof (Z)
	174
	90.0
	184
	90.0

	HH
	Head to Header
	368
	18.0
	262
	50.2

	HW
	Head to Windshield
	341
	0.0
	595
	0.0

	HR
	Head to Side Header (Y)
	203
	
	240
	

	NR
	Nose to Rim
	400
	8.0
	
	

	CD
	Chest to Dash
	532
	
	401
	

	CS
	Chest to Steering Hub
	326
	9.2
	
	

	RA
	Rim to Abdomen
	201
	0.0
	
	

	KDL
	Left Knee to Dash
	195
	30.7
	101
	

	KDR
	Right Knee to Dash
	169
	
	115
	28.8

	PA
	Pelvic Angle
	
	23.0
	
	26.6

	TA
	Tibia Angle
	
	43.3
	
	49.4

	KK
	Knee to Knee (Y)
	320
	
	214
	

	SK
	Striker to Knee
	588
	87.1
	704
	86.3

	ST
	Striker to Head
	510
	8.7
	514
	28.1

	SH
	Striker to H-Point
	252
	124.4
	391
	105.2

	SHY
	Striker to H-Point (Y)
	262
	
	292
	

	HS
	Head to Side Window
	312
	
	346
	

	HD
	H-Point to Door (Y)
	147
	
	182
	

	AD
	Arm to Door (Y)
	141
	
	201
	

	AA
	Ankle to Ankle
	310
	
	164
	


DATA SHEET NO. 21
TARGET VEHICLE SEAT BELT POSITIONING DATA

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344



[image: image7.wmf] 

 

Dummy’s Centerline

 

TBI

 

'D' Ring

 

Shoulder Belt 

Portion

 

Lap Belt 

Portion

 

1/8" Thick 

Aluminum 

Plate

 

Inboard 

Anchorage

 

Flo

orpan

 

Buckle 

Assembly

 

PBL

 

PBU

 

Outboard 

Anchor

 

Locking 

Retractor

 


SEAT BELT POSITIONING MEASUREMENTS

	Measurement Description
	Units
	Driver
	Passenger

	PBU - Top surface of reference to belt upper edge
	mm
	330
	267

	PBL - To surface of reference to belt lower edge
	mm
	245
	182


DATA SHEET NO. 22
TARGET VEHICLE ACCELEROMETER LOCATION AND DATA SUMMARY

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


ACCELEROMETER PEAK DATA (CFC 60)
	 Accelerometer Location 
	Peak Values

	
	Units
	Max
	Time
	Min
	Time

	 Left Rear Seat Crossmember X
	G's
	1.6
	132
	-33.1
	53

	 Right Rear Seat Crossmember X
	G's
	*
	*
	*
	*

	 Front Brake Caliper Left X
	G's
	13.7
	71
	-64.3
	53

	 Front Brake Caliper Right X
	G's
	37.9
	69
	-76.0
	32

	 Dash Panel Center X
	G's
	19.1
	63
	-70.5
	49

	 Top Engine X 
	G’s
	32.6
	70
	-134.0
	39

	 Bottom Engine X
	G’s
	**
	**
	**
	**

	 Toe Pan (Accelerator) X
	G’s
	29.5
	65
	-62.9
	50

	 Toe Pan (Accelerator) Z
	G’s
	33.7
	71
	-20.8
	49

	 Toe Pan (Footrest) X
	G’s
	11.0
	20
	-54.7
	38

	 Toe Pan (Footrest) Z
	G’s
	15.0
	73
	-7.8
	40

	 Rear Deck Z
	G’s
	19.1
	91
	-24.5
	57

	 Rear Compartment Center X 
	G’s
	2.9
	118
	-33.5
	51


* No Valid Data collected after 30 msec                                                                                                                            
** No Valid Data collected 

DATA SHEET NO. 22… (Continued)
TARGET VEHICLE ACCELEROMETER LOCATION AND DATA SUMMARY

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


ACCELELEROMETER LOCATIONS
	 Accelerometer Location 
	Pre Test 

Measurements (mm)
	Post Test Measurements (mm)

	
	X
	Y
	Z
	X
	Y
	Z

	 Left Rear Seat Crossmember 
	1812
	-325
	470
	1812
	-325
	527

	 Right Rear Seat Crossmember
	1816
	325
	470
	1816
	325
	499

	 Front Brake Caliper Left 
	3870
	-695
	312
	3889
	-702
	329

	 Front Brake Caliper Right
	3935
	695
	312
	3895
	709
	309

	 Dash Panel Center 
	3431
	-5
	1024
	3210
	-5
	1010

	 Top Engine 
	3997
	13
	837
	3796
	2
	757

	 Bottom Engine 
	4062
	-20
	225
	3921
	-14
	134

	 Toe Pan (Accelerator) 
	3495
	-345
	395
	3425
	-291
	426

	 Toe Pan (Footrest) 
	3510
	-550
	505
	3447
	-518
	553

	 Rear Deck 
	842
	0
	1082
	842
	0
	1143

	 Rear Compartment Center
	1981
	0
	370
	1981
	0
	390


Reference Points:
X - Rear Surface of Vehicle (+ forward)




Y - Vehicle Centerline
(+ to right)





Z - Ground Plane (+ up)

DATA SHEET NO. 23
TARGET VEHICLE HYBRID III ATD INJURY CRITERIA AND SENSOR DATA

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


HEAD PRIMARY PEAK ACCELERATIONS

	Location
	Axis
	Units
	Driver
	Passenger

	
	
	
	Max
	Time
	Min
	Time
	Max
	Time
	Min
	Time

	Head CG
	X
	G's
	15.4
	231
	-192.3
	84
	18.8
	191
	-50.1
	67

	Head CG
	Y
	G's
	7.5
	82
	-14.6
	84
	5.9
	39
	-12.8
	38

	Head CG
	Z
	G's
	45.8
	84
	-6.2
	18.9
	22.2
	50
	-7.8
	34

	Head CG Resultant
	N/A
	G's
	197.5
	84
	
	
	50.5
	67
	
	


HEAD INJURY CRITERIA (HIC)

	Location
	Driver
	Passenger

	
	HIC
	Avg. G's
	T1
	T2
	HIC
	Avg. G's
	T1
	T2

	HIC 36
	899.9
	144.3
	82.1
	85.7
	358.1
	39.7
	51.5
	87.5

	HIC 15
	899.9
	144.3
	82.1
	85.7
	224.9
	46.8
	61.8
	76.8


HEAD PEAK ACCELERATIONS

	Location
	Axis
	Units
	Driver
	Passenger

	
	
	
	Max
	Time
	Min
	Time
	Max
	Time
	Min
	Time

	Head Front
	X
	G's
	15.9
	231
	-190.4
	84
	17.6
	192
	-49.9
	66

	Head Front
	Y
	G's
	9.0
	90.4
	-26.3
	84
	10.2
	60
	-16.0
	50

	Head Front
	Z
	G's
	93.6
	84
	-28.6
	88
	36.4
	50
	-8.2
	33

	Head Front Res
	N/A
	G's
	213.4
	84
	
	
	50.1
	72
	
	


HEAD PEAK ACCELERATIONS

	Location
	Axis
	Units
	Driver
	Passenger

	
	
	
	Max
	Time
	Min
	Time
	Max
	Time
	Min
	Time

	Head Left
	X
	G's
	18.1
	232
	-198.4
	84
	16.5
	193
	-49.2
	70

	Head Left
	Y
	G's
	4.2
	82
	-23.3
	84
	6.1
	57
	-8.2
	50

	Head Left
	Z
	G's
	44.2
	80
	-2.2
	18
	22.4
	55
	-8.0
	70

	Head Left Res
	N/A
	G's
	203.1
	84
	
	
	49.8
	70
	
	


HEAD PEAK ACCELERATIONS

	Location
	Axis
	Units
	Driver
	Passenger

	
	
	
	Max
	Time
	Min
	Time
	Max
	Time
	Min
	Time

	Head Upper
	X
	G's
	27.6
	233
	-136.6
	87
	27.5
	190
	-56.6
	66

	Head Upper
	Y
	G's
	4.8
	81
	-16.0
	87
	12.1
	71
	-11.0
	57

	Head Upper
	Z
	G's
	52.0
	84
	-3.1
	18
	20.4
	50
	-6.0
	37

	Head Upper Res
	N/A
	G's
	144.5
	87
	
	
	57.0
	66
	
	


DATA SHEET NO. 23... (Continued)

TARGET VEHICLE HYBRID III ATD INJURY CRITERIA AND SENSOR DATA

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


UPPER NECK PEAK FORCES AND MOMENTS

	 Location
	Axis
	Units
	Driver
	Passenger

	
	
	
	Max
	Time
	Min
	Time
	Max
	Time
	Min
	Time

	 Neck Force
	X
	Newtons
	1588
	85
	-401
	77
	601
	64
	-209
	27

	 Neck Force
	Y
	Newtons
	102
	100
	-136
	83
	71
	74
	-172
	88

	 Neck Force
	Z
	Newtons
	1545
	84
	-874
	90
	584
	39
	-419
	225

	 Neck Moment
	X
	N•m
	10.5
	103
	-4.7
	83
	8.3
	89
	-8.8
	116

	 Neck Moment
	Y
	N•m
	28.4
	282
	-87.2
	87
	40.8
	64
	-19.3
	230

	 Neck Moment
	Z
	N•m
	3.3
	98
	-2.5
	154
	4.9
	99
	-4.7
	163


LOWER NECK PEAK FORCES AND MOMENTS

	 Location
	Axis
	Units
	Driver
	Passenger

	
	
	
	Max
	Time
	Min
	Time
	Max
	Time
	Min
	Time

	 Neck Force
	X
	Newtons
	516
	89
	-906
	84
	276
	61
	-207
	105

	 Neck Force
	Y
	Newtons
	205
	92
	-53
	282
	71
	80
	-204
	92

	 Neck Force
	Z
	Newtons
	2103
	84
	-451
	90
	920
	39
	-419
	225

	 Neck Moment
	X
	N•m
	9.0
	95
	-8.4
	85
	9.1
	201
	-26.0
	106

	 Neck Moment
	Y
	N•m
	48.4
	82
	-88.4
	87
	42.9
	138
	-42.6
	39

	 Neck Moment
	Z
	N•m
	12.6
	93
	-3.3
	286
	
	
	
	


CHEST PRIMARY PEAK ACCELERATIONS

	Location
	Axis
	Units
	Driver
	Passenger

	
	
	
	Max
	Time

12

	Min
	Time
	Max
	Time
	Min
	Time

	Chest CG
	X
	G's
	3.9
	290
	-42.8
	87
	4.2
	227
	-51.2
	54

	Chest CG
	Y
	G's
	1.3
	65
	-3.0
	90
	5.7
	40
	-13.5
	84

	Chest CG
	Z
	G's
	7.9
	74
	-18.0
	90
	4.1
	224
	-10.2
	75

	Chest CG Resultant
	N/A
	G's
	42.9
	87
	
	
	51.5
	54
	
	


CHEST PEAK DISPLACEMENTS

	 Location
	Axis
	Units
	Driver
	
	Passenger

	
	
	
	Max
	Time
	Min
	Time
	Max
	Time
	Min
	Time

	 Chest 
	X
	mm
	
	
	-27.3
	87
	
	
	-18.2
	57


CHEST CLIP (3MSEC)

	Location
	Driver
	Passenger

	
	CLIP
	T1
	T2
	CLIP
	T1
	T2

	Chest CG Primary
	42.1
	85.7
	88.7
	48.3
	52.1
	55.1


SEAT BELT SENSOR PEAK VALUES
	 Location
	Axis
	Units
	Driver
	Passenger

	
	
	
	Max
	Time
	Min
	Time
	Max
	Time
	Min
	Time

	 Lap Belt Force
	N/A
	Newtons
	6874
	60
	
	
	2378
	50
	
	

	 Shoulder Belt Force
	N/A
	Newtons
	4884
	64
	
	
	4016
	55
	
	


DATA SHEET NO. 23... (Continued)

TARGET VEHICLE HYBRID III ATD INJURY CRITERIA AND SENSOR DATA

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


FEMUR PEAK FORCES

	 Location
	Axis
	Units
	Driver
	Passenger

	
	
	
	Max
	Time
	Min
	Time
	Max
	Time
	Min
	Time

	 Left Femur
	Z
	Newtons
	942
	53
	-874
	66
	443
	25
	-2884
	54

	 Right Femur
	Z
	Newtons
	1388
	57
	-537
	43
	424
	32
	-2337
	54


UPPER AND LOWER TIBIA PEAK FORCES AND MOMENTS

	 Location
	Axis
	Units
	Driver
	Passenger

	
	
	
	Max
	Time
	Min
	Time
	Max
	Time
	Min
	Time

	Left Lower Moment
	X
	N•m
	77.1
	73
	-23.4
	31
	
	
	
	

	Left Lower Moment
	Y
	N•m
	17.0
	106
	-57.1
	60
	
	
	
	

	Left Lower Force
	X
	Newtons
	129
	105
	-301
	59
	
	
	
	

	Left Lower Force 
	Y
	Newtons
	400
	63
	-99
	156
	
	
	
	

	Left Lower Force

Left Lower Force
	Z
	Newtons
	208
	183
	-2135
	33
	
	
	
	

	Left Upper Moment
	X
	N•m
	*
	*
	*
	*
	
	
	
	

	Left Upper Moment
	Y
	N•m
	53.4
	66
	-44.0
	45
	
	
	
	

	Left Upper Force 
	X
	Newtons
	410
	77
	-79
	36
	
	
	
	

	Left Upper Force
	Z
	Newtons
	33
	182
	-1568
	34
	
	
	
	

	Right Lower Moment
	X
	N•m
	22.5
	42
	-52.4
	89
	
	
	
	

	Right Lower Moment
	Y
	N•m
	19.4
	106
	-44.1
	54
	
	
	
	

	Right Lower Force 
	X
	Newtons
	83
	255
	-621
	76
	
	
	
	

	Right Lower Force 
	Y
	Newtons
	72
	125
	-285
	43
	
	
	
	

	Right Lower Force
	Z
	Newtons
	232
	166
	-3177
	43
	
	
	
	

	Right Upper Moment
	X
	N•m
	38.4
	44
	-31.9
	116
	
	
	
	

	Right Upper Moment
	Y
	N•m
	38.9
	108
	-66.4
	55
	
	
	
	

	Right Upper Force
	X
	Newtons
	361
	55
	-193
	107
	
	
	
	

	Right Upper Force
	Z
	Newtons
	187
	170
	-2480
	43
	
	
	
	


* No valid data collected 
DATA SHEET NO. 23... (Continued)

TARGET VEHICLE HYBRID III ATD INJURY CRITERIA AND SENSOR DATA

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


KNEE SHEAR DISPLACEMENT

	Location
	Axis
	Units
	Driver
	Passenger

	
	
	
	Max
	Time
	Min
	Time
	Max
	Time
	Min
	Time

	Left
	Z
	mm
	
	
	-5.4
	66
	
	
	-0.6
	89

	Right
	Z
	mm
	
	
	-1.4
	57
	
	
	-0.4
	65


TIBIA MID SHAFT ACCELERATIONS

	Location
	Axis
	Units
	Driver
	Passenger

	
	
	
	Max
	Time
	Min
	Time
	Max
	Time
	Min
	Time

	Left  
	X
	G's
	6.3
	282
	-89
	59
	
	
	
	

	Left  
	Y
	G's
	21.5
	30
	-24.5
	71
	
	
	
	

	Right 
	X
	G's
	8.2
	107
	-47.7
	76
	
	
	
	

	Right 
	Y
	G's
	12.0
	88
	-17.5
	43
	
	
	
	


FOOT PEAK ACCELERATIONS

	Location
	Axis
	Units
	Driver
	Passenger

	
	
	
	Max
	Time
	Min
	Time
	Max
	Time
	Min
	Time

	Left Mid Foot 
	X
	G's
	32.6
	44
	-65.2
	64
	
	
	
	

	Left Mid Foot 
	Y
	G's
	25.0
	33
	-77.8
	58
	
	
	
	

	Left Mid Foot 
	Z
	G's
	*
	*
	*
	*
	
	
	
	

	Right Mid Foot
	X
	G's
	14.5
	100
	-51.9
	51
	
	
	
	

	Right Mid Foot 
	Y
	G's
	50.2
	52
	-17.8
	33
	
	
	
	

	Right Mid Foot
	Z
	G's
	24.3
	67
	-59.8
	40
	
	
	
	


* No Valid Data Collected after 55 msec
FOOT ROTATIONS

	Location
	Units
	Driver
	Passenger

	
	
	Max
	Time
	Min
	Time
	Max
	Time
	Min
	Time

	Left Dorsiflexion /

Plantar Flexion 
	Degrees
	21.2
	77
	-9.2
	222
	
	
	
	

	Left Inversion / Eversion
	Degrees
	46.2
	88
	10.6
	235
	
	
	
	

	Left Internal / External 
	Degrees
	17.7
	19
	-2.4
	81
	
	
	
	

	Right Dorsiflexion /

Plantar Flexion 
	Degrees
	24.0
	88
	-19.7
	194
	
	
	
	

	Right Inversion / Eversion
	Degrees
	18.0
	0
	-13.0
	97
	
	
	
	

	Right Internal / External 
	Degrees
	11.6
	131
	2.3
	228
	
	
	
	


DATA SHEET NO. 23... (Continued)

TARGET VEHICLE HYBRID III ATD INJURY CRITERIA AND SENSOR DATA

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


HEAD REDUNDANT PEAK ACCELERATIONS

	Location
	Axis
	Units
	Driver
	Passenger

	
	
	
	Max
	Time
	Min
	Time
	Max
	Time
	Min
	Time

	Head CG
	X
	G's
	14.9
	231
	-192.5
	84
	18.4
	192
	-49.1
	69

	Head CG
	Y
	G's
	6.2
	82
	-16.3
	84
	4.9
	39
	-11.9
	38

	Head CG
	Z
	G's
	48.6
	84
	-6.4
	19
	20.6
	50
	-7.1
	34

	Head CG Resultant
	N/A
	G's
	198.6
	84
	
	
	49.3
	66
	
	


CHEST REDUNDANT PEAK ACCELERATIONS

	Location
	Axis
	Units
	Driver
	Passenger

	
	
	
	Max
	Time

12

	Min
	Time
	Max
	Time
	Min
	Time

	Chest CG
	X
	G's
	4.1
	290
	-42.1
	86
	4.2
	228
	-50.5
	54

	Chest CG
	Y
	G's
	2.6
	65
	-2.8
	95
	4.8
	46
	-11.1
	84

	Chest CG
	Z
	G's
	7.4
	73
	-19.3
	90
	3.8
	224
	-10.0
	75

	Chest CG Resultant
	N/A
	G's
	42.2
	86
	
	
	50.7
	54
	
	


REDUNDANT HEAD INJURY CRITERIA (HIC)

	Location
	Driver
	Passenger

	
	HIC
	Avg. G's 
	T1
	T2
HIC
	HIC
	Avg. G's
	T1
	T2

	HIC 36
	925.9
	142.8
	82.0
	85.8
	345.5
	39.2
	51.3
	87.3

	HIC 15
	925.9
	142.8
	82.0
	85.8
	216.1
	46.1
	61.4
	76.4


REDUNDANT CHEST CLIP (3MSEC)

	Location
	Driver
	Passenger

	
	CLIP
	T1
	T2
	CLIP
	T1
	T2

	Chest CG Redundant
	41.7
	85.8
	88.8
	47.6
	52.1
	55.1


NECK INJURY CRITERIA

	Location
	Driver
	Passenger

	Nte (Tension – Extension)
	0.9
	0.1

	Ntf  (Tension – Flexion)
	0.3
	0.3

	Nce (Compression – Extension)
	0.8
	0.3

	Ncf  (Compression – Flexion)
	0.2
	0.2


DATA SHEET NO. 24
TARGET VEHICLE SEAT BELT ASSESSMENT AND POST TEST AIRBAG DATA

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


BELT LENGTH DATA

	Measurement Description
	Units
	Driver
	Passenger

	Retractor reel to "D" ring
	mm
	140
	80

	Shoulder belt length as measured on ATD
	mm
	855
	937

	Lap belt length as measured on ATD
	mm
	765
	620

	Remainder of belt on reel
	mm
	1462
	1587

	Total belt length for continuous webbing systems
	mm
	3222
	3224


BELT SPOOL-OUT DATA

	Measurement Description
	Units
	Driver
	Passenger

	Pretensioner
	mm
	
	

	At the Shoulder
	mm
	217
	12

	At Retractor
	mm
	192
	35


* No Valid Data
POST TEST AIRBAG DATA
	Number of Vent Holes

	Driver
	2
	Passenger
	2

	Size of Vent Holes (Note whether round or square)

	Driver
	round
	Passenger
	round

	Total Vent Area

	Driver
	21.2 cm2
	Passenger
	30.2 cm2

	Deflated Airbag Length and Width Dimensions or, if Round, Diameter

	Driver **
	Length: 
	Width:
	Diameter: 562mm

	Passenger
	Length: 830 mm
	Width: 720 mm
	Diameter: 

	Is the Airbag Tethered?

	Driver
	Yes
	
	Length:  266mm

	Passenger
	
	No
	Length:


** The driver airbag had a big tear during impact which was 394mm long.
DATA SHEET NO. 25
TARGET VEHICLE FMVSS 212 DATA

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


Windshield Mounting Details:

Windshield glass is secured to the vehicle frame with a rubber trim and glue.

The standard requires that the post-test retention measurement be a minimum of 75 percent of the pretest total periphery measurement for vehicles not equipped with occupant passive restraints and 50 percent for each side of the windshield for vehicles, which are equipped with occupant passive restraints.

WINDSHIELD PERIPHERY MEASUREMENTS

	Measurement
	Pre-Test (mm)
	Post-Test (mm)
	% of Retention

	Left Side
	2133
	2133
	100

	Right Side
	2138
	2138
	100

	Total
	4271
	4271
	100


[image: image32.wmf] 

I

 

J

 

K

 

C

 

F

 

D

 

E

 

A

 

B

 

B

 

H

 

M

 

G

 

L

 

N

 

O

 

P

 

Q

 

[image: image33.wmf] 

A

 

B

 

C

 

D

 

5

 

4

 

3

 

2

 

1

 


	Item
	Units
	Segment Length
	Molding Width

	A
	mm
	1148
	17

	B
	mm
	1486
	11

	C
	mm
	825
	22


DATA SHEET NO. 26
	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


TARGET VEHICLE FMVSS 219 (Partial) DATA

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX


	Item
	Units 
	Value

	A
	mm
	1148

	B
	mm
	443

	C
	mm
	1486

	D
	mm
	825

	E
	mm
	541

	F
	mm
	500


[image: image34.png]Treadwear, traction
and temperature grades

Trepy
compagition
used

USS. DOT satety

standard code Passenger

car tire
Width of tire
meters

Ratio of height to
‘width (aepect ratio)

Load index &
speed symbol
Diameter of

wheel ininches Rainl





AREA OF PROTECTED ZONE FAILURES - NONE

B. Provide coordinates of the area that the protected zone was penetrated more than 0.25 inches by a vehicle component other than one that is normally in contact with the windshield. None
	X
	Y

	
	

	
	

	
	

	
	


B. 
Provide coordinates of the area beneath the protected zone that the inner surface of the windshield was penetrated by a vehicle component. None

	X
	Y

	
	

	
	

	
	

	
	


DATA SHEET NO. 27
TARGET VEHICLE MEASUREMENTS

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


	No.
	Measurement Description
	Units
	Pre-Test
	Post-Test
	Difference

	X1
	Total length of vehicle at centerline
	mm
	4800
	4289
	511

	X2
	RSOV to front of engine
	mm
	3969
	3771
	198

	X3
	RSOV to firewall centerline
	mm
	3714
	3725
	-11

	X4
	RSOV to leading edge of right door
	mm
	3325
	3332
	-7

	X5
	RSOV to leading edge of left door
	mm
	3324
	3344
	-20

	X6
	RSOV to lower leading edge of right door
	mm
	3320
	3315
	5

	X7
	RSOV to lower leading edge of left door
	mm
	3320
	3319
	1

	X8
	RSOV to upper leading edge of right door
	mm
	2183
	2196
	-13

	X9
	RSOV to upper leading edge of left door
	mm
	2180
	2208
	-28

	X10
	RSOV to lower trailing edge of right door
	mm
	2190
	2186
	4

	X11
	RSOV to lower trailing edge of left door
	mm
	2189
	2194
	-5

	X12
	RSOV to bottom of right 'A' pillar
	mm
	3290
	3299
	-9

	X13
	RSOV to bottom of left 'A' pillar
	mm
	3287
	3306
	-19

	X14
	RSOV to firewall on right side
	mm
	3668
	3491
	177

	X15
	RSOV to firewall on left side
	mm
	3668
	3477
	191

	X16
	RSOV to steering column
	mm
	2810
	2882
	-72

	X17
	Center of steering column to left 'A' pillar
	mm
	380
	376
	4

	X18
	Center of steering column to headlining
	mm
	400
	398
	2

	X19
	RSOV to right side of front bumper
	mm
	4577
	4316
	261

	X20
	RSOV to left side of front bumper
	mm
	4574
	4441
	133

	X21
	Width of engine block
	mm
	450
	450
	0

	Z22
	Right Front Sill to Ground Plane
	mm
	207
	159
	48

	Z23
	Left Front Sill to Ground Plane
	mm
	200
	232
	-32

	Z24
	Right Rear Sill to Ground Plane
	mm
	220
	208
	12

	Z25
	Left Rear Sill to Ground Plane
	mm
	212
	293
	-81

	X26
	Firewall to Engine or Transaxle
	mm
	255
	46
	209

	Z27
	Vertical Dim. From Door Sill to Centerline of St. Col.
	mm
	650
	627
	23

	X28
	Wheelbase of Vehicle (left side)
	mm
	2740
	2701
	39

	Y29
	Width of Vehicle at Maximum Width Point
	mm
	1822
	1828
	-6

	X30
	RSOV to Engine Target
	mm
	4093
	2909
	184

	X31
	RSOV to Compartment Target
	mm
	2728
	2718
	10

	X32
	RSOV to Bumper Target
	mm
	4620
	4348
	272

	X33
	RSOV to Frame Cross member
	mm
	3976
	3884
	92


DATA SHEET NO. 27... (Continued)

TARGET VEHICLE MEASUREMENTS

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344



[image: image8.wmf]
[image: image9.jpg]z22-23 Z24-25




[image: image10.jpg]



DATA SHEET NO. 27... (Continued)

TARGET VEHICLE MEASUREMENTS

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


TOP OF BUMPER
	No.
	Measurement Description
	Units
	Pre-Test
	Post-Test
	Difference

	C1
	Crush Zone 1 on left side
	mm
	426
	570
	144

	C2
	Crush Zone 2 on left side
	mm
	266
	630
	364

	C3
	Crush Zone 3 on left side
	mm
	206
	697
	491

	C4
	Crush Zone 4 on right side
	mm
	206
	656
	450

	C5
	Crush Zone 5 on right side
	mm
	267
	534
	267

	C6
	Crush Zone 6 on right side
	mm
	423
	687
	264


BOTTOM OF BUMPER

	No.
	Measurement Description
	Units
	Pre-Test
	Post-Test
	Difference

	C1
	Crush Zone 1 on left side
	mm
	427
	582
	155

	C2
	Crush Zone 2 on left side
	mm
	263
	648
	385

	C3
	Crush Zone 3 on left side
	mm
	204
	707
	503

	C4
	Crush Zone 4 on right side
	mm
	204
	676
	472

	C5
	Crush Zone 5 on right side
	mm
	261
	541
	280

	C6
	Crush Zone 6 on right side
	mm
	424
	690
	266


CENTER GRILL

	No.
	Measurement Description
	Units
	Pre-Test
	Post-Test
	Difference

	C1
	Crush Zone 1 on left side
	mm
	325
	775
	450

	C2
	Crush Zone 2 on left side
	mm
	292
	759
	467

	C3
	Crush Zone 3 on left side
	mm
	268
	745
	477

	C4
	Crush Zone 4 on right side
	mm
	269
	742
	473

	C5
	Crush Zone 5 on right side
	mm
	291
	753
	462

	C6
	Crush Zone 6 on right side
	mm
	322
	760
	438


FRONT HOOD

	No.
	Measurement Description
	Units
	Pre-Test
	Post-Test
	Difference

	C1
	Crush Zone 1 on left side
	mm
	344
	726
	382

	C2
	Crush Zone 2 on left side
	mm
	310
	790
	480

	C3
	Crush Zone 3 on left side
	mm
	296
	783
	487

	C4
	Crush Zone 4 on right side
	mm
	295
	787
	492

	C5
	Crush Zone 5 on right side
	mm
	309
	782
	473

	C6
	Crush Zone 6 on right side
	mm
	344
	752
	408


	No.
	Measurement Description
	Units
	Pre-Test
	Post-Test
	Difference

	L
	Length of Region
	mm
	1532
	1424
	108


Note:  Crush is positive.  Measurements are taken from a reference plane in front of the vehicle.

DATA SHEET NO. 27... (Continued)

TARGET VEHICLE MEASUREMENTS

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


TARGET VEHCLE STRUCTURAL MEASUREMENT

	
	Elements
	Pre-Test (mm)

	1
	Total Length
	4800

	2
	Total Width
	1822

	3
	Bumper Top Height
	540

	4
	Bumper Bottom Height
	430

	5
	Longitudinal Member Top Height
	529

	6
	Distance between Longitudinal Members
	858

	7
	Longitudinal Member Width
	85

	8
	Engine Top Height
	837

	9
	Engine Bottom Height
	189

	10
	Engine and gearbox width
	895

	11
	Front bumper-engine distance
	580

	12
	Front shock absorber fixing height
	833

	13
	Bonnet leading edge height
	651

	14
	Front shock absorber fixing width
	964

	15
	Front bumper – front axle distance
	1017

	16
	Front axle – a pillar distance
	443

	17
	A-pillar – B-pillar distance
	1090

	18
	B-Pillar – rear axle distance
	1089

	19
	B-pillar – C-pillar distance
	988

	20
	Roof sill bottom height
	1311

	21
	Roof sill top height
	1427

	22
	Floor sill bottom height
	203

	23
	Floor sill top height
	333


DATA SHEET NO. 28
TARGET VEHICLE PHOTOGRAPHIC REFERENCE TARGET LOCATIONS
	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


[image: image35.wmf] 

A

 

B

 

C

 

D

 

5

 

4

 

3

 

2

 

1

 


	Item
	Value

	A
	354

	B
	911

	C
	2418

	D
	610

	 E
	1772

	F
	

	G
	1788

	H
	1037

	I
	1437

	J
	935

	K
	933

	L
	1495

	M
	1059

	N
	1490

	O
	936

	P
	942

	Q
	1432


DATA SHEET NO. 29
TARGET VEHICLE INTRUSION MEASUREMENTS

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


DOOR OPENING WIDTH

	Item
	Description
	Units
	Pre-Test
	Post-Test
	Difference

	A
	Left Side Upper
	mm
	1044
	1041
	3

	B
	Left Side Lower
	mm
	1030
	1023
	7

	D
	Right Side Upper
	mm
	1042
	1035
	7

	E
	Right Side Lower
	mm
	1024
	1021
	3


WHEELBASE MEASUREMENTS

	Item
	Description
	Units
	Pre-Test
	Post-Test
	Difference

	C
	Left Side Wheelbase
	mm
	2740
	2701
	39

	F
	Right Side Wheelbase
	mm
	2740
	2597
	143



[image: image11.wmf] 

C

 

B

 

A

 

 

F

 

E

 

D

 


DATA SHEET NO. 29... (Continued)

TARGET VEHICLE INTRUSION MEASUREMENTS

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


DRIVER COMPARTMENT INTRUSION

	Item
	Description
	Units
	Pre-Test
	Post-Test
	Difference

	AB
	Door Opening (Inside window jam)
	mm
	1044
	1041
	3

	CX
	Left Knee Bolster to X
	mm
	354
	346
	8

	DX
	Right Knee Bolster to X
	mm
	330
	342
	-12

	EX
	Brake Pedal to X
	mm
	600
	586
	14

	FX
	Foot Rest to X
	mm
	638
	619
	19

	GX
	Center of Steering Column Wheel Hub to X
	mm
	95
	121
	-26


X = Front of Seat Track (stationary)

[image: image12.wmf] 

 A

 

 B

 

 X

 

G

 

 FX

 

 AB

 

 CX

 

 DX

 

 EX

 

GX

 


DRIVER COMPARTMENT  

DATA SHEET NO. 29... (Continued)
TARGET VEHICLE INTRUSION MEASUREMENTS

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


[image: image36.wmf] 

A

 

C

 

B

 

FRONT VIEW OF WINDSHIELD

 

Zero Point

 

Y

 

X

 


Measurement reference point for X and Z axis is the forward outboard seat mounting bolt.

Columns A through D are evenly spaced.

Rows 1 and 2 are on the toe kick portion of the floor pan. Rows 3, 4, and 5 are located on the most level portion of the floor pan.

Row 3 will be at the intersection of the toe kick and the level sections of the floor pan.

DRIVER FLOOR PAN X-AXIS

	
	Pre-Test
	Post-Test
	Difference

	
	A
	B
	C
	D
	A
	B
	C
	D
	A
	B
	C
	D

	1
	
	710
	702
	707
	
	687
	671
	692
	
	23
	31
	15

	2
	
	675
	668
	666
	
	657
	639
	652
	
	18
	29
	14

	3
	595
	592
	589
	587
	592
	591
	581
	577
	3
	1
	8
	10

	4
	396
	393
	388
	390
	394
	387
	380
	383
	2
	6
	8
	7

	5
	195
	193
	188
	190
	189
	187
	182
	186
	6
	6
	6
	4


DRIVER FLOOR PAN Z-AXIS

	
	Pre-Test
	Post-Test
	Difference

	
	A
	B
	C
	D
	A
	B
	C
	D
	A
	B
	C
	D

	1
	
	13
	18
	18
	
	23
	29
	25
	
	-10
	-11
	-7

	2
	
	-21
	-17
	-19
	
	-14
	-11
	-13
	
	-7
	-6
	-6

	3
	-62
	-66
	-65
	-66
	-69
	-78
	-80
	-81
	7
	12
	15
	15

	4
	-62
	-60
	-62
	-63
	-67
	-63
	-67
	-66
	5
	3
	5
	3

	5
	-61
	-59
	-62
	-49
	-53
	-56
	-56
	-43
	-8
	-3
	-6
	-6


DATA SHEET NO. 29... (Continued)

TARGET VEHICLE INTRUSION MEASUREMENTS

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


[image: image37.jpg]z22-23 Z24-25



Measurement reference point for X and Z axis is the forward outboard seat mounting bolt.

Columns A through D are evenly spaced.

Rows 1 and 2 are on the toe kick portion of the floor pan. Rows 3, 4, and 5 are located on the most level portion of the floor pan.

Row 3 will be at the intersection of the toe kick and the level sections of the floor pan.

PASSENGER FLOOR PAN X-AXIS

	
	Pre-Test
	Post-Test
	Difference

	
	A
	B
	C
	D
	A
	B
	C
	D
	A
	B
	C
	D

	1
	715
	714
	722
	
	691
	684
	692
	
	24
	30
	30
	

	2
	693
	692
	689
	
	676
	665
	670
	
	17
	27
	19
	

	3
	609
	609
	609
	613
	602
	600
	608
	609
	7
	9
	1
	4

	4
	412
	409
	413
	411
	412
	409
	413
	411
	0
	0
	0
	0

	5
	208
	208
	211
	212
	202
	208
	211
	212
	6
	0
	0
	0


PASSENGER FLOOR PAN Z-AXIS

	
	Pre-Test
	Post-Test
	Difference

	
	A
	B
	C
	D
	A
	B
	C
	D
	A
	B
	C
	D

	1
	31
	35
	29
	
	49
	50
	49
	
	-18
	-15
	-20
	

	2
	7
	13
	-3
	
	16
	19
	11
	
	-9
	-6
	-14
	

	3
	-64
	-61
	-61
	-59
	-68
	-54
	-58
	-55
	4
	-7
	-3
	-4

	4
	-71
	-60
	-60
	-57
	-78
	-65
	-59
	-57
	7
	5
	-1
	0

	5
	-54
	-59
	-62
	-56
	-61
	-59
	-57
	-44
	7
	0
	-5
	-12


DATA SHEET NO. 29... (Continued)

TARGET VEHICLE INTRUSION MEASUREMENTS

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


[image: image38.jpg]



UNDERBODY FLOORBOARD DEFORMATION

	Measurement
	Pre-Test
	Post-Test
	Difference

	A
	273
	269
	4

	B
	416
	416
	0

	C
	433
	433
	0

	D
	270
	269
	1

	E
	305
	300
	5

	F
	422
	422
	0

	G
	420
	420
	0

	H
	305
	296
	9


DATA SHEET NO. 30
TARGET VEHICLE IIHS MEASUREMENTS

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


	Location
	
	Difference

	A Pillar
	X
	-2

	
	Y
	10

	
	Z
	2

	B Pillar
	X
	-1

	
	Y
	5

	
	Z
	2

	Brake Pedal
	X
	2

	
	Y
	7

	
	Z
	8

	Footrest
	X
	-18

	
	Y
	12

	
	Z
	8

	Knee Bolster Left
	X
	5

	
	Y
	5

	
	Z
	-1

	Knee Bolster Right
	X
	7

	
	Y
	7

	
	Z
	-1

	Toepan Left
	X
	-32

	
	Y
	5

	
	Z
	18

	Toepan Center
	X
	-35

	
	Y
	4

	
	Z
	18

	Toepan Right
	X
	-57

	
	Y
	8

	
	Z
	19

	Outboard Front Seat Bolt
	X
	-1

	
	Y
	4

	
	Z
	0

	Inboard Front Seat Bolt
	X
	1

	
	Y
	6

	
	Z
	-6

	Inboard Aft Seat Bolt
	X
	-1

	
	Y
	1

	
	Z
	2

	Outboard Aft Seat Bolt
	X
	-1

	
	Y
	4

	
	Z
	2


DATA SHEET NO. 30… (Continued)

TARGET VEHICLE IIHS MEASUREMENTS

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


	Location
	
	Difference

	Bumper 1 (Extreme LH)
	X
	-183

	
	Y
	-64

	
	Z
	-3

	Bumper 2
	X
	-218

	
	Y
	-81

	
	Z
	-8

	Bumper 3
	X
	-368

	
	Y
	-77

	
	Z
	-17

	Bumper 4
	X
	-443

	
	Y
	-31

	
	Z
	-88

	Bumper 5
	X
	-481

	
	Y
	-29

	
	Z
	-112

	Bumper 6 (Mid)
	X
	-507

	
	Y
	-22

	
	Z
	-137

	Bumper 7
	X
	-433

	
	Y
	-9

	
	Z
	-116

	Bumper 8
	X
	-357

	
	Y
	7

	
	Z
	-93

	Bumper 9
	X
	-271

	
	Y
	11

	
	Z
	-69

	Bumper 10
	X
	-238

	
	Y
	11

	
	Z
	-51

	Bumper 11 (Extreme RH)
	X
	-284

	
	Y
	42

	
	Z
	-57


X – Positive forward, Y – Positive to the left, Z – Positive up

DATA SHEET NO. 31
CAMERA LOCATIONS

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


	No.
	Camera View
	Location (mm) *
	Lens (mm)
	Speed (fps)

	
	
	X
	Y
	Z
	
	

	1
	West Wide
	829
	15319
	1670
	13
	**

	2
	West Bullet Rear Close-Up
	889
	9123
	1580
	25
	1000

	3
	West Bullet Front Close-Up
	1719
	8972
	1497
	25
	1000

	4
	West Target Front Close-Up
	-5702
	9833
	1580
	25
	1005

	5
	West Target Rear Close-Up
	-3299
	9368
	1603
	25
	948

	6
	East Bullet Rear Close-Up
	5941
	-36256
	1779
	13
	***

	7
	East Bullet Front Close-Up
	-1601
	-18043
	1547
	13
	833

	8
	East Target Front Close-Up
	-425
	-14376
	1627
	50
	***

	9
	East Target Rear Close-Up
	-1065
	-14145
	1759
	50
	***

	10
	East Wide
	-6319
	-12890
	1701
	35
	1000

	11
	East Overall
	3380
	-12624
	1727
	35
	833

	12
	Overhead
	532
	0
	4831
	8
	***

	13
	Bullet Driver Onboard
	
	
	
	8
	**

	14
	Bullet Passenger Onboard
	
	
	
	8
	**

	15
	Target Driver Onboard
	
	
	
	8
	**

	16
	Target Passenger Onboard
	
	
	
	8
	**


*COORDINATES:

+X = film plane forward of impact plane





+Y = film plane to right of monorail centerline



+Z = film plane above ground level

**No Timing

***Camera did not run



DATA SHEET NO. 31... (Continued)

CAMERA LOCATIONS

	Bullet Vehicle:
	1999 Dodge Grand Caravan SE

	Target Vehicle:
	2004 Honda Accord EX

	NHTSA No.:
	RX0322

	NHTSA No.:
	R45344


[image: image39.wmf] 

X

 

Y

 

10

 

4

 

5

 

6

 

2

 

1

 

3

 

8

 

9

 

7

 

16

 

15

 

14

 

13

 

12

 

11

 

7

 











� EMBED Word.Picture.8  ���





� EMBED Word.Picture.8  ���





� EMBED Word.Picture.8  ���





� EMBED Word.Picture.8  ���





Driver





Passenger
























































� EMBED Word.Picture.8  ���





� EMBED Word.Picture.8  ���








� EMBED WP10Doc  ���






































� EMBED MSPhotoEd.3  ���





� EMBED Word.Picture.8  ���








� EMBED Word.Picture.8  ���











� EMBED Word.Picture.8  ���





� EMBED Word.Picture.8  ���





� EMBED Word.Picture.8  ���





� EMBED Word.Picture.8  ���





Passenger





Driver








8

[image: image40.wmf] 

A

 

C

 

B

 

FRONT VIEW OF WINDSHIELD

 

Zero Point

 

Y

 

X

 

_1062413874.doc


GX







 EX







 DX







 CX







 AB







 FX







G







 X



















 B







 A












_1109423364.bin

_1143353706.doc


C







A



















B











FRONT VIEW OF WINDSHIELD







Zero Point







Y







X












_1145424676.doc


C







A



















B











FRONT VIEW OF WINDSHIELD







Zero Point







Y







X












_1150263051.doc


LEFT SIDE VIEW







STEERING COLUMN ASSEMBLY







1







2







3












_1144154334.doc


X







Y







15







10







4







5







7







6







2







1







3







8







11







16







9







14







13







12







7












_1143297012.doc


D







A







E







B







F







C







Lower Edge of Protected Zone







FRONT VIEW OF WINDSHIELD







Zero Point







Y







X












_1143353494.unknown

_1106736884.doc














1







2







3







4







5







A







B







C







D




















































































_1106736997.doc














1







2







3







4







5







A







B







C







D




















































































_1062398800.doc
[image: image1.png]






_1062412741.unknown

_1062413578.doc














C



















































































D



























A	































F















































B







E












_1062413357.doc
[image: image1.bmp][image: image2.bmp]













F











G











N















K







J







I



















Q



















B







C







A







O























L







E



















H







B











M















































D







P












_1062402346.doc
 





Dummy’s Centerline







TBI







'D' Ring







Shoulder Belt Portion















Lap Belt Portion











1/8" Thick Aluminum Plate















Locking Retractor















Outboard Anchor











Inboard Anchorage











Floorpan







Buckle Assembly



















PBU







PBL
















_1062410801.doc


D







A







E







B







F







C







Lower Edge of Protected Zone







FRONT VIEW OF WINDSHIELD







Zero Point







Y







X












_1062398663.doc


CD	Chest to Dash



CS	Chest to Steering Wheel Hub



HH	Head to Header



HW	Head to Windshield



HZ	Head to Roof



KDA	Knee to Dash Angle



KDL	Left Knee to Dash



KDR	Right Knee to Dash



NA	Nose to Rim Angle



NR	Nose to Rim



PA	Pelvic Angle



RA	Rim to Abdomen



SA	Seat Back Angle



SCA	Steering Column Angle



SH	Striker to H-Point



SK	Striker to Knee



ST	Striker to Head



SWA	Steering Wheel Angle



TA	Tibial Angle



WA	Windshield Angle







 HW







HZ







 PA(







 SA(







 TA(







 KDA(







 SCA(







 NA(







 WA(







  HH







 SWA(







AD	Arm to Door



HD	H-Point to Door



HR	Head to Side Header



HS	Head to Side Window



KK	Knee to Knee



SHY	Striker to H-Point



          (Y Axis)











ST







SK







NR







SH







Seat Back Angle Line







CS







KD







CD







STRIKER







SHY







HR







HD







AD







KK







HS















Vertical



Transverse



Plane







Vertical Longitudinal Planes







DRVR







PASS












