APPENDIX D
CHILD DUMMY DATA
SUMMARY OF TEST

	Measurement Description
	Units
	Threshold
	P3 ATD
	P4 ATD

	Head Injury Criteria (HIC36)
	N/A
	N/A
	788
	991

	Head Injury Criteria (HIC15)
	N/A
	390
	595
	594

	Max. Thorax Accel. (3msec Clip)
	G’s
	50
	52
	57



Both child dummies were instrumented with head, chest, and upper six axial neck force and moment sensors. 

The right rear (Position 3) child dummy (S/N 093) and left rear (Position 4) child dummy (S/N 090) were calibrated previous to this test. Child dummy certification information can be found in this Appendix.


Positions 3 and 4 were rear facing and used the vehicle LATCH attachment.
TEST VEHICLE WEIGHTS

	
	Units
	As Tested (ATW) (Axle)

	
	
	Front 
	Rear
	Total

	Left
	kg
	545.7
	427.3
	

	Right
	kg
	526.2
	415.9
	

	Ratio
	%
	56.0
	44.0
	

	Totals
	kg
	1071.9
	843.2
	1915.1


As tested weight of vehicle includes two ATDs, two CRABIs with CRS, 
cargo, equipment and instrumentation.
TEST NOTES


There was no valid data collected for:


LRP Child Seat Y after 100 msec.

TEST DUMMY INFORMATION

	Description
	Position 3 CRS
	Position 4 CRS

	Dummy Type / Serial No.
	12 month old CRABI / 093
	12 month old CRABI / 090

	Number of Data Channels
	12
	12

	Restraint System
	Graco Snugride
(Rear Facing)
	Compass I-410

(Rear Facing)


POST TEST SEAT DATA

	Location
	Seat Movement (mm)
	Seat Back Failure

	P1 (Left Front)
	0
	None

	P2 (Right Front)
	0
	None

	P3 (Right Rear)
	0
	None

	P4 (Left Rear)
	0
	None


VISIBLE DUMMY CONTACT POINTS

	Description
	Position 3 CRS
	Position 4 CRS

	Head Contact
	Back of head to seatback
	Back of head to seatback

	Upper Torso Contact
	None
	None

	Lower Torso Contact
	None
	None

	Left Foot Contact
	Foot to vehicle rear seat 
	Foot to vehicle rear seat 

	Right Foot Contact
	Foot to vehicle rear seat 
	Foot to vehicle rear seat 


CHILD DUMMY POSITIONING IN VEHICLE

	Child Restraint System (Position 3)
	Graco Snugride (Rear Facing)

	NHTSA No.
	M80301


	[image: image1.png]converted 3¢1f0e13.CMC - Microsoft Intemet Explorer =18] %]

Image 1

Dummy Measurements for Rear-facing CRS Passengers

X—2View





	
	[image: image2.png]Head to Roof

Head to Side Window
Chest to Door

Arm to Door

H-Point to Door

Knee to Knee

Foot to Foot

Nose to Front Seat Back
Chest to Front Seat Back
Knee to Front Seat Back
Toe to Front Seat Back
Seat Back Angle

Tor oIl







	Measurement
	Pre-Test (mm)
	Post-Test (mm)

	
	P3 CRS (093)
	P3 CRS (093)

	SA (deg)
	7.0
	

	HS
	485
	460

	CD
	353
	357

	AD
	248
	245

	HD
	276
	290

	HZ
	417
	410

	NB
	509
	542

	CB
	415
	451

	KK
	103
	118

	FF
	105
	130

	KB - LEFT
	195
	250

	KB – RIGHT
	195
	245

	TB - LEFT
	40
	111

	TB - RIGHT
	41
	143


All dimensions in mm (unless noted)

P3 – Right Rear Passenger (Rear Facing)
CHILD DUMMY POSITIONING IN VEHICLE

	Child Restraint System (Position 4)
	Compass I-410 (Rear Facing)

	NHTSA No.
	M80301


	[image: image3.png]converted 3¢1f0e13.CMC - Microsoft Intemet Explorer =18] %]

Image 1

Dummy Measurements for Rear-facing CRS Passengers

X—2View





	
	[image: image4.png]Head to Roof

Head to Side Window
Chest to Door

Arm to Door

H-Point to Door

Knee to Knee

Foot to Foot

Nose to Front Seat Back
Chest to Front Seat Back
Knee to Front Seat Back
Toe to Front Seat Back
Seat Back Angle

Tor oIl







	Measurement
	Pre-Test (mm)
	Post-Test (mm)

	
	P4 CRS (090)
	P4 CRS (090)

	SA (deg)
	7.0
	

	HS
	487
	510

	CD
	374
	413

	AD
	255
	243

	HD
	283
	281

	HZ
	365
	369

	NB
	467
	506

	CB
	388
	415

	KK
	95
	101

	FF
	100
	113

	KB - LEFT
	191
	220

	KB – RIGHT
	191
	230

	TB - LEFT
	26
	50

	TB - RIGHT
	27
	89


All dimensions in mm (unless noted)

P4 – Left Rear Passenger (Rear facing)
CRS PERFORMANCE DATA

	Child Restraint System (Position 3)
	Graco Snugride (Rear Facing)

	Child Restraint System (Position 4)
	Compass I-410 (Rear Facing)

	NHTSA No.
	M80301


POSITION 3 CRS POST-TEST INSPECTION

	Location
	Damage
	Remarks

	Upper Tether Strap
	
	

	Upper Tether Buckle
	
	

	Upper Tether Hook
	
	

	Vehicle Upper Tether Anchor
	None
	

	Lower Anchor Strap
	None
	

	Lower Anchor Buckle
	
	

	Lower Anchor Hooks
	None
	

	Vehicle Lower CRS Anchors
	None
	

	Five Point Harness Connections
	None
	

	Cracks on CRS
	None
	

	Fabric Tears on CRS
	None
	

	Vehicle Seat Structure
	None
	

	Vehicle Seat Fabric Tears
	None
	

	Child Dummy
	None
	CRABI 12 Month Old


POSITION 4 CRS POST-TEST INSPECTION 

	Location
	Damage
	Remarks

	Upper Tether Strap
	
	

	Upper Tether Buckle
	
	

	Upper Tether Hook
	
	

	Vehicle Upper Tether Anchor
	None
	

	Lower Anchor Strap
	None
	

	Lower Anchor Buckle
	None
	

	Lower Anchor Hooks
	
	

	Vehicle Lower CRS Anchors
	None
	

	Five Point Harness Connections
	None
	

	Cracks on CRS
	None
	

	Fabric Tears on CRS
	None
	

	Vehicle Seat Structure
	None
	

	Vehicle Seat Fabric Tears
	None
	

	Child Dummy
	None
	CRABI 12 Month Old


CRS CAMERA DATA

	Child Restraint System (Position 3)
	Graco Snugride (Rear Facing)

	Child Restraint System (Position 4)
	Compass I-410 (Rear Facing)

	NHTSA No.
	M80301



[image: image5.wmf]2

1


	No.
	Camera View
	Location (mm) *
	Angle (deg)
	Lens

(mm)
	Speed

(fps)

	
	
	X
	Y
	Z
	
	
	

	1
	Left Side CRS Lateral View
	
	
	
	
	8
	1000

	2
	Right Side CRS Lateral View
	
	
	
	
	8
	1000


*COORDINATES:

+X = film plane rearward of barrier

+Y = film plane to right of monorail centerline

+Z = film plane above ground level
PHOTOGRAPHS

TABLE OF PHOTOGRAPHS

	
	
	Page No.

	Figure No.  AUTONUM  \* Arabic 
	Close-up View of Position 3 CRS Label
	D-8

	Figure No.  AUTONUM  \* Arabic 
	Pre-Test Front View of Position 3 CRS
	D-9

	Figure No.  AUTONUM  \* Arabic 
	Post-Test Front View of Position 3 CRS
	D-9

	Figure No.  AUTONUM  \* Arabic 
	Pre-Test Rear View of Position 3 CRS
	D-10

	Figure No.  AUTONUM  \* Arabic 
	Post-Test Rear View of Position 3 CRS
	D-10

	Figure No.  AUTONUM  \* Arabic 
	Pre-Test Left Side View of Position 3 CRS
	D-11

	Figure No.  AUTONUM  \* Arabic 
	Post-Test Left Side View of Position 3 CRS
	D-11

	Figure No.  AUTONUM  \* Arabic 
	Pre-Test Right Side View of Position 3 CRS
	D-12

	Figure No.  AUTONUM  \* Arabic 
	Post-Test Right Side View of Position 3 CRS
	D-12

	Figure No.  AUTONUM  \* Arabic 
	Close-up View of Position 4 CRS Label
	D-13

	Figure No.  AUTONUM  \* Arabic 
	Pre-Test Front View of Position 4 CRS
	D-14

	Figure No.  AUTONUM  \* Arabic 
	Post-Test Front View of Position 4 CRS
	D-14

	Figure No.  AUTONUM  \* Arabic 
	Pre-Test Rear View of Position 4 CRS
	D-15

	Figure No.  AUTONUM  \* Arabic 
	Post-Test Rear View of Position 4 CRS
	D-15

	Figure No.  AUTONUM  \* Arabic 
	Pre-Test Left Side View of Position 4 CRS
	D-16

	Figure No.  AUTONUM  \* Arabic 
	Post-Test Left Side View of Position 4 CRS
	D-16

	Figure No.  AUTONUM  \* Arabic 
	Pre-Test Right Side View of Position 4 CRS
	D-17

	Figure No.  AUTONUM  \* Arabic 
	Post-Test Right Side View of Position 4 CRS
	D-17

	Figure No.  AUTONUM  \* Arabic 
	Pre-Test Position 3 Left Side View
	D-18

	Figure No.  AUTONUM  \* Arabic 
	Post-Test Position 3 Left Side View
	D-18

	Figure No.  AUTONUM  \* Arabic 
	Pre-Test Position 4 Left Side View
	D-19

	Figure No.  AUTONUM  \* Arabic 
	Post-Test Position 4 Left Side View
	D-19

	Figure No.  AUTONUM  \* Arabic 
	Pre-Test Position 3 Right Side View
	D-20

	Figure No.  AUTONUM  \* Arabic 
	Post-Test Position 3 Right Side View
	D-20

	Figure No.  AUTONUM  \* Arabic 
	Pre-Test Position 4 Right Side View
	D-21

	Figure No.  AUTONUM  \* Arabic 
	Post-Test Position 4 Right Side View
	D-21

	Figure No.  AUTONUM  \* Arabic 
	Post-Test Position 3 & 4 Feet Contact
	D-22

	Figure No.  AUTONUM  \* Arabic 
	Post-Test Position 4 Head Contact
	D-22


CHILD DUMMY RESPONSE DATA TRACES

TABLE OF DATA PLOTS

	
	
	Page No.

	
	List of Data Plots Provided in this Section
	

	Figure No.  AUTONUM  \* Arabic 
	LRP Child Head X Acceleration vs. Time
	D-25

	Figure No.  AUTONUM  \* Arabic 
	LRP Child Head Y Acceleration vs. Time
	D-25

	Figure No.  AUTONUM  \* Arabic 
	LRP Child Head Z Acceleration vs. Time
	D-25

	Figure No.  AUTONUM  \* Arabic 
	LRP Child Head Resultant Acceleration vs. Time
	D-25

	Figure No.  AUTONUM  \* Arabic 
	LRP Child Chest X Acceleration vs. Time
	D-26

	Figure No.  AUTONUM  \* Arabic 
	LRP Child Chest Y Acceleration vs. Time
	D-26

	Figure No.  AUTONUM  \* Arabic 
	LRP Child Chest Z Acceleration vs. Time
	D-26

	Figure No.  AUTONUM  \* Arabic 
	LRP Child Chest Resultant Acceleration vs. Time
	D-26

	Figure No.  AUTONUM  \* Arabic 
	RRP Child Head X Acceleration vs. Time
	D-27

	Figure No.  AUTONUM  \* Arabic 
	RRP Child Head Y Acceleration vs. Time
	D-27

	Figure No.  AUTONUM  \* Arabic 
	RRP Child Head Z Acceleration vs. Time
	D-27

	Figure No.  AUTONUM  \* Arabic 
	RRP Child Head Resultant Acceleration vs. Time
	D-27

	Figure No.  AUTONUM  \* Arabic 
	RRP Child Chest X Acceleration vs. Time
	D-28

	Figure No.  AUTONUM  \* Arabic 
	RRP Child Chest Y Acceleration vs. Time
	D-28

	Figure No.  AUTONUM  \* Arabic 
	RRP Child Chest Z Acceleration vs. Time
	D-28

	Figure No.  AUTONUM  \* Arabic 
	RRP Child Chest Resultant Acceleration vs. Time
	D-28

	
	
	

	
	
	


	The following dummy response data can be found in the R&D section of the NHTSA website at ww.nhtsa.dot.gov

	LRP Child Upper Neck Force X vs. Time

	LRP Child Upper Neck Force Y vs. Time

	LRP Child Upper Neck Force Z vs. Time

	LRP Child Upper Neck Force Resultant vs. Time

	LRP Child Upper Neck Moment X vs. Time

	LRP Child Upper Neck Moment Y vs. Time

	LRP Child Upper Neck Moment Z vs. Time

	LRP Child Upper Neck Moment Resultant vs. Time

	RRP Child Upper Neck Force X vs. Time

	RRP Child Upper Neck Force Y vs. Time

	RRP Child Upper Neck Force Z vs. Time

	RRP Child Upper Neck Force Resultant vs. Time

	RRP Child Upper Neck Moment X vs. Time

	RRP Child Upper Neck Moment Y vs. Time

	RRP Child Upper Neck Moment Z vs. Time

	RRP Child Upper Neck Moment Resultant vs. Time

	LRP Child Seat X Acceleration vs. Time

	LRP Child Seat Y Acceleration vs. Time

	LRP Child Seat Z Acceleration vs. Time

	LRP Child Seat Resultant Acceleration vs. Time

	RRP Child Seat X Acceleration vs. Time

	RRP Child Seat Y Acceleration vs. Time

	RRP Child Seat Z Acceleration vs. Time

	RRP Child Seat Resultant Acceleration vs. Time


CHILD DUMMY CALIBRATION INFORMATION


























_1079857546.unknown

