REPORT NUMBER:  SNCAP-CAL-07-09
NEW CAR ASSESSMENT PROGRAM 

SIDE IMPACT TEST

FUJI HEAVY INDUSTRIES, LTD
2008 SUBARU IMPREZA
4-DOOR SEDAN
NHTSA NUMBER:  M85501
PREPARED BY:

CALSPAN CORPORATION 

P.O. BOX 400

BUFFALO, NEW YORK 14225

[image: image89.emf] 

LEFT SIDE VIEW  

1   2  

3  

4   5  

6   8  

7  

9  

10  

11  

13  

12   14  

15  

16  

17  

18  

5   4  


Test Date:  September 13, 2007
FINAL REPORT

PREPARED FOR:

U.S. DEPARTMENT OF TRANSPORTATION

NATIONAL HIGHWAY TRAFFIC SAFETY ADMINISTRATION

RULEMAKING

OFFICE OF CRASHWORTHINESS STANDARDS

MAIL CODE: NVS-111

1200 NEW JERSEY AVE SE, ROOM W43-410

WASHINGTON, D.C. 20590


This final test report was prepared for the U.S. Department of Transportation, National Highway Traffic Safety Administration, in response to Contract Number DTNH22-03-D-22005.


This publication is distributed by the U.S. Department of Transportation, National Highway Traffic Safety Administration, in the interest of information exchange. The opinions, findings and conclusions expressed in this publication are those of the author(s) and not necessarily those of the Department of Transportation or the National Highway Traffic Safety Administration. The United States Government assumes no liability for its contents or use thereof. If trade or manufacturers' names or products are mentioned it is only because they are considered essential to the object of the publication and should not be construed as an endorsement. The United States Government does not endorse products or manufacturers.

	Prepared by:
	
	
	Date:
	

	
	Vincent M. Paolini, Project Engineer
	
	
	

	
	
	
	
	

	Reviewed by:
	
	
	Date:
	

	
	Michael J. Kilgallon, Program Manager

Transportation Sciences Center
	
	
	

	
	
	
	
	


Technical Report Documentation Page

	1. Report No.

SNCAP-CAL-07-09
	2. Government Accession No. 


	3. Recipient's Catalog No.

	4. Title and Subtitle

Final Report of New Car Assessment Program

2008 Subaru Impreza 4-Door Sedan
NHTSA No.: M85501

	5. Report Date

September 13, 2007

	
	6. Performing Organization Code

CAL

	7. Author(s)

Vincent M. Paolini, Project Engineer
	8. Performing Organization Report No.

8765-SNCAP-26

	9. Performing Organization Name and Address

Calspan Corporation Transportation Sciences Center

P.O. Box 400

Buffalo, New York 14225
	10. Work Unit No.



	
	11. Contract or Grant No.

DTNH22-03-D-22005

	12. Sponsoring Agency Name and Address

 U.S. Department of Transportation

 National Highway Traffic Safety Administration

 Rulemaking, Office of Crashworthiness Standards

 Mail Code: NVS-111

 1200 New Jersey Ave., SE, Room W43-410

 Washington, D.C. 20590
	13. Type of Report and Period Covered: Final Report

September 2007


	
	14. Sponsoring Agency Code

NVS-111

	15. Supplementary Notes

	16. Abstract

A 55/28 kph 900 Impact Moving Deformable Barrier NCAP Side Impact Test was conducted on the subject 2008 Subaru Impreza 4-Door Sedan in accordance with the specifications of the Office of Crashworthiness Standards Test Procedure for the generation of consumer information on vehicle side crash protection. This test was conducted at the Calspan Corporation Crash Test Facility in Buffalo, New York, on September 13, 2007.The impact velocity of the Moving Deformable Barrier (MDB) was 62.6 km/h, and the ambient temperature at the struck side (driver side) of the vehicle was 22°C. The target vehicle's maximum post test static crush was 277 mm at level 3. The test vehicle’s occupant performance is as follows:

DRIVER
PASS.

Left Upper Rib (LUR) Accel., g                               

55.5
63.0
Left Lower Rib (LLR) Accel., g                               

39.3
68.8
Lower Spine (T12) Accel., g                                    

43.7
75.1
Thoracic Trauma Index (TTI)                                 

50
72
Pelvis (PEV) Accel., g                                            

53
71
HIC

142.0
312.1
The doors on the struck side of the vehicle did not separate from the body at the hinges or latches and the opposite doors did not open during the side impact event. 

	17. Key Words

New Car Assessment Program (NCAP)

Side Impact

Side Impact Hybrid III Dummy (SID/HIII)

Occupant Side Impact Protection
	18. Distribution Statement

Copies of this report are available from:

National Highway Traffic Safety Adm.

Technical Ref. Division,

1200 New Jersey Ave. SE

Washington, D.C. 20590

	19. Security Class. (of this report)

Unclassified
	20. Security Class. (of this page)

Unclassified
	21. No. of Pages

121
	22. Price



Form DOT F1700.7 (8-72)

TABLE OF CONTENTS

	Section
	
	Page No.

	1
	Purpose and Test Procedure
	1-1

	2
	Summary of NCAP Side Impact Test
	2-1

	
	
	

	Data Sheet No.
	
	Page No.

	1
	General Test and Vehicle Parameter Data
	2-3

	2
	Test Vehicle Tire Information
	2-5

	3
	Test Vehicle Information
	2-6

	4
	Moving Deformable Barrier (MDB) Summary of Results
	2-8

	5
	Post Test Observations
	2-9

	6
	Vehicle Pre-Test and Post Test Measurements
	2-10

	7
	SID/HIII Longitudinal Clearance Dimensions
	2-11

	8
	SID/HIII Lateral Clearance Dimensions
	2-12

	9
	Vehicle Side Measurements
	2-13

	10
	Vehicle Exterior Crush Profiles
	2-14

	11
	Vehicle Damage Profile Distances
	2-15

	12
	Deformable Barrier Honeycomb Face Static Crush
	2-16

	13
	Vehicle Accelerometer Locations 
	2-17

	14
	MDB Accelerometer Locations 
	2-18

	15
	Vehicle Structural Measurements
	2-19

	16
	High Speed Camera Locations and Data
	2-20

	17
	Summary of FMVSS 301 Data
	2-21

	
	
	

	Appendix
	
	Page No

	A
	Photographs
	A-1

	B
	SID/HIII, Vehicle and MDB Response Data
	B-1

	C
	SID/HIII Configuration and Performance Verification Data
	C-1

	D
	Test Equipment and Calibration Information
	D-1


SECTION 1

PURPOSE AND TEST PROCEDURE

PURPOSE


This side impact test was conducted as part of the FY' 2008 test program sponsored by the National Highway Traffic Safety Administration (NHTSA), under Contract No. DTNH22-03-D-22005. The purpose of this test was to evaluate side impact protection in a 2008 Subaru Impreza 4-Door Sedan manufactured by Fuji Heavy Industries, LTD.  The side impact test was conducted in accordance with the Laboratory Test Procedure for New Car Assessment Program Side Impact Testing dated July 1997.
SECTION 2
SUMMARY OF NCAP SIDE IMPACT TEST


A model year 2008 Subaru Impreza 4-Door Sedan was impacted on the left (driver's) side by a Moving Deformable Barrier (MDB) which was moving forward in a 27° crabbed position to the tow road guidance system at a velocity of 62.6 km/h. The specified impact velocity range is from 61.1 to 62.7 km/h. The test (target) vehicle was stationary and positioned 63° to the line of forward motion. The weight of the vehicle as tested was 1596.0 kg and the test weight of the MDB was 1362.5 kg. The test was conducted at the Calspan Corporation Transportation Sciences Center on September 13, 2007.


One (1) real-time motion picture camera and ten (10) high-speed motion picture cameras were used to document the impact event. The pre-test and post-test conditions were recorded by one (1) real-time motion picture camera. Camera locations and pertinent camera information are documented in the data sheets. Pre- and post-test photographs of the vehicle and Side Impact Dummies (SID/HIIIs) can be found in Appendix A.

Two 50th percentile adult male SID/HIII's were placed in the driver (P1) and left rear passenger  (P4) designated seating positions according to instructions specified in the Laboratory Test Procedure for New Car Assessment Program Side Impact Testing dated July 1997. Each SID/HIII was instrumented in the following locations: 

(     Left Upper Rib (LUR) uni-axial accelerometer (Y-axis primary and redundant)

(     Left Lower Rib (LLR) uni-axial accelerometer (Y-axis primary and redundant)

(     Lower Thoracic Spine (T12) uni-axial accelerometer (Y-axis primary and redundant)

(     Pelvic (PEV) section uni-axial accelerometer (Y-axis primary and redundant)

(     Head Center of Gravity (CG) tri-axial accelerometers (X, Y and Z axes primary and redundant)

· Upper Neck load cell (Fx, Fy, Fz, Mx, My, Mz)


The test vehicle was instrumented with twenty-one (21) structural accelerometers and the MDB was instrumented with five (5) accelerometers.
2.2 GENERAL COMMENTS


The test vehicle sustained a maximum static crush of 277 mm at level 3, 750 mm rearward of the left vertical impact point. The driver and passenger SID/HIII's, Serial Nos. 269 and 270 respectively, were calibrated just prior to this test. 


Test data and observations are presented in this section of the report. Appendix A contains the still photograph prints. Appendix B contains the driver and passenger SID/HIII's, vehicle, and MDB response data traces. Appendix C contains the SID/HIII's configuration and performance verification data. Appendix D contains the test equipment information.


The occupant data is summarized below:

	ATD position
	HIC(36)
	T1
	T2
	TTI (G’s)
	Peak Pelvis (G’s)

	Driver
	142.0
	43.4
	79.3
	50
	53

	Passenger
	312.1
	40.5
	67.5
	72
	71


	SUPPLEMENTAL RESTRAINT INFORMATION

	Restraint Type
	Left Front (Driver)
	Left Rear (Passenger)

	
	Installed
	Deployed
	Installed
	Deployed

	Front Airbag
	Yes
	No
	NA
	NA

	Side Torso Airbag
	Yes
	Yes
	NA
	NA

	Side Head/Torso Combination Airbag
	NA
	NA
	NA
	NA

	Curtain Airbag
	Yes
	Yes
	Yes
	Yes


The test instrumentation data listed in Appendix B can be found on the NHTSA website: www.nhtsa.dot.gov.
DATA SHEET NO. 1

GENERAL TEST AND VEHICLE PARAMETER DATA

	Test Vehicle:
	2008 Subaru Impreza
	NHTSA No.
	M85501

	Test Program:
	NCAP Side Impact
	Test Date:
	September 13, 2007


	TEST VEHICLE INFORMATION AND VEHICLE OPTIONS

	Make
	Fuji Heavy Industries, LTD
	
	Driver Front Airbag
	Yes

	Model
	Impreza
	
	Driver Side Curtain Airbag
	Yes

	Body Style
	4-Door Sedan
	
	Driver Side Torso Airbag
	Yes

	NHTSA No.
	M85501
	
	Driver Pretensioners
	Yes

	VIN
	JF1GE61618G502174
	
	Driver Load Limiters
	Yes

	Color
	Dark Gray
	
	Driver Power Seats
	No

	Engine Disp.(L)
	2.5
	
	Rear Pass. Side Curtain Airbag
	Yes

	Engine Cylinders
	4
	
	Rear Pass. Side Torso Airbag
	No

	Engine Placement
	Longitudinal
	
	Rear Pass. Pretensioners
	Yes

	Transmission Type
	Manual
	
	Rear Pass. Load Limiters
	Yes

	Transmission Speeds
	5
	
	Rear Pass. Power Seats
	NA

	Final Drive
	4 Wheel
	
	Tilt Wheel
	Yes

	Air Conditioning
	Yes
	
	Anti-lock Brakes
	Yes

	Power Steering
	Yes
	
	Traction Control
	Yes

	Power Brakes
	Yes
	
	Power Windows
	Yes

	Delivery Date
	8/23/07
	
	Power Door Locks
	Yes

	Odometer Reading (km)
	39
	
	Automatic Door Locks (ADL)
	No

	Dealer
	West Herr Subaru
	
	Owner’s Manual Details Instructions on Disabling ADLs
	NA


	DATA FROM CERTIFICATION LABEL

	Manufactured By
	Fuji Heavy Industries, LTD
	
	GVWR (kg)
	1950

	
	
	
	GAWR Front (kg)
	990

	Date of Manufacture
	06/2007
	
	GAWR Rear (kg)
	1000


	VEHICLE CAPACITY DATA

	Measured Parameter
	Front
	Rear
	Third
	Total

	Type of Seats
	Bucket
	Bench
	
	

	Number Of Occupants
	2
	3
	
	5

	Capacity Wt. (VCW) (kg)
	
	
	
	385.0

	Cargo Wt. (RCLW) (kg)
	
	
	
	44.8


DATA SHEET NO. 1 (continued)

GENERAL TEST AND VEHICLE PARAMETER DATA

	Test Vehicle:
	2008 Subaru Impreza
	NHTSA No.
	M85501

	Test Program:
	NCAP Side Impact
	Test Date:
	September 13, 2007


	TEST VEHICLE WEIGHTS

	
	Units
	As Delivered (UVW) (Axle)
	Fully Loaded (Axle)
	As Tested (ATW) (Axle)

	
	
	Front
	Rear
	Total
	Front
	Rear
	Total
	Front
	Rear
	Total

	Left
	kg
	397.0
	311.0
	
	449.5
	395.5
	
	436.0
	393.5
	

	Right
	kg
	392.0
	294.0
	
	397.5
	361.5
	
	404.0
	362.5
	

	Ratio
	%
	56.6
	43.4
	
	53.0
	47.0
	
	52.6
	47.4
	

	Totals
	kg
	789.0
	605.0
	1394.0
	853.5
	756.0
	1609.5
	840.0
	756.0
	1596.0


	TARGET TEST WEIGHT CALCULATION

	Measured Parameter
	Units
	Value

	Total Delivered Weight (UVW)
	kg
	1394.0

	Weight of 2 P572E ATDs (81.2 kg each)
	kg
	162.4

	Rated Cargo/Luggage Weight (RCLW)
	kg
	44.8

	Calculated Vehicle Target Weight (TVTW)
	kg
	1601.2

	* Actual As Tested Weight (ATW) will be TVTW -4.5/-9.1 kg

	Weight of Ballast (including instrumentation package and cameras):
	39.6
	kg


	TEST VEHICLE ATTITUDES

	
	Units
	LF
	RF
	LR
	RR

	As Delivered
	mm
	700
	710
	700
	701

	Fully Loaded 
	mm
	679
	701
	664
	674

	As Tested
	mm
	682
	701
	663
	673


	Test Vehicle vertical impact line AND CG

	Measurement Description
	Units
	Value

	Test Vehicle Wheel Base
	mm
	2627

	Target Impact Point Aft of Front Axle
	mm
	374

	Actual Impact Point Aft of Front Axle
	mm
	374

	As Tested CG (aft of front axle)
	mm
	1244


DATA SHEET NO. 2

TEST VEHICLE TIRE INFORMATION

	Test Vehicle:
	2008 Subaru Impreza
	NHTSA No.
	M85501

	Test Program:
	NCAP Side Impact
	Test Date:
	September 13, 2007


[image: image1.png]



	DATA FROM TIRE PLACARD

	Measured Parameter
	Front
	Rear

	Maximum Tire Pressure (kPa)
	300
	300

	Cold / Test Pressure (kPa)
	230
	230

	Recommended Tire Size
	P205/55R16
	P205/55R16

	Tire Size on Vehicle
	P205/55R16
	P205/55R16

	Tire Manufacturer
	Bridgestone
	Bridgestone

	Tire Name
	Potenza RE 
	Potenza RE 

	Tire Type
	Passenger
	Passenger

	Tire Width (mm)
	205
	205

	Ratio of Height to Width (aspect ratio)
	55
	55

	Radial
	Yes
	Yes

	Wheel Diameter
	16
	16

	Load Index & Speed Symbol
	89V
	89V

	Treadwear
	260
	260

	Traction Grade
	A
	A

	Temperature Grade
	A
	A


DATA SHEET NO. 3

TEST VEHICLE INFORMATION

	Test Vehicle:
	2008 Subaru Impreza
	NHTSA No.
	M85501

	Test Program:
	NCAP Side Impact
	Test Date:
	September 13, 2007


NORMAL DESIGN RIDING POSITION

The driver and passenger seat back is positioned to the manufacturer's designated angle.
[image: image69.wmf] 

Upright

 

Position

 

Adjuster

 

Inclinometer

 

Seat

 

Back

 

Angle

 

(Degrees)

 

FRONT SEAT ASSEMBLY

 

Seat Cushion

 


	SEAT BACK POSITION

	
	Driver Seat
	Rear Seat

	Test Detent (forward-most detent defined as 0)
	Detent 7
	Not Adjustable

	Angle (deg. from forward-most locking position)
	14
	Not Adjustable

	Alternative Measurements to Verify Test Position
	N/A
	N/A


[image: image70.wmf] 

Seat Cushion

 

Seat Travel Area

 

 

A

 

A

 

B

 


SEAT FORE/AFT POSITIONS

	The total seat travel was measured from forward most position to rearmost position irrespective of vertical seat height in those positions. The seat was set at the longitudinal mid position with vertical adjustment at the lowest position obtainable for both the driver and passenger.
	


	SEAT FORE/AFT POSITION

	
	Driver Seat
	Rear Seat

	Total Fore/Aft Travel (A) (mm)
	254
	N/A (fixed)

	Test Position (B) (mm)
	127
	N/A

	Test Detent (forward-most detent defined as 0)
	9
	N/A (fixed)

	Total Number of Detents (including 0)
	19
	N/A (fixed)


Data Sheet No. 3 (CONTINUED)

TEST VEHICLE INFORMATION

	Test Vehicle:
	2008 Subaru Impreza
	NHTSA No.
	M85501

	Test Program:
	NCAP Side Impact
	Test Date:
	September 13, 2007


FUEL SYSTEM INFORMATION
[image: image71.wmf] 

Seat Cushion

 

 

A

 

A

 

B

 

Seat Travel Area

 

The test vehicle is equipped with an electric fuel pump. The fuel pump operates for approximately two seconds after the ignition is placed in the “ON” position, after which the fuel pump automatically shuts off. The fuel filler door is located on the left rear fender. The standard fuel tank occupies the area under the rear seat.
	Fuel Tank Capacity

	
	Liters

	Usable Capacity of “Standard” Fuel Tank
	60.0

	Usable Capacity of “Optional” Fuel Tank
	N/A

	Stoddard Used For Test (92%-94% of Fuel Tank Usable Capacity)
	55.2


Steering Column AdJUSTMENT
[image: image72.wmf] 

Vapor Volume

 

Unusable Capacity

 

Filler Neck Volume

 

Filler Cap

 

VEHICLE FUEL TANK ASSEMBLY

 

Fuel Line

 

Usable

 

Capacity

 

Steering wheel and column adjustments are made so that the steering wheel hub is at the geometric center of the locus it describes when moved through its full range of motion. An aluminum plate is placed across the rim of the steering wheel, an inclinometer is placed on the plate and the angle is measured. 
	STEERING COLUMN POSITION

	
	Fore/Aft Position (mm)
	Tilt (degrees)
	Tilt (detent)

	Lowermost Position No. 1
	N/A
	20.5
	N/A

	Geometric Center Position No. 2 *
	N/A
	22.0
	N/A

	Uppermost Position No. 3
	N/A
	23.5
	N/A


* Test position

DATA SHEET NO. 4
MOVING DEFORMABLE BARRIER (MDB) SUMMARY OF RESULTS

	Test Vehicle:
	2008 Subaru Impreza
	NHTSA No.
	M85501

	Test Program:
	NCAP Side Impact
	Test Date:
	September 13, 2007


	MDB SPECIFICATIONS

	Measurement Description
	Length (mm)

	Overall Width of Framework Carriage
	1250

	Overall Length Including Honeycomb Face
	4120

	Wheel base of Framework Carriage
	2590

	Tread of Framework Carriage (front & rear)
	1875

	C.G. Location aft of Front Axle
	1104


	MDB WEIGHTS

	
	Units
	Front Axle
	Rear Axle
	Total

	Left
	kg
	409.5
	281.5
	

	Right
	kg
	372.5
	299.0
	

	Ratio
	%
	57.4
	42.6
	

	Totals
	kg
	782.0
	580.5
	1362.5


	MDB SPEED AND IMPACT ANGLE DATA

	Measured Parameter
	Units
	Requirement
	Value

	Trap No. 1 Velocity (Primary)
	km/h
	61.1 to 62.7
	62.6

	Trap No. 2 Velocity (Redundant)
	km/h
	61.1 to 62.7
	62.6


POST TEST OBSERVATIONS

	MDB LEFT EDGE IMPACT POINT DATA

	Measured Parameter
	Units
	Requirement
	Value

	Horizontal Offset
	mm
	+/- 50
	0 mm

	Vertical Offset
	mm
	+/-20
	9 mm above


DATA SHEET NO. 5

POST TEST OBSERVATIONS
	Test Vehicle:
	2008 Subaru Impreza
	NHTSA No.
	M85501

	Test Program:
	NCAP Side Impact
	Test Date:
	September 13, 2007


	TEST DUMMY INFORMATION AND CONTACT POINTS

	Description
	Front Seat SID/HIII
	Rear Seat SID/HIII

	Dummy Type / Serial No.
	SID/HIII / 269
	SID/HIII / 270

	Head Contact
	Side of Head with Side Curtain airbag
	Side of Head with Side Curtain airbag

	Upper Torso Contact
	Side Torso Airbag
	Interior Door

	Lower Torso Contact
	Side Torso Airbag/Door Trim
	Interior Door

	Left Knee Contact
	Door Trim
	Door Trim

	Right Knee Contact
	Dashpanel
	No Contact


	POST TEST DOOR OPENING AND SEAT TRACK INFORMATION

	Description
	Front
	Rear

	Locked/Unlocked Doors
	Doors were unlocked
	Doors were unlocked

	Left Side Door Opening
	Door remained closed and latched
	Door remained closed and latched

	Right Side Door Opening
	Door remained closed and latched; Door opened without tools
	Door remained closed and latched; Door opened without tools

	Seat Movement
	0
	0

	Seat Back Failure
	None
	None


	POST TEST STRUCTURAL OBSERVATIONS

	Critical Areas of Performance
	Observations and Conclusions

	Pillar Performance
	No Separation

	Sill Separation
	None

	Windshield Damage
	None

	Window Damage
	Left Front and Left Rear Passenger Window Broke

	Other Notable Effects
	None


	SUPPLEMENTAL RESTRAINT INFORMATION

	Restraint Type
	Left Front (Driver)
	Left Rear (Passenger)

	
	Installed
	Deployed
	Installed
	Deployed

	Front Airbag
	Yes
	No
	NA
	NA

	Side Torso Airbag
	Yes
	Yes
	NA
	NA

	Side Head/Torso Combination Airbag
	NA
	NA
	NA
	NA

	Curtain Airbag
	Yes
	Yes
	Yes
	Yes


DATA SHEET NO. 6

VEHICLE PRE-TEST AND POST-TEST MEASUREMENTS

	Test Vehicle:
	2008 Subaru Impreza
	NHTSA No.
	M85501

	Test Program:
	NCAP Side Impact
	Test Date:
	September 13, 2007


[image: image73.wmf]LEFT SIDE VIEW

STEERING COLUMN ASSEMBLY

1

2

3


All Measurements in mm

	Code
	Measurement Description
	Pre-Test

(delivered)
	Pre-Test

(as tested)
	Post-Test

(as tested)
	Difference

	A
	Wheelbase
	2627
	2627
	2599
	28

	B
	Front Axle to FSOV
	954
	954
	983
	-29

	C
	Rear Axle to RSOV
	997
	997
	977
	20

	D
	Total Length at Centerline
	4578
	4578
	4559
	19

	E
	Front Bumper Thickness
	144
	144
	144
	0

	F
	Front Bumper Bottom to Ground
	385
	377
	398
	-21

	G
	Sill Height at Front Wheel Well
	157
	134
	138
	-4

	H
	Sill Height at Front Door Leading Edge
	188
	165
	192
	-27

	I
	Sill Height at "B" Pillar
	193
	163
	207
	-44

	J1
	Sill Height at Rear Wheel Well
	178
	141
	154
	-13

	J2
	Pinch Weld Height at Rear Wheel Well
	191
	154
	181
	-27

	K
	Sill Height Aft of Rear Wheel Well
	188
	145
	152
	-7

	L
	Rear Bumper Thickness
	220
	220
	220
	0

	M
	Rear Bumper Bottom to Ground
	432
	394
	393
	1

	N
	Sill Height to Window Bottom Sill
	720
	720
	675
	45

	O
	Front Door Leading Edge to Impact CL
	733
	733
	698
	35

	P
	Rear Door Trailing Edge to Impact CL
	1139
	1139
	1057
	82

	Q
	Front Window Opening
	423
	423
	402
	21

	R
	Right Side Length
	4477
	4477
	4469
	8

	S
	Left Side Length
	4461
	4461
	4453
	8

	T
	Vehicle Width at "B" Post
	1740
	1740
	1488
	252


DATA SHEET NO. 7

SID/HIII LONGITUDINAL CLEARANCE DIMENSIONS

	Test Vehicle:
	2008 Subaru Impreza
	NHTSA No.
	M85501

	Test Program:
	NCAP Side Impact
	Test Date:
	September 13, 2007



[image: image2.emf] 

CD  

KD  

CS  

NR  

  HH  

 HW  

 PA   

PHX  

HZ   HZ  

KB  

 PA   

PHX  

PHZ  

PHZ  

KDA   

CB  

NB  


	Driver Code
	Pass. Code
	Measurement Description
	Driver S/N 269
	Passenger S/N 270

	
	
	
	Length(mm)
	Angle(°)
	Length(mm)
	Angle(°)

	HH
	
	Head to Header
	352
	
	
	

	HW
	
	Head to Windshield
	595
	
	
	

	HZ
	HZ
	Head to Roof
	210
	
	151
	

	NR
	NB
	Nose to Rim/Nose to Seatback
	455
	
	649
	

	CD
	CB
	Chest to Dash or Seatback
	572
	
	545
	

	CS
	
	Chest to Steering Wheel
	362
	
	
	

	KDL
	KBL
	Left Knee to Dash or Seatback
	163
	22
	190
	32

	KDR
	KBR
	Right Knee to Dash or Seatback
	153
	20
	180
	32

	PA
	PA
	Pelvic Angle
	
	23.6
	
	24.4

	PHX
	PHX
	H-Point to Striker (X-Axis)
	186
	
	223
	

	PHZ
	PHZ
	H-Point to Striker (Z-Axis)
	173
	
	267
	


DATA SHEET NO. 8

SID/HIII LATERAL CLEARANCE DIMENSIONS

	Test Vehicle:
	2008 Subaru Impreza
	NHTSA No.
	M85501

	Test Program:
	NCAP Side Impact
	Test Date:
	September 13, 2007



[image: image3.emf]   

FRONT VIEW OF DUMMY  

HR  

HS  

HD  

AD  


	Code
	Measurement Description
	Units
	Driver

S/N 269
	Passenger

S/N 270

	HR
	Head to Side Header
	mm
	207
	183

	HS
	Head to Side Window
	mm
	312
	354

	AD1
	Arm to Door (at upper rib level)
	mm
	109
	100

	AD2
	Arm to Door (at lower rib level)
	mm
	126
	99

	HD
	H-Point to Door
	mm
	140
	182


DATA SHEET NO. 9

VEHICLE SIDE MEASUREMENTS
	Test Vehicle:
	2008 Subaru Impreza
	NHTSA No.
	M85501

	Test Program:
	NCAP Side Impact
	Test Date:
	September 13, 2007


[image: image74.wmf] 

A

 

F

 

E

 

B

 

C

 

D

 

G

 

H

 

I

 

K

 

J

 

M

 

L

 

N

 

Q

 

P

 

O

 

LEFT SIDE VIEW

 


Measurements are taken with vehicle in the as tested condition.




Measurements along the vertical 750 mm.




All measurements below in mm.

	Level
	Measurement Description
	Maximum Exterior Static Crush
	Height Above Ground
	Distance From Impact

	1
	Sill Top
	104
	235
	1350

	2
	Occupant H-Point
	241
	431
	750

	3
	Mid Door
	277
	604
	750

	4
	Window Sill
	260
	854
	1500

	5
	Window
	5
	1376
	1800

	
	Maximum Penetration
	277
	
	


DATA SHEET NO. 10

VEHICLE EXTERIOR CRUSH PROFILES

	Test Vehicle:
	2008 Subaru Impreza
	NHTSA No.
	M85501

	Test Program:
	NCAP Side Impact
	Test Date:
	September 13, 2007


	[image: image4.emf]-300

-200

-100

0

100

200

300

400

500

600

700

800

900

-900 -750 -600 -450 -300 -150 0 150 300 450 600 750 900 1050 1200 1350 1500 1650 1800 1950 2100 2250 2400 2550 2700 2850 3000

Distance from impact point (mm)

Side profile (1000 mm from centerline)

LEVEL 1 Side Sill:   235 mm above ground

LEVEL 2 H-Point:  431 mm above ground

LEVEL 3 Mid Door:  604 mm above ground

LEVEL 4 Window Sill:  854 mm above ground

LEVEL 5 Window Top:  1376 mm above ground


[image: image5.emf]NOTE: All dimensions are in millimeters with a tolerance of ±3 mm

DISTANCE IN MILLIMETERS (mm) FROM IMPACT POINT

LEVEL

HEIGHT 

(mm)

-900 -750 -600 -450 -300 -150 0 150 300 450 600 750 900 1050 1200 1350 1500 1650 1800 1950 2100 2250 2400 2550 2700 2850 3000

LEVEL 1

PRE

   --    --    --    --    --    --    -- 164 169 169 170 171 172 172 173 174 175 175 167    --    --    --    --    --    --    --    --

SIDE

235

POST

   --    --    --    --    --    --    -- 237 239 240 244 251 257 275 266 278 271 250 215    --    --    --    --    --    --    --    --

SILL

CRUSH

  N/A   N/A   N/A   N/A   N/A   N/A   N/A 73 70 71 74 80 85 103 93 104 96 75 48   N/A   N/A   N/A   N/A   N/A   N/A   N/A   N/A

LEVEL 2

PRE

192 144    --    --    --    -- 121 132 133 133 134 135 136 137 140 142 143 146 144    --    --    --    --    -- 165 179 201

H

431

POST

185 141    --    --    --    -- 151 301 352 365 366 376 372 372 373 383 380 370 322    --    --    --    --    -- 170 186 210

POINT

CRUSH

-7 -3   N/A   N/A   N/A   N/A 30 169 219 232 232 241 236 235 233 241 237 224 178   N/A   N/A   N/A   N/A   N/A 5 7 9

LEVEL 3

PRE

227 172 129    --    -- 119 126 132 132 131 132 132 133 135 138 139 141 143 144 141    --    --    -- 156 173 187 211

MID

604

POST

215 164 127    --    -- 139 152 294 364 389 397 409 381 365 369 382 402 403 389 258    --    --    -- 157 176 190 216

DOOR

CRUSH

-12 -8 -2   N/A   N/A 20 26 162 232 258 265 277 248 230 231 243 261 260 245 117   N/A   N/A   N/A 1 3 3 5

LEVEL 4

PRE

   -- 412 316 255 227 213 204 195 189 182 176 174 172 169 169 168 170 174 180 167 195 208 212 217 221 231 250

WINDOW

854

POST

   -- 396 304 248 226 217 212 226 286 334 384 385 384 376 351 402 430 428 367 251 176 200 207 212 216 226 249

SILL

CRUSH

  N/A -16 -12 -7 -1 4 8 31 97 152 208 211 212 207 182 234 260 254 187 84 -19 -8 -5 -5 -5 -5 -1

LEVEL 5

PRE

   --    --    --    --    --    --    --    --    --    --    --    -- 511 440 438 440 444 451 463 494 710    --    --    --    --    --    --

WINDOW

1376

POST

   --    --    --    --    --    --    --    --    --    --    --    -- 496 426 430 436 445 455 468 490 685    --    --    --    --    --    --

TOP

CRUSH

  N/A   N/A   N/A   N/A   N/A   N/A   N/A   N/A   N/A   N/A   N/A   N/A -15 -14 -8 -4 1 4 5 -4 -25   N/A   N/A   N/A   N/A   N/A   N/A




DATA SHEET NO. 11

VEHICLE DAMAGE PROFILE DISTANCES

	Test Vehicle:
	2008 Subaru Impreza
	NHTSA No.
	M85501

	Test Program:
	NCAP Side Impact
	Test Date:
	September 13, 2007


[image: image75.wmf] 

LEFT SIDE VIEW

 

                

-

1200 

–

1050 

–

900 

–

750 

–

600 

–

450  

–

300  

–

150     0     150   300   450    600   750   900  1050 1200 1350 1500  1650 1800 1950 2100  2250 2400  2550 2600 27

50  2900 3050 

 

Level 5

 

Level 3

 

Level 4

 

Level 2

 

Level 1

 

All Measurements Shown in mm

 


	DAMAGE PROFILE DISTANCES

	DPD
	Distance from Impact Point in mm
	Level
	Pre-Test (mm)
	Post-Test (mm)
	Max Static Crush (mm)

	1 (LR)
	2850
	431
	179
	182
	3

	2
	2220
	854
	148
	172
	24

	3
	1590
	604
	127
	377
	250

	4
	960
	604
	123
	373
	250

	5
	330
	431
	128
	333
	205

	6 (LF)
	-300
	854
	193
	189
	-4


Reference plane is parallel to test vehicle longitudinal centerline.

Given dimensions = Reference plane to vehicle body.

DATA SHEET NO. 12

DEFORMABLE BARRIER HONEYCOMB FACE STATIC CRUSH

	Test Vehicle:
	2008 Subaru Impreza
	NHTSA No.
	M85501

	Test Program:
	NCAP Side Impact
	Test Date:
	September 13, 2007


[image: image76.wmf] 

 

-

1200

 

 

-

900

 

 

–

600

 

 

–

300

 

 

  0

 

 

 

300

 

 

600

 

 

900

 

  1200

 

 1500

 

  1800

 

  2100

 

  2400

 

  2600

 

 2900

 

  3050

 

0

 

300

 

600

 

1500

 

900

 

1200

 

1500

 

Test Vehicle Longitudinal Centerline

 

Rearwardmost Point 

of Induced Damage

 

Forwardmos

t Point 

 

Of Induced Damage

 

Reference Plane

 

TOP VIEW

 

All Dimensions Shown in millimeters

 


	[image: image6.wmf]DISTANCE RIGHT OF CENTER (mm)

DISTANCE LEFT OF CENTER (mm)

LEVEL

HEIGHT 

AT CL 

(mm)*

800

700

600

500

400

300

200

100

0

100

200

300

400

500

600

700

800

LEVEL

HEIGHT 

AT CL 

(mm)*

MAX 

CRUSH

LEVEL 4

PRE

619

619

619

619

619

619

619

619

619

619

619

619

619

619

619

619

619

LEVEL 4

TOP

813

POST

662

621

609

616

631

664

671

654

633

633

636

638

638

663

698

729

783

TOP

813

164

STACK

CRUSH

43

2

-10

-3

12

45

52

35

14

14

17

19

19

44

79

110

164

STACK

LEVEL 3

PRE

619

619

619

619

619

619

619

619

619

619

619

619

619

619

619

619

619

LEVEL 3

MID

686

POST

651

628

620

626

626

638

654

643

625

623

623

623

627

635

648

683

748

MID

686

129

LEVEL

CRUSH

32

9

1

7

7

19

35

24

6

4

4

4

8

16

29

64

129

LEVEL

LEVEL 2

PRE

619

619

619

619

619

619

619

619

619

619

619

619

619

619

619

619

619

LEVEL 2

TOP

533

POST

691

688

681

677

679

680

686

683

679

675

675

678

678

685

691

704

721

TOP

533

102

BUMPER

CRUSH

72

69

62

58

60

61

67

64

60

56

56

59

59

66

72

85

102

BUMPER

LEVEL 1

PRE

535

519

518

518

518

518

518

518

518

518

518

518

518

518

518

519

535

LEVEL 1

MID

432

POST

667

656

652

651

657

671

667

659

648

640

628

629

627

630

637

654

687

MID

432

153

BUMPER

CRUSH

132

137

134

133

139

153

149

141

130

122

110

111

109

112

119

135

152

BUMPER




DATA SHEET NO. 13

VEHICLE ACCELEROMETER LOCATIONS

	Test Vehicle:
	2008 Subaru Impreza
	NHTSA No.
	M85501

	Test Program:
	NCAP Side Impact
	Test Date:
	September 13, 2007


[image: image77.wmf] 

Level 1

 

432 mm

 

Level 3

 

686 mm

 

Level 4

 

813 mm

 

Mounting

 

Block

 

MDB Frame

 

Level 1 

–

 Bumper Centerline

 

Level 2 

–

 Top of Bumper

 

Level 3 

–

 Middle Stack Level

 

Level 4 

–

 Top of Stack

 

Grou

nd Surface

 

Right

 

Left

 

Level 2

 

533 mm

 


	Loc. No.
	 Accelerometer Location
	Measurements (mm)

	
	
	X
	Y
	Z

	1
	 Right Sill at Front Seat
	2769
	629
	-249

	2
	 Right Sill at Rear Seat
	1926
	602
	-215

	3
	 Rear Floorpan Above Axle
	1083
	-10
	-570

	4
	 Left Sill at Rear Door
	2019
	-598
	-209

	5
	 Left Sill at Front Door
	2879
	-629
	-250

	6
	 Left Front Door C/L**
	-
	-
	-

	7
	 Rear Occupant Compartment
	1941
	430
	-203

	8
	 Left Front Door Mid-Rear**
	-
	-
	-

	9
	 Left Front Door Upper C/L**
	-
	-
	-

	10
	 Left Rear Door Mid-Rear**
	-
	-
	-

	11
	 Left Rear Door Upper C/L**
	-
	-
	-

	12
	 Left Lower B-Post
	2060
	-665
	-400

	13
	 Left Middle B-Post
	1992
	-649
	-909

	14
	 Left Lower A-Post
	3087
	-651
	-366

	15
	 Left Middle A-Post 
	2988
	-633
	-978

	16
	 Front Seat Track
	2210
	-548
	-264

	17
	 Rear Seat Track or Structure
	1756
	-590
	-341

	18
	 Vehicle CG
	2408
	8
	-420


Reference Points
X - Test Vehicle Rear Bumper (+ forward)   




Y - Test Vehicle Centerline (+ to right)




Z - Ground Plane (+ down)

** Accelerometer was not requested by the COTR.

DATA SHEET NO. 14

MDB ACCELEROMETER LOCATIONS

	Test Vehicle:
	2008 Subaru Impreza
	NHTSA No.
	M85501

	Test Program:
	NCAP Side Impact
	Test Date:
	September 13, 2007



[image: image7.emf] 

X  

Y  

Z  

1  

2  

 

 


	Loc. No.
	 Accelerometer Location
	Measurements (mm)

	
	
	X
	Y
	Z

	1
	 MDB CG
	1859
	0
	-330

	2
	 MDB Rear
	386
	-660
	-660


Reference Points
X - MDB Rear Bumper (+ forward)


Y - MDB Centerline (+ to right)


Z - Ground Plane (+ down)

DATA SHEET NO. 15

VEHICLE STRUCTURAL MEASUREMENTS
	Test Vehicle:
	2008 Subaru Impreza
	NHTSA No.
	M85501

	Test Program:
	NCAP Side Impact
	Test Date:
	September 13, 2007


	
	Elements
	Pre-Test (mm)

	1
	Total Length
	4578

	2
	Total Width
	1740

	3
	Bumper Top Height
	515

	4
	Bumper Bottom Height
	328

	5
	Longitudinal Member Top Height
	491

	6
	Distance between Longitudinal Members
	988

	7
	Longitudinal Member Width
	64

	8
	Engine Top Height
	816

	9
	Engine Bottom Height
	234

	10
	Engine and gearbox width
	732

	11
	Front bumper-engine distance
	442

	12
	Front shock absorber fixing height
	829

	13
	Bonnet leading edge height
	694

	14
	Front shock absorber fixing width
	1127

	15
	Front bumper – front axle distance
	954

	16
	Front axle – a pillar distance
	1153

	17
	A-pillar – B-pillar distance
	402

	18
	B-Pillar – rear axle distance
	1072

	19
	B-pillar – C-pillar distance
	979

	20
	Roof sill bottom height
	1365

	21
	Roof sill top height
	1416

	22
	Floor sill bottom height
	177

	23
	Floor sill top height
	261


DATA SHEET NO. 16

HIGH SPEED CAMERA LOCATIONS AND DATA

	Test Vehicle:
	2008 Subaru Impreza
	NHTSA No.
	M85501

	Test Program:
	NCAP Side Impact
	Test Date:
	September 13, 2007



[image: image8.emf] 

X  

Y  

27 o  

5  

9  

8  

1  

2  

7  

6  

3  

4  

10  

63 o  

11  

IMPACT   POINT  


	No.
	Camera View
	Location (mm)
	Angle (deg)
	Lens (mm)
	Film Speed (fps)

	
	
	X
	Y
	Z
	
	
	

	1
	Overhead Overall 
	72
	812
	-4880
	-90
	8
	500

	2
	Overhead Close-up
	195
	855
	-4880
	-90
	28
	1000

	3
	MDB Onboard, Impact Point Close-up
	-1470
	0
	-847
	0
	13
	500

	4
	MDB Onboard, Centerline of Impact
	-1140
	838
	-1587
	-17
	7.5
	500

	5
	Right Side, Ground Level, Overall
	0
	10097
	1038
	-3
	50
	1000

	6
	Left Side, Ground Level, Overall
	-1955
	-1721
	971
	-2
	28
	1000

	7
	Vehicle Onboard Front SID/HIII, Front
	552
	-419
	1244
	-10
	25
	1000

	8
	Vehicle Onboard Front SID/HIII, Side
	1540
	824
	1022
	-8
	13
	1000

	9
	Vehicle Onboard Rear SID/HIII, Side
	1530
	1710
	1059
	-9
	12.5
	1000

	10
	Secondary Impact Point
	-4980
	2824
	1026
	-2
	50
	500

	11
	Real Time Coverage
	
	
	
	
	
	30


Reference Points
X - Impact Line (Positive – Impact Point Forward)



Y - MDB Left Edge Impact Point (Positive – To Right)



Z - Ground Plane (Positive – Down)
DATA SHEET NO. 17

SUMMARY OF FMVSS 301 DATA

	Test Vehicle:
	2008 Subaru Impreza
	NHTSA No.
	M85501

	Test Program:
	NCAP Side Impact
	Test Date:
	September 13, 2007


	FUEL SYSTEM INTEGRITY POST IMPACT DATA

	Time Interval
	FMVSS 301 Maximum Allowable Spillage
	Spillage (g)

	Impact Until Motion Ceases
	28 g
	0

	First Five Minutes Following Impact
	142 g
	0

	Next 25 Minutes
	28 g / 1 minute
	0


	Spillage Location(s)
	None


STATIC ROLLOVER DATA

	[image: image9.wmf]
	[image: image10.wmf]
	[image: image11.wmf]
	[image: image12.wmf]


	Rollover Stage
	Rotation Time

(spec. 1 -3 min)
	FMVSS 301

Hold Time
	Total Time
	Next Whole Minute Interval

	0º - 90º
	1
	minutes
	13
	seconds
	5
	minutes
	6
	minutes
	13
	seconds
	7
	minutes

	90º - 180º
	1
	minutes
	01
	seconds
	5
	minutes
	6
	minutes
	1
	seconds
	7
	minutes

	180º-270º
	1
	minutes
	01
	seconds
	5
	minutes
	6
	minutes
	1
	seconds
	7
	minutes

	270º-360º
	1
	minutes
	12
	seconds
	5
	minutes
	6
	minutes
	12
	seconds
	7
	minutes


	Rollover Stage
	Spillage (g)

	
	First 5 min. from onset of rotation
	6th min.
	7th min.
	8th min.

(if required)

	0º - 90º
	0
	0
	0
	0

	90º - 180º
	0
	0
	0
	0

	180º-270º
	0
	0
	0
	0

	270º-360º
	0
	0
	0
	0

	FMVSS 301 Maximum Allowable

(for each 90º stage)
	142
	28
	28
	28


	Rollover Stage
	Spillage Location(s)

	0º - 90º
	None

	90º - 180º
	None

	180º-270º
	None

	270º-360º
	None


APPENDIX A

PHOTOGRAPHS

TABLE OF PHOTOGRAPHS
	FIGURE
	TITLE
	PAGE

	A-1
	As Received Left Front ¾ View
	A-4

	A-2
	As Received Right Rear ¾ View
	A-4

	A-3
	Vehicle Certification Label
	A-5

	A-4
	Vehicle Tire Placard Label
	A-5

	A-5
	Pre-Test Front View
	A-6

	A-6
	Post-Test Front View
	A-6

	A-7
	Pre-Test Left Front ¾ View
	A-7

	A-8
	Post-Test Left Front ¾ View
	A-7

	A-9
	Pre-Test Left Side View
	A-8

	A-10
	Post-Test Left Side View
	A-8

	A-11
	Pre-Test Left Rear ¾ View
	A-9

	A-12
	Post-Test Left Rear ¾ View
	A-9

	A-13
	Pre-Test Rear View
	A-10

	A-14
	Post-Test Rear View
	A-10

	A-15
	Pre-Test Right Rear ¾ View
	A-11

	A-16
	Post-Test Right Rear ¾ View
	A-11

	A-17
	Pre-Test Right Side View
	A-12

	A-18
	Post-Test Right Side View
	A-12

	A-19
	Pre-Test Right Front ¾ View
	A-13

	A-20
	Post-Test Right Front ¾ View
	A-13

	A-21
	Pre-Test Frontal View of MDB Impactor Face
	A-14

	A-22
	Post-Test Frontal View of MDB Impactor Face
	A-14

	A-23
	Pre-Test Left Side View of MDB Impactor Face
	A-15

	A-24
	Post-Test Left Side View of MDB Impactor Face
	A-15

	A-25
	Pre-Test Right Side View of MDB Impactor Face
	A-16

	A-26
	Post-Test Right Side View of MDB Impactor Face
	A-16

	A-27
	Pre-Test Top View of MDB Impactor Face
	A-17

	A-28
	Post-Test Top View of MDB Impactor Face
	A-17

	A-29
	Pre-Test Left Side View of Aligned MDB and Vehicle
	A-18

	A-30
	Pre-Test Right Side View of Aligned MDB and Vehicle
	A-18

	A-31
	Pre-Test Overhead View of Aligned MDB and Vehicle
	A-19

	A-32
	Post-Test Overhead View of MDB and Vehicle
	A-19

	A-33
	Pre-Test Close-Up View of Impact Point Target
	A-20

	A-34
	Post-Test Close-Up View of Impact Point Target
	A-20


TABLE OF PHOTOGRAPHS (continued)
	FIGURE
	TITLE
	PAGE

	A-35
	Pre-Test Right Occupant Compartment View of Driver
	A-21

	A-36
	Post-Test Right Occupant Compartment View of Driver
	A-21

	A-37
	Pre-Test Right Occupant Compartment View of Passenger
	A-22

	A-38
	Post-Test Right Occupant Compartment View of Passenger
	A-22

	A-39
	Pre-Test Left Occupant Compartment View of Driver
	A-23

	A-40
	Post-Test Left Occupant Compartment View of Driver
	A-23

	A-41
	Pre-Test Left Occupant Compartment View of Passenger
	A-24

	A-42
	Post-Test Left Occupant Compartment View of Passenger
	A-24

	A-43
	Pre-Test Left Front Interior Trim
	A-25

	A-44
	Post-Test Left Front Interior Trim
	A-25

	A-45
	Pre-Test Left Rear Interior Trim
	A-26

	A-46
	Post-Test Left Rear Interior Trim
	A-26

	A-47
	Pre-Test Left Front ¾ View of Left Side Doors
	A-27

	A-48
	Post-Test Left Front ¾ View of Left Side Doors
	A-27

	A-49
	Pre-Test Left Rear ¾ View of Left Side Doors
	A-28

	A-50
	Post-Test Left Rear ¾ View of Left Side Doors
	A-28

	A-51
	Rollover 90 Degrees
	A-29

	A-52
	Rollover 180 Degrees
	A-29

	A-53
	Rollover 270 Degrees
	A-30

	A-54
	Rollover 360 Degrees
	A-30

	A-55
	Impact Photo
	A-31


	[image: image13.jpg]




	Figure A-1: As Received Left Front ¾ View

	[image: image14.jpg]




	Figure A-2: As Received Right Rear ¾ View


	[image: image15.jpg]I mc FOR 10 ICABLE FEDERAL WOTOR |
'Eﬁ FE??“ ER, A@l‘)’ THEFT PREIVFH'NION %TAmAnns; o

ECT ON THE DATE OF MANUFACTUR

1 I\mlllllH||IIIIIIIIIlIIHIlIlIIllIlIlIl






	Figure A-3: Vehicle Certification Label

	[image: image16.jpg]




	Figure A-4: Vehicle Tire Placard Label


	[image: image17.jpg]




	Figure A-5: Pre-Test Front View

	[image: image18.jpg]Wl C42144-4009-2308
" 55/28 kph 90° IMPAC
2008 SUBARU IMPREZ
NHTSA M85501
09/13/2007






	Figure A-6: Post-Test Front View


	[image: image19.jpg]|
1] , & o
guas?? ‘fmnumumm. #






	Figure A-7: Pre-Test Left Front ¾ View

	[image: image20.jpg]Transpm,tatmn
Safetv &
7/

ecurity






	Figure A-8: Post-Test Left Front ¾ View


	[image: image21.jpg]P~ -
-
-
=]
E— 1|
g ; ‘\ d\ 2
e = - w—

.‘Q_,,.‘t,..‘nm‘\ ~
[

ht..l‘ll-ll L ssmsszsa:m 3
. 77 sEEEEEEEE
s PL %






	Figure A-9: Pre-Test Left Side View

	[image: image22.jpg]




	Figure A-10: Post-Test Left Side View


	[image: image23.jpg]NHTSA M50t
0913200 B






	Figure A-11: Pre-Test Left Rear ¾ View

	[image: image24.jpg].
NINNEREANENE N -
i [l 1}

'\ ! i L]

inEERNEm g gy

Yl

"
' g

P

(e
S
¢
.





	Figure A-12: Post-Test Left Rear ¾ View


	[image: image25.jpg]C42144-4009-2308
55/28 kph 90° IMPACT
2008 SUBARU IMPREZA

NHTSA M85501
09/13/2007






	Figure A-13: Pre-Test Rear View

	[image: image26.jpg]C42144-4009-2308

55/28 kph 90" IMPACT W

2008 SUBARU IMPREZA
NHTSA M85501
09/13/2007






	Figure A-14: Post-Test Rear View


	[image: image27.jpg]N





	Figure A-15: Pre-Test Right Rear ¾ View

	[image: image28.jpg]' RRRTRL “\4 -

AED, &






	Figure A-16: Post-Test Right Rear ¾ View


	[image: image29.jpg]$5/28 kph 90" IMPACT

2008 SUBARU IMPREZA
NHTSA MB5501






	Figure A-17: Pre-Test Right Side View

	[image: image30.jpg]Ca21444000.2305 R

 55/28 kph 90° IMPACT
Bl 2008 SUBARU IMPREZA
NHTSA M85501
09/13/2007






	Figure A-18: Post-Test Right Side View


	[image: image31.jpg]052} W '\%& A\
200% SUBIRU RS
IO IR






	Figure A-19: Pre-Test Right Front ¾ View

	[image: image32.jpg]>
%mmww
222838
SEZ28
2228
5227
So742
FLpS
A‘D\S“
222
S8

A < 03






	Figure A-20: Post-Test Right Front ¾ View


	[image: image33.jpg]s s






	Figure A-21: Pre-Test Frontal View of MDB Impactor Face

	[image: image34.jpg]




	Figure A-22: Post-Test Frontal View of MDB Impactor Face


	[image: image35.jpg]‘M85501 = 1=

| e
ESSPEERE 1

D S 4 & — ”





	Figure A-23:Pre-Test Left Side View of MDB Impactor Face

	[image: image36.jpg]




	Figure A-24: Post-Test Left Side View of MDB Impactor Face


	[image: image37.jpg]B )
B






	Figure A-25: Pre-Test Right Side View of MDB Impactor Face

	[image: image38.jpg]




	Figure A-26: Post-Test Right Side View of MDB Impactor Face


	[image: image39.jpg]C42144-4009-2308

‘ ! 55/28 kph 90° IMPACT
i 2008 SUBARU IMPREZA
| ‘ NHTSA M85501

! = 09/13/2007

e B

—

' s -
|...----~--.....—.,.
& Wl

v ¥ -

ip— " w6 y .0
=] - 7"
P

e






	Figure A-27: Pre-Test Top View of MDB Impactor Face

	[image: image40.jpg]C42144- 0050
55128 kph 90° IMPA
2008 SUBARU IMPREZA

S NHTSA mgs55(
M85501 ¥, | 09/13/2007

)

cTH

<






	Figure A-28: Post-Test Top View of MDB Impactor Face


	[image: image41.jpg]‘ —

/AL B | -
\f \
L am, ‘
-a ““‘
A %Y

fffff
EE&(

et -\%“E
3 +






	Figure A-29: Pre-Test Left Side View of Aligned MDB and Vehicle

	[image: image42.jpg]




	Figure A-30: Pre-Test Right Side View of Aligned MDB and Vehicle


	[image: image43.jpg]




	Figure A-31: Pre-Test Overhead View of Aligned MDB and Vehicle

	[image: image44.jpg]ph 90° IMPAS

RTINS






	Figure A-32: Post-Test Overhead View of MDB and Vehicle


	[image: image45.jpg]




	Figure A-33: Pre-Test Close-Up View of Impact Point Target

	[image: image46.jpg]




	Figure A-34: Post-Test Close-Up View of Impact Point Target


	[image: image47.jpg]




	Figure A-35: Pre-Test Right Occupant Compartment View of Driver

	[image: image48.jpg]




	Figure A-36: Post-Test Right Occupant Compartment View of Driver


	[image: image49.jpg]




	Figure A-37: Pre-Test Right Occupant Compartment View of Passenger

	[image: image50.jpg]




	Figure A-38: Post-Test Right Occupant Compartment View of Passenger


	[image: image51.jpg]




	Figure A-39: Pre-Test Left Occupant Compartment View of Driver

	[image: image52.jpg]




	Figure A-40: Post-Test Left Occupant Compartment View of Driver


	[image: image53.jpg]




	Figure A-41: Pre-Test Left Occupant Compartment View of Passenger

	[image: image54.jpg]




	Figure A-42: Post-Test Left Occupant Compartment View of Passenger


	[image: image55.jpg]© M85501






	Figure A-43: Pre-Test Left Front Interior Trim

	[image: image56.jpg]




	Figure A-44: Post-Test Left Front Interior Trim


	[image: image57.jpg]M85501

B






	Figure A-45: Pre-Test Left Rear Interior Trim

	[image: image58.jpg]




	Figure A-46: Post-Test Left Rear Interior Trim


	[image: image59.jpg]= 2 -
- ;; a =

»""—"""-‘--

? Y i ~Saaa
. S
8

/ | | /-é W






	Figure A-47: Pre-Test Left Front ¾ View of Left Side Doors

	[image: image60.jpg]




	Figure A-48: Post-Test Left Front ¾ View of Left Side Doors


	[image: image61.jpg]




	Figure A-49: Pre-Test Left Rear ¾ View of Left Side Doors

	[image: image62.jpg]~
\

’ Py’
773
',"’I 2

m@ ,.’
[ |
rll
L]
G w
.
\ X
\, am
| |
]
\

)
| st






	Figure A-50: Post-Test Left Rear ¾ View of Left Side Doors


	[image: image63.jpg]L00ZIEVIB0
10658 VSLHN
yZ3UdW nyvans 800z
406 udX 82/5S

S&
ZSEQ
Tl
ErEa s
ESZ1 S .
o T (
SEce8 ¢
Saz=zs?
SAIE0
2238
Eow AW ¢
— IS
Ma5201] =

Ay
\






	Figure A-51: Rollover 90 Degrees

	[image: image64.jpg]e —

.-l"ll...‘..






	Figure A-52: Rollover 180 Degrees


	[image: image65.jpg]




	Figure A-53: Rollover 270 Degrees

	[image: image66.jpg]C42144-4009-2308

55128 kph 90 IMPACT
2008 SUBARU IMPREZA

NHTSA M85501
09/13/2007






	Figure A-54: Rollover 360 Degrees


	[image: image67.jpg]




	Figure A-55: Impact Photo

	

	


APPENDIX B

SID/HIII, VEHICLE AND MDB RESPONSE DATA

(SAE sign convention)

	DATA CHANNEL FILTER CLASS SUMMARY

	Data Type
	SAE Filter Class

	Dummy Head Accelerations
	CFC 1000

	Rib Accelerations
	FIR 100

	Spine Accelerations
	FIR 100

	Pelvis Accelerations
	FIR 100


	DATA CHANNEL TITLE KEY

	Prefix
	Suffix

	V1 = Vehicle 1 (Moving Barrier)
	Ax = Acceleration, X-direction

	V2 = Vehicle 2 (Test Vehicle)
	Ay = Acceleration, Y-direction

	P1 = Left Front Seating Position (Driver)
	Az = Acceleration, Z-direction

	P4 = Left Second Row Seating Position (Passenger)
	Fx = Force, X-direction

	A1-A18 = Accelerometer Location Number
	Fy = Force, Y-direction

	
	Fz = Force, Z-direction

	
	Mx = Moment about X

	
	My = Moment about Y

	
	Mz = Moment about Z


TABLE OF DATA PLOTS
	PLOT
	PLOT  NAME[UNITS, CHANNEL FILTER CLASS]
	PAGE

	1
	V2P1 Head Ax [g, CFC_1000]
	B-5

	2
	V2P1 Head Ay [g, CFC_1000]
	B-5

	3
	V2P1 Head Az [g, CFC_1000]
	B-5

	4
	V2P1 Head Ar [g, CFC_1000]
	B-5

	5
	V1P1 Upper Rib Ay [g, FIR_100]
	B-6

	6
	V1P1 Lower Rib Ay [g, FIR_100]
	B-6

	7
	V1P1 Lower Spine Ay [g, FIR_100]
	B-6

	8
	V1P1 Pelvic Ay [g, FIR_100]
	B-6

	9
	V2P4 Head Ax [g, CFC_1000]
	B-7

	10
	V2P4 Head Ay [g, CFC_1000]
	B-7

	11
	V2P4 Head Az [g, CFC_1000]
	B-7

	12
	V2P4 Head Ar [g, CFC_1000]
	B-7

	13
	V1P4 Upper Rib Ay [g, FIR_100]
	B-8

	14
	V1P4 Lower Rib Ay [g, FIR_100]
	B-8

	15
	V1P4 Lower Spine Ay [g, FIR_100]
	B-8

	16
	V1P4 Pelvic Ay [g, FIR_100]
	B-8


The following dummy, vehicle and load cell response data can be found in the research and development section of the NHTSA website at: www.nhtsa.dot.gov
	
	V2P1 Head Ax
	
	V2A1 Right Front Sill Ax

	
	V2P1 Head Ay
	
	V2A1 Right Front Sill Ay

	
	V2P1 Head Az
	
	V2A1 Right Front Sill Az

	
	V2P1 Upper Neck Fx
	
	V2A2 Right Rear Sill Ax

	
	V2P1 Upper Neck Fy
	
	V2A2 Right Rear Sill Ay

	
	V2P1 Upper Neck Fz
	
	V2A2 Right Rear Sill Az

	
	V2P1 Upper Neck Mx
	
	V2A3 Rear Floorpan Ax

	
	V2P1 Upper Neck My
	
	V2A3 Rear Floorpan Ay

	
	V2P1 Upper Neck Mz
	
	V2A3 Rear Floorplan Az

	
	V2P1 Upper Rib Ay
	
	V2A4 Left Rear Sill Ay

	
	V2P1 Upper Rib Redundant Ay
	
	V2A5 Left Front Sill Ay

	
	V2P1 Lower Rib Ay
	
	V2A6 Left Front Door C/L Ay

	
	V2P1 Lower Rib Redundant Ay
	
	V2A7 Right Rear Compartment Ay

	
	V2P1 Lower Spine Ay
	
	V2A8 Left Front Door Midrear Ay

	
	V2P1 Lower Spine Redundant Ay
	
	V2A9 Left Front Door Upper C/L Ay

	
	V2P1 Pelvic Ay
	
	V2A10 Left Rear Door Midrear Ay

	
	V2P1 Pelvic Redundant Ay
	
	V2A11 Left Rear Door Upper C/L Ay

	
	V2P4 Head Ax
	
	V2A12 Left Lower B Post Ay

	
	V2P4 Head Ay
	
	V2A13 Left Mid B Post Ay

	
	V2P4 Head Az
	
	V2A14 Left Lower A Post Ay

	
	V2P4 Upper Neck Fx
	
	V2A15 Left Mid A Post Ay

	
	V2P4 Upper Neck Fy
	
	V2A16 Front Seat Track Ay

	
	V2P4 Upper Neck Fz
	
	V2A17 Rear Seat Track Ay

	
	V2P4 Upper Neck Mx
	
	V2A18 Target CG Ax

	
	V2P4 Upper Neck My
	
	V2A18 Target CG Ay

	
	V2P4 Upper Neck Mz
	
	V2A18 Target CG Az

	
	V2P4 Upper Rib Ay
	
	V1 Moving Barrier CG Ax

	
	V2P4 Upper Rib Redundant Ay
	
	V1 Moving Barrier CG Ay

	
	V2P4 Lower Rib Ay
	
	V1 Moving Barrier CG Az

	
	V2P4 Lower Rib Redundant Ay
	
	V1 Moving Barrier Left Rail Ax

	
	V2P4 Lower Spine Ay
	
	V1 Moving Barrier Left Rail Ay

	
	V2P4 Lower Spine Redundant Ay
	
	

	
	V2P4 Pelvic Ay
	
	

	
	V2P4 Pelvic Redundant Ay
	
	


TEST NOTES
The following data anomalies occurred: None
APPENDIX C

DUMMY CONFIGURATION AND PERFORMANCE VERIFICATION DATA
	SUMMARY

	SID H3 PRE & POST TEST CALIBRATION

	

	CONFIGURED FOR 
	LEFT
	 SIDE IMPACT

	

	

	Date:
	8/30/07, 9/25/07
	   Sequential Test Number:
	1

	
	
	   Laboratory Technician:
	 B. Swiecicki

	

	TEST PARAMETER
	SPECIFICATION
	SID H3 NO.:
	269
	SID H3 NO.:
	270

	
	
	PRE

TEST
	POST

TEST
	PRE

TEST
	POST

TEST

	SH- Seated Height (mm)
	889 - 909
	899
	899
	899
	899

	RH- Rib Height (mm)
	501 - 521
	508
	505
	505
	505

	HP- Hip Pivot Height (mm)
	99 ref.
	99
	99
	99
	99

	RD- Rib from Back Line (mm)
	229 - 241
	234
	234
	234
	234

	KH- Knee Pivot from Back Line (mm)
	511 - 526
	518
	518
	518
	518

	KV- Knee Pivot to Floor (mm)
	490 - 505
	493
	493
	495
	493

	HW- Hip Width (mm)
	356 - 391
	384
	384
	384
	386

	THORAX IMPACTS

	TEMPERATURE (C)
	18.9 - 25.5
	21.1
	21.1
	21.1
	21.1

	RELATIVE. HUMIDITY (%)
	10 -70
	34
	35
	34
	35

	PROBE SPEED (m/s)
	4.27 - 4.33
	4.27
	4.30
	4.28
	4.28

	UPPER RIB (g's)
	37 - 46
	40.0
	40.4
	39.3
	38.8

	LOWER RIB (g's)
	37 - 46
	40.5
	40.3
	40.7
	40.2

	LOWER SPINE (g's)
	15 -22
	18.1
	18.0
	21.7
	20.9

	PELVIS IMPACT

	TEMPERATURE (C)
	18.9 - 25.5
	21.1
	21.1
	21.1
	21.1

	RELATIVE HUMIDITY (%)
	10 - 70
	35
	35.0
	34
	36

	PROBE SPEED (m/s)
	4.27 - 4.33
	4.28
	4.33
	4.29
	4.29

	PELVIS (g's)
	40 -60 
	55.4
	49.5
	44.3
	43.1


REMARKS: None

	CALIBRATION TEST RESULTS

	PRE-TEST

	

	SID H3 NO.:
	269
	

	

	CONFIGURED FOR 
	LEFT
	 SIDE IMPACT


	CALIBRATION TEST RESULTS SUMMARY

	PRE-TEST

	

	CONFIGURED FOR 
	LEFT
	 SIDE IMPACT

	

	

	SID H3 Serial No.:
	 269
	   Sequential Test Number:
	1

	Date:
	8/24/07 - 8/30/07
	   Laboratory Technician:
	 B. Swiecicki

	

	TEST
	COMMENTS

	EXTERNAL DIMENSIONS
	Passed all requirements.

	THORACIC SHOCK ABSORBER TEST
	Passed all requirements.

	LATERAL THORAX IMPACT TEST
	Passed all requirements.

	LATERAL PELVIS IMPACT TEST
	Passed all requirements.

	HEAD DROP TEST*
	Passed all requirements.

	LATERAL NECK BEND TEST*
	Passed all requirements.

	ABDOMINAL COMPRESSION TEST*
	Passed all requirements.

	LUMBAR FLEXION TEST*
	Passed all requirements.

	* Test not required for SID certification.


REMARKS: None

	EXTERNAL DIMENSIONS

	PRE-TEST

	

	CONFIGURED FOR 
	LEFT
	 SIDE IMPACT

	

	

	SID H3 Serial No.:
	 269
	   Sequential Test Number:
	1

	Date:
	8/31/07
	   Laboratory Technician:
	 B. Swiecicki

	

	TEST PARAMETER
	SPECIFICATION
	TEST RESULTS

	SH- Seated Height (mm)
	889 – 909
	899

	RH- Rib Height (mm)
	502 – 520
	508

	HP- Hip Pivot Height (mm)
	99 ref.
	99

	RD- Rib from Back Line (mm)
	229 – 241
	234

	KH- Knee Pivot from Back Line (mm)
	511 – 526
	518

	KV- Knee Pivot to Floor (mm)
	490 – 505
	493

	HW- Hip Width (mm)
	356 - 391
	384


REMARKS: None

	PRE-TEST DUMMY INSPECTION LIST

	

	CONFIGURED FOR 
	LEFT
	 SIDE IMPACT

	

	

	SID H3 Serial No.:
	 269
	   Sequential Test Number:
	1

	Date:
	8/23/07
	   Laboratory Technician:
	 B. Swiecicki

	

	PART
	ITEMS CHECKED
	COMMENTS

	SKIN
	VISUAL INSPECTION
	OK

	HEAD
	VISUAL, BALLAST, ACCELEROMETER MOUNT
	OK

	NECK
	VISUAL, CABLE TORQUE
	OK

	SPINE BOX
	VISUAL, BALLAST, WELDMENT, ACCELEROMETER MOUNT
	OK

	RIB CAGE
	VISUAL, MEASURE, STIFFENERS
	OK

	STERNUM
	VISUAL
	OK

	LUMBAR SPINE
	VISUAL
	OK

	ABDOMEN
	VISUAL
	OK

	PELVIS
	VISUAL, PALPATE, ACCELEROMETER MOUNT
	OK

	UPPER LEGS
	VISUAL
	OK

	KNEES
	VISUAL, STOPS, INSERTS
	OK

	LOWER LEGS
	VISUAL, RANGE OF MOTION
	OK

	ANKLES
	VISUAL, RANGE OF MOTION
	OK

	FEET
	VISUAL, RANGE OF MOTION
	OK

	JOINTS
	1 TO 2 g RANGE
	OK

	OTHER
	NONE
	-


REMARKS: None

	CALIBRATION TEST RESULTS

	PRE-TEST

	

	SID H3 NO.:
	270
	

	

	CONFIGURED FOR 
	LEFT
	 SIDE IMPACT


	CALIBRATION TEST RESULTS SUMMARY

	PRE-TEST

	

	CONFIGURED FOR 
	LEFT
	 SIDE IMPACT

	

	

	SID H3 Serial No.:
	 270
	   Sequential Test Number:
	1

	Date:
	8/30/07, 9/25/07
	   Laboratory Technician:
	 B. Swiecicki

	

	TEST
	COMMENTS

	EXTERNAL DIMENSIONS
	Passed all requirements.

	THORACIC SHOCK ABSORBER TEST
	Passed all requirements.

	LATERAL THORAX IMPACT TEST
	Passed all requirements.

	LATERAL PELVIS IMPACT TEST
	Passed all requirements.

	HEAD DROP TEST*
	Passed all requirements.

	LATERAL NECK BEND TEST*
	Passed all requirements.

	ABDOMINAL COMPRESSION TEST*
	Passed all requirements.

	LUMBAR FLEXION TEST*
	Passed all requirements.

	* Test not required for SID certification.


REMARKS: None

	EXTERNAL DIMENSIONS

	PRE-TEST

	

	CONFIGURED FOR 
	LEFT
	 SIDE IMPACT

	

	

	SID H3 Serial No.:
	 270
	   Sequential Test Number:
	1

	Date:
	8/31/07
	   Laboratory Technician:
	 B. Swiecicki

	

	TEST PARAMETER
	SPECIFICATION
	TEST RESULTS

	SH- Seated Height (mm)
	889 - 909
	899

	RH- Rib Height (mm)
	502 - 520
	505

	HP- Hip Pivot Height (mm)
	99 ref.
	99

	RD- Rib from Back Line (mm)
	229 - 241
	234

	KH- Knee Pivot from Back Line (mm)
	511 - 526
	518

	KV- Knee Pivot to Floor (mm)
	490 - 505
	495

	HW- Hip Width (mm)
	356 - 391
	384


REMARKS: None

	PRE-TEST DUMMY INSPECTION LIST

	

	CONFIGURED FOR 
	LEFT
	 SIDE IMPACT

	

	

	SID H3 Serial No.:
	 270
	   Sequential Test Number:
	1

	Date:
	8/23/07
	   Laboratory Technician:
	 B. Swiecicki

	

	PART
	ITEMS CHECKED
	COMMENTS

	SKIN
	VISUAL INSPECTION
	OK

	HEAD
	VISUAL, BALLAST, ACCELEROMETER MOUNT
	OK

	NECK
	VISUAL, CABLE TORQUE
	OK

	SPINE BOX
	VISUAL, BALLAST, WELDMENT, ACCELEROMETER MOUNT
	OK

	RIB CAGE
	VISUAL, MEASURE, STIFFENERS
	OK

	STERNUM
	VISUAL
	OK

	LUMBAR SPINE
	VISUAL
	OK

	ABDOMEN
	VISUAL
	OK

	PELVIS
	VISUAL, PALPATE, ACCELEROMETER MOUNT
	OK

	UPPER LEGS
	VISUAL
	OK

	KNEES
	VISUAL, STOPS, INSERTS
	OK

	LOWER LEGS
	VISUAL, RANGE OF MOTION
	OK

	ANKLES
	VISUAL, RANGE OF MOTION
	OK

	FEET
	VISUAL, RANGE OF MOTION
	OK

	JOINTS
	1 TO 2 g RANGE
	OK

	OTHER
	NONE
	-


REMARKS: None

	CALIBRATION TEST RESULTS

	POST TEST

	

	SID H3 NO.:
	269
	

	

	CONFIGURED FOR 
	LEFT
	 SIDE IMPACT


	CALIBRATION TEST RESULTS SUMMARY

	POST TEST

	

	CONFIGURED FOR 
	LEFT
	 SIDE IMPACT

	

	

	SID H3 Serial No.:
	 269
	   Sequential Test Number:
	1

	Date:
	9/25/07
	   Laboratory Technician:
	 B. Swiecicki

	

	TEST
	COMMENTS

	EXTERNAL DIMENSIONS
	Passed all requirements.

	LATERAL THORAX IMPACT TEST
	Passed all requirements.

	LATERAL PELVIS IMPACT TEST
	Passed all requirements.

	HEAD DROP TEST*
	Passed all requirements.

	LATERAL NECK BEND TEST*
	Passed all requirements.

	ABDOMINAL COMPRESSION TEST*
	Passed all requirements.

	LUMBAR FLEXION TEST*
	Passed all requirements.

	* Test not required for SID certification.


REMARKS: None

	EXTERNAL DIMENSIONS

	POST TEST

	

	CONFIGURED FOR 
	LEFT
	 SIDE IMPACT

	

	

	SID H3 Serial No.:
	 269
	   Sequential Test Number:
	1

	Date:
	9/25/07
	   Laboratory Technician:
	 B. Swiecicki

	

	TEST PARAMETER
	SPECIFICATION
	TEST RESULTS

	SH- Seated Height (mm)
	889 - 909
	899

	RH- Rib Height (mm)
	502 - 520
	505

	HP- Hip Pivot Height (mm)
	99 ref.
	99

	RD- Rib from Back Line (mm)
	229 - 241
	234

	KH- Knee Pivot from Back Line (mm)
	511 - 526
	518

	KV- Knee Pivot to Floor (mm)
	490 - 505
	493

	HW- Hip Width (mm)
	356 - 391
	384


REMARKS: None

	POST TEST DUMMY INSPECTION LIST

	

	CONFIGURED FOR 
	LEFT
	 SIDE IMPACT

	

	

	SID H3 Serial No.:
	 269
	   Sequential Test Number:
	1

	Date:
	9/19/07
	   Laboratory Technician:
	 B. Swiecicki

	

	PART
	ITEMS CHECKED
	COMMENTS

	SKIN
	VISUAL INSPECTION
	OK

	HEAD
	VISUAL, BALLAST, ACCELEROMETER MOUNT
	OK

	NECK
	VISUAL, CABLE TORQUE
	OK

	SPINE BOX
	VISUAL, BALLAST, WELDMENT, ACCELEROMETER MOUNT
	OK

	RIB CAGE
	VISUAL, MEASURE, STIFFENERS
	OK

	STERNUM
	VISUAL
	OK

	LUMBAR SPINE
	VISUAL
	OK

	ABDOMEN
	VISUAL
	OK

	PELVIS
	VISUAL, PALPATE, ACCELEROMETER MOUNT
	OK

	UPPER LEGS
	VISUAL
	OK

	KNEES
	VISUAL, STOPS, INSERTS
	OK

	LOWER LEGS
	VISUAL, RANGE OF MOTION
	OK

	ANKLES
	VISUAL, RANGE OF MOTION
	OK

	FEET
	VISUAL, RANGE OF MOTION
	OK

	JOINTS
	1 TO 2 g RANGE
	OK

	OTHER
	NONE
	-


REMARKS: None

	CALIBRATION TEST RESULTS

	POST TEST

	

	SID H3 NO.:
	270
	

	

	CONFIGURED FOR 
	LEFT
	 SIDE IMPACT


	CALIBRATION TEST RESULTS SUMMARY

	POST TEST

	

	CONFIGURED FOR 
	LEFT
	 SIDE IMPACT

	

	

	SID H3 Serial No.:
	 270
	   Sequential Test Number:
	1

	Date:
	9/25/07
	   Laboratory Technician:
	 B. Swiecicki

	

	TEST
	COMMENTS

	EXTERNAL DIMENSIONS
	Passed all requirements.

	LATERAL THORAX IMPACT TEST
	Passed all requirements.

	LATERAL PELVIS IMPACT TEST
	Passed all requirements.

	HEAD DROP TEST*
	Passed all requirements.

	LATERAL NECK BEND TEST*
	Passed all requirements.

	ABDOMINAL COMPRESSION TEST*
	Passed all requirements.

	LUMBAR FLEXION TEST*
	Passed all requirements.

	* Test not required for SID certification.


REMARKS: None

	EXTERNAL DIMENSIONS

	POST TEST

	

	CONFIGURED FOR 
	LEFT
	 SIDE IMPACT

	

	

	SID H3 Serial No.:
	 270
	   Sequential Test Number:
	1

	Date:
	9/25/07
	   Laboratory Technician:
	 B. Swiecicki

	

	TEST PARAMETER
	SPECIFICATION
	TEST RESULTS

	SH- Seated Height (mm)
	889 - 909
	899

	RH- Rib Height (mm)
	502 - 520
	505

	HP- Hip Pivot Height (mm)
	99 ref.
	99

	RD- Rib from Back Line (mm)
	229 - 241
	234

	KH- Knee Pivot from Back Line (mm)
	511 - 526
	518

	KV- Knee Pivot to Floor (mm)
	490 - 505
	493

	HW- Hip Width (mm)
	356 - 391
	386


REMARKS: None

	POST TEST DUMMY INSPECTION LIST

	

	CONFIGURED FOR 
	LEFT
	 SIDE IMPACT

	

	

	SID H3 Serial No.:
	 270
	   Sequential Test Number:
	1

	Date:
	9/19/07
	   Laboratory Technician:
	 B. Swiecicki

	

	PART
	ITEMS CHECKED
	COMMENTS

	SKIN
	VISUAL INSPECTION
	OK

	HEAD
	VISUAL, BALLAST, ACCELEROMETER MOUNT
	OK

	NECK
	VISUAL, CABLE TORQUE
	OK

	SPINE BOX
	VISUAL, BALLAST, WELDMENT, ACCELEROMETER MOUNT
	OK

	RIB CAGE
	VISUAL, MEASURE, STIFFENERS
	OK

	STERNUM
	VISUAL
	OK

	LUMBAR SPINE
	VISUAL
	OK

	ABDOMEN
	VISUAL
	OK

	PELVIS
	VISUAL, PALPATE, ACCELEROMETER MOUNT
	OK

	UPPER LEGS
	VISUAL
	OK

	KNEES
	VISUAL, STOPS, INSERTS
	OK

	LOWER LEGS
	VISUAL, RANGE OF MOTION
	OK

	ANKLES
	VISUAL, RANGE OF MOTION
	OK

	FEET
	VISUAL, RANGE OF MOTION
	OK

	JOINTS
	1 TO 2 g RANGE
	OK

	OTHER
	NONE
	-


REMARKS: None

	TEMPERATURE TRACE

	

	[image: image68.emf]2008 Subaru Impreza M85501 Environmental Conditions

0.0

10.0

20.0

30.0

40.0

50.0

60.0

70.0

9/12/2007 19:12 9/12/2007 22:48 9/13/2007 2:24 9/13/2007 6:00 9/13/2007 9:36 9/13/2007 13:12 9/13/2007 16:48

Date/Time

Temp (ºC)

%rH




APPENDIX D
TEST EQUIPMENT AND CALIBRATION INFORMATION
	TEST EQUIPMENT LIST AND CALIBRATION INFORMATION

	

	SID/HIII INSTRUMENTATION

	

	
	

	
	FRONT SID/HIII NO.: 269

	
	SERIAL NUMBER
	MANUFACTURER
	CALIBRATION DATE

	HEAD AX
	AC-P22943
	ENDEVCO
	11-Jul-07

	HEAD AY
	AC-P35789
	ENDEVCO
	11-Jul-07

	HEAD AZ
	AC-P13323
	ENDEVCO
	11-Jul-07

	UPPER NECK FX
	LC-442Fx
	DENTON
	08-Nov-06

	UPPER NECK FY
	LC-442Fy
	DENTON
	08-Nov-06

	UPPER NECK FZ
	LC-442Fz
	DENTON
	08-Nov-06

	UPPER NECK MX
	LC-442Mx
	DENTON
	08-Nov-06

	UPPER NECK MY
	LC-442My
	DENTON
	08-Nov-06

	UPPER NECK MZ
	LC-442Mz
	DENTON
	08-Nov-06

	UPPER RIB
	AC-P39731
	ENDEVCO
	11-Jul-07

	LOWER RIB
	AC-P38896
	ENDEVCO
	11-Jul-07

	LOWER SPINE
	AC-P38188
	ENDEVCO
	11-Jul-07

	PELVIS
	AC-P38132
	ENDEVCO
	11-Jul-07

	UPPER RIB REDUNDANT
	AC-P39740
	ENDEVCO
	11-Jul-07

	LOWER RIB REDUNDANT
	AC-P35786
	ENDEVCO
	11-Jul-07

	LOWER SPINE REDUNDANT
	AC-P38216
	ENDEVCO
	11-Jul-07

	PELVIS REDUNDANT
	AC-P35757
	ENDEVCO
	11-Jul-07

	
	

	
	

	
	

	
	REAR SID/HIII NO.: 270

	
	SERIAL NUMBER
	MANUFACTURER
	CALIBRATION DATE

	HEAD AX
	AC-P35812
	ENDEVCO
	11-Jul-07

	HEAD AY
	AC-P18739
	ENDEVCO
	11-Jul-07

	HEAD AZ
	AC-P19212
	ENDEVCO
	11-Jul-07

	UPPER NECK FX
	LC-1629Fx
	DENTON
	08-Nov-06

	UPPER NECK FY
	LC-1629Fy
	DENTON
	08-Nov-06

	UPPER NECK FZ
	LC-1629Fz
	DENTON
	08-Nov-06

	UPPER NECK MX
	LC-1629Mx
	DENTON
	08-Nov-06

	UPPER NECK MY
	LC-1629My
	DENTON
	08-Nov-06

	UPPER NECK MZ
	LC-1629Mz
	DENTON
	08-Nov-06

	UPPER RIB
	AC-P18558
	ENDEVCO
	11-Jul-07

	LOWER RIB
	AC-P18537
	ENDEVCO
	12-Jul-07

	LOWER SPINE
	AC-P18639
	ENDEVCO
	11-Jul-07

	PELVIS
	AC-P21171
	ENDEVCO
	11-Jul-07

	UPPER RIB REDUNDANT
	AC-P35818
	ENDEVCO
	11-Jul-07

	LOWER RIB REDUNDANT
	AC-P19222
	ENDEVCO
	12-Jul-07

	LOWER SPINE REDUNDANT
	AC-P16576
	ENDEVCO
	11-Jul-07

	PELVIS REDUNDANT
	AC-P22639
	ENDEVCO
	12-Jul-07


REMARKS: None

	TEST EQUIPMENT LIST AND CALIBRATION INFORMATION

	

	VEHICLE AND MDB INSTRUMENTATION

	

	
	

	
	VEHICLE AND MDB INSTRUMENTS

	
	SERIAL NUMBER
	MANUFACTURER
	CALIBRATION DATE

	RIGHT FRONT SILL (X)
	AC-P18785
	ENDEVCO
	09-Aug-07

	RIGHT FRONT SILL (Y)
	AC-P23926
	ENDEVCO
	09-Aug-07

	RIGHT FRONT SILL (Z)
	AC-P32218
	ENDEVCO
	09-Aug-07

	RIGHT REAR SILL (X)
	AC-P23939
	ENDEVCO
	09-Aug-07

	RIGHT REAR SILL (Y)
	AC-P23999
	ENDEVCO
	09-Aug-07

	RIGHT REAR SILL (Z)
	AC-P23993
	ENDEVCO
	09-Aug-07

	REAR FLOORPAN ABOVE AXLE (X)
	AC-P17237
	ENDEVCO
	09-Aug-07

	REAR FLOORPAN ABOVE AXLE (Y)
	AC-P16625
	ENDEVCO
	09-Aug-07

	REAR FLOORPAN ABOVE AXLE (Z)
	AC-J32838
	ENDEVCO
	09-Aug-07

	LEFT REAR SILL (Y)
	AC-P16845
	ENDEVCO
	24-Apr-07

	LEFT FRONT SILL (Y)
	AC-P26266
	ENDEVCO
	10-Aug-07

	LEFT FRONT DOOR CENTERLINE (Y)
	AC-FA2486
	ICS
	09-Jul-07

	RIGHT REAR SEAT OCCUPANT COMP. (Y)
	AC-P23288
	ENDEVCO
	24-Apr-07

	MID REAR OF LEFT FRONT DOOR (Y)
	AC-P21393
	ENDEVCO
	24-Apr-07

	LEFT FRONT DOOR UPPER C\L (Y)
	AC-J37854
	ENDEVCO
	10-Aug-07

	MID REAR OF LEFT REAR DOOR (Y)
	AC-P23136
	ENDEVCO
	10-Aug-07

	LEFT REAR DOOR UPPER C\L (Y)
	AC-P23164
	ENDEVCO
	11-Jul-07

	LOWER LEFT B- PILLAR (Y)
	AC-P18728
	ENDEVCO
	10-Aug-07

	MIDDLE LEFT B-PILLAR (Y)
	AC-P23134
	ENDEVCO
	30-Jun-07

	LOWER LEFT A-PILLAR (Y)
	AC-P18792
	ENDEVCO
	23-Mar-07

	UPPER LEFT A-PILLAR (Y)
	AC-P17535
	ENDEVCO
	22-Mar-07

	FRONT SEAT TRACK (Y)
	AC-C15007
	ENDEVCO
	12-Sep-07

	REAR SEAT TRACK (Y)
	AC-C16416
	ENDEVCO
	12-Sep-07

	VEHICLE CG (X)
	AC-C16499
	ENDEVCO
	12-Sep-07

	VEHICLE CG (Y)
	AC-C14948
	ENDEVCO
	12-Sep-07

	VEHICLE CG (Z)
	AC-C16680
	ENDEVCO
	12-Sep-07

	MDB CG (X)
	AC-P18785
	ENDEVCO
	09-Aug-07

	MDB CG (Y)
	AC-P23926
	ENDEVCO
	09-Aug-07

	MDB CG (Z)
	AC-P32218
	ENDEVCO
	09-Aug-07

	MDB REAR FRAME MEMBER (X)
	AC-P23939
	ENDEVCO
	09-Aug-07

	MDB REAR FRAME MEMBER (Y)
	AC-P23999
	ENDEVCO
	09-Aug-07


REMARKS: None

�EMBED Word.Picture.8���





�EMBED Word.Picture.8���





�EMBED Word.Picture.8���





� EMBED Word.Picture.8  ���





� EMBED Word.Picture.8  ���





� EMBED Word.Picture.8  ���





� EMBED Word.Picture.8  ���





� EMBED Word.Picture.8  ���





� EMBED Word.Picture.8  ���





� EMBED Word.Picture.8  ���








i
	
	iii
	8765-SNCAP-26



[image: image78.emf] 

LEFT SIDE VIEW  

1   2  

3  

4   5  

6   8  

7  

9  

10  

11  

13  

12   14  

15  

16  

17  

18  

5   4  

[image: image79.png]Treadwear, traction
and temperature grades

Trepy
compagition
used

USS. DOT satety

standard code Passenger

car tire
Width of tire
meters

Ratio of height to
‘width (aepect ratio)

Load index &
speed symbol
Diameter of

wheel ininches Rainl




[image: image80.wmf]LEFT SIDE VIEW

STEERING COLUMN ASSEMBLY

1

2

3

[image: image81.wmf] 

Upright

 

Position

 

Adjuster

 

Inclinometer

 

Seat

 

Back

 

Angle

 

(Degrees)

 

FRONT SEAT ASSEMBLY

 

Seat Cushion

 

[image: image82.wmf] 

Seat Cushion

 

Seat Travel Area

 

 

A

 

A

 

B

 

[image: image83.wmf] 

Seat Cushion

 

 

A

 

A

 

B

 

Seat Travel Area

 

[image: image84.wmf] 

Vapor Volume

 

Unusable Capacity

 

Filler Neck Volume

 

Filler Cap

 

VEHICLE FUEL TANK ASSEMBLY

 

Fuel Line

 

Usable

 

Capacity

 

[image: image85.wmf] 

A

 

F

 

E

 

B

 

C

 

D

 

G

 

H

 

I

 

K

 

J

 

M

 

L

 

N

 

Q

 

P

 

O

 

LEFT SIDE VIEW

 

[image: image86.wmf] 

LEFT SIDE VIEW

 

                

-

1200 

–

1050 

–

900 

–

750 

–

600 

–

450  

–

300  

–

150     0     150   300   450    600   750   900  1050 1200 1350 1500  1650 1800 1950 2100  2250 2400  2550 2600 27

50  2900 3050 

 

Level 5

 

Level 3

 

Level 4

 

Level 2

 

Level 1

 

All Measurements Shown in mm

 

[image: image87.wmf] 

 

-

1200

 

 

-

900

 

 

–

600

 

 

–

300

 

 

  0

 

 

 

300

 

 

600

 

 

900

 

  1200

 

 1500

 

  1800

 

  2100

 

  2400

 

  2600

 

 2900

 

  3050

 

0

 

300

 

600

 

1500

 

900

 

1200

 

1500

 

Test Vehicle Longitudinal Centerline

 

Rearwardmost Point 

of Induced Damage

 

Forwardmos

t Point 

 

Of Induced Damage

 

Reference Plane

 

TOP VIEW

 

All Dimensions Shown in millimeters

 

[image: image88.wmf] 

Level 1

 

432 mm

 

Level 3

 

686 mm

 

Level 4

 

813 mm

 

Mounting

 

Block

 

MDB Frame

 

Level 1 

–

 Bumper Centerline

 

Level 2 

–

 Top of Bumper

 

Level 3 

–

 Middle Stack Level

 

Level 4 

–

 Top of Stack

 

Grou

nd Surface

 

Right

 

Left

 

Level 2

 

533 mm

 

_1188385051.doc
[image: image1.bmp]

X







Y







1











































2







Z




















_1188386003.doc
[image: image1.emf] 


A  




Seat Cushion



















B







�A











Seat Travel Area












_1198307949.doc






 HW







HZ







 PA(















KDA(



















  HH























NB







NR







PHX











CS







KD







CD







 PA(







KB







CB







PHZ







PHZ







HZ







PHX




































_1198309945.doc
 





HR











HS



















AD























HD



























































FRONT VIEW OF DUMMY












_1188386042.doc
[image: image1.emf] 


A  




Seat Cushion



















B







�A











Seat Travel Area












_1188385307.doc














I











M











N















L







J







K



















LEFT SIDE VIEW



















C







D







F







O























B







H



















E







A











P















































G







Q












_1188385379.doc


 -1200		-900	 –600	 –300		  0			300		600		900	  1200	 1500	  1800	  2100	  2400	  2600	 2900	  3050







0







300







600







1500







900







1200







1500







Test Vehicle Longitudinal Centerline







Rearwardmost Point of Induced Damage







Forwardmost Point 



Of Induced Damage







Reference Plane







TOP VIEW







All Dimensions Shown in millimeters












_1188385425.doc














6











10











15















9







11







13







4







5











LEFT SIDE VIEW



















2







4







1







16























3







7











18











17







5











12















































8







14












_1188385364.doc






























Level 2







































LEFT SIDE VIEW































                -1200 –1050 –900 –750 –600 –450  –300  –150     0     150   300   450    600   750   900  1050 1200 1350 1500  1650 1800 1950 2100  2250 2400  2550 2600 2750  2900 3050 



























Level 4























All Measurements Shown in mm











Level 3















































Level 5







Level 1












_1188385088.doc
[image: image1.bmp]

X







Y







27o







5







9







8







1







2







7







6







3







4







10







63o







11







IMPACT



POINT












_1188384214.doc


Seat Cushion







Upright



Position







Adjuster







Inclinometer







Seat



Back







Angle



(Degrees)











FRONT SEAT ASSEMBLY












_1188384383.doc


Usable



Capacity











Vapor Volume







Unusable Capacity







Filler Neck Volume







Filler Cap







VEHICLE FUEL TANK ASSEMBLY







Fuel Line












_1188384893.doc
[image: image1.bmp][image: image2.bmp]

Level 1



432 mm







Level 2



533 mm







Level 3



686 mm







Level 4



813 mm







Mounting



Block







MDB Frame







Level 1 – Bumper Centerline



Level 2 – Top of Bumper



Level 3 – Middle Stack Level



Level 4 – Top of Stack







Ground Surface







Right







Left












_1062398800.doc
[image: image1.png]






_984601894.doc


LEFT SIDE VIEW







STEERING COLUMN ASSEMBLY







1







2







3












