REPORT NUMBER: NCAP-MGA-2008-011
NEW CAR ASSESSMENT PROGRAM

FRONTAL BARRIER IMPACT TEST

BAYERISCHE MOTORENWERKE
2008 BMW X5
NHTSA NUMBER:  M80504
PREPARED BY:

MGA RESEARCH CORPORATION

5000 WARREN ROAD

BURLINGTON, WI 53105


[image: image30.jpg]



Test Date:  December 19, 2007
Final Report Date:  February 5, 2008
FINAL REPORT

PREPARED FOR:

U.S. DEPARTMENT OF TRANSPORTATION

NATIONAL HIGHWAY TRAFFIC SAFETY ADMINISTRATION

RULEMAKING

OFFICE OF CRASHWORTHINESS STANDARDS

1200 NEW JERSEY AVENUE, SE, ROOM W43-410
WASHINGTON, D.C. 20590

This final test report was prepared for the U.S. Department of Transportation, National Highway Traffic Safety Administration, in response to Contract Number DTNH22-06-D-00028.

This publication is distributed by the U.S. Department of Transportation, National Highway Traffic Safety Administration, in the interest of information exchange. The opinions, findings and conclusions expressed in this publication are those of the author(s) and not necessarily those of the Department of Transportation or the National Highway Traffic Safety Administration. The United States Government assumes no liability for its contents or use thereof. If trade or manufacturers' names or products are mentioned it is only because they are considered essential to the object of the publication and should not be construed as an endorsement. The United States Government does not endorse products or manufacturers. 
[image: image1.png]



Prepared by:  _____________________________      
Date:  2/05/08
Ben Fischer, Project Engineer

[image: image17.wmf]LEFT SIDE VIEW

STEERING COLUMN ASSEMBLY

1

2

3


Reviewed by: _____________________________       
Date:  2/05/08
David Winkelbauer, Facility Director
Technical Report Documentation Page

	1. Report No.

 NCAP-MGA-2008-011
	2. Government Accession No. 
	3. Recipient's Catalog No.

	4. Title and Subtitle

 Final Report of New Car Assessment Program Testing

 of a 2008 BMW X5
 NHTSA No.: M80504

	5. Report Date

February 5, 2008

	
	6. Performing Organization Code

 MGA

	7. Author(s)

Ben Fischer, Project Engineer
	8. Performing Organization Report No.

 NCAP-MGA-2008-011

	9. Performing Organization Name and Address

 MGA Research Corporation

 5000 Warren Road

 Burlington, WI 53105
	10. Work Unit No.



	
	11. Contract or Grant No.

 DTNH22-06-D-00028

	12. Sponsoring Agency Name and Address

 U.S. Department of Transportation

 National Highway Traffic Safety Administration

 Rulemaking, Office of Crashworthiness Standards

 1200 New Jersey Ave, SE, Room W43-410, NVS-111
 Washington, D.C. 20590


	13. Type of Report and Period Covered

Final Report

 12/19/2007 – 2/05/2008

	
	14. Sponsoring Agency Code

 NVS-111

	15. Supplementary Notes

	16. Abstract

A frontal barrier impact was conducted on a 2008 BMW X5 at MGA Research Corporation on December 19, 2007. This test was conducted to obtain data indicant of FMVSS 208, 212, 219 (partial), 301, and foot well intrusion performance. The impact velocity was 56.3 km/h. The ambient temperature at the barrier face at the time of impact was 21 degrees Celsius. The vehicle's maximum post test static crush is 597 mm located to the left of the vehicle’s centerline. The test vehicle is equipped with a 3-point continuous belt system and an airbag in both front outboard seating positions. With respect to FMVSS 208 "Occupant Crash Protection", the occupant injury criteria  summary is as follows:

Measurement Description

Units

Threshold

Driver ATD

Pass. ATD

Head Injury Criteria (HIC)

N/A

1000
405
574
Max. Thorax Accel. (3ms Clip)

G’s

60
41
48
Left Femur Force

Newton

10009
-412
-3354
Right Femur Force

Newton

10009
-3113
-233


	17. Key Words

 56.3 km/h NCAP Frontal Barrier Impact Test

 New Car Assessment Program (NCAP)

 2008 BMW X5
 NHTSA No: M80504
	18. Distribution Statement

Copies of this report are available from:

National Highway Traffic Safety Adm.

Technical Ref. Division,

1200 New Jersey Ave, SE 

Washington, D.C. 20590

	19. Security Classif. (of this report)

 Unclassified
	20. Security Classif. (of this page) 

Unclassified
	21. No. of Pages

           169
	22. Price


Form DOT F1700.7 (8-72)

TABLE OF CONTENTS

	Section
	
	Page No

	    1
	Purpose and Summary of the Test
	      1

	    2
	Occupant and Vehicle Information / Data Sheets
	      3

	
	
	

	Data Sheet No.
	
	Page No.

	    1
	Crash Test Summary 
	      3

	    2
	General Test and Vehicle Parameter Data
	      4

	    3
	Test Vehicle Tire Information
	      6

	    4
	Test Vehicle Information
	      7

	    5
	Dummy Positioning in Vehicle
	      9

	    6
	Seat Belt Positioning Data
	     11

	    7
	Vehicle Accelerometer Locations
	     12

	    8
	Summary of FMVSS 212 and FMVSS 219 (Partial) Data
	     13

	    9
	Summary of FMVSS 301 Data
	     14

	   10
	Vehicle Measurements
	     15

	   11
	Camera Locations
	     18

	   12
	Photographic Reference Target Locations
	     20

	   13
	Vehicle Intrusion Measurements
	     21

	   14
	Load Cell Locations on Fixed Barrier
	     24

	   15
	Accident Investigation Division Data
	     25

	   16
	Dummy/Vehicle Temperature Stabilization Chart
	     26

	
	
	

	Appendix
	
	

	    A
	Photographs
	      A

	    B
	Dummy Response Data Traces
	      B

	    C
	Dummy Calibration Data 
	      C

	    D
	Child Dummy Data
	      D


SECTION 1

PURPOSE AND SUMMARY OF TEST

PURPOSE


This frontal barrier impact test is part of the Vehicle Barrier Impact Testing Program sponsored by the National Highway Traffic Safety Administration (NHTSA) under contract number DTNH22-06-D-00028.  The purpose of this test was to obtain vehicle crashworthiness and occupant restraint system performance data for an impact in excess of the current 48.3 kph requirements.

SUMMARY


A load cell barrier was impacted by a 2008 BMW X5 at a velocity of 56.3 kph.  The test was performed at MGA Research Corporation on December 19, 2007.  Pre-and post-test photographs of the vehicle and dummies can be found in Appendix A.

One real-time camera and sixteen high-speed cameras were used to document the frontal barrier impact event.  Camera locations and other pertinent camera information can be found in this report.


Two Part 572E, 50th percentile male anthropomorphic test devices (ATDs), were placed in the driver and right-front passenger seating positions according to dummy placement instructions specified in the Laboratory Indicant Test Procedure.


Both ATDs were fully instrumented with head, chest and pelvis tri-axial accelerometers, chest displacement potentiometer, upper neck transducers, right/left femur load cells, and lower leg instrumentation.  The driver (position 1) ATD (Serial No. 065) and right-front passenger (position 2) ATD (Serial No. 066) were calibrated previous to this test.  Certification details, along with instrumentation calibration data, are found in Appendix C.


The 102 channels of data were recorded on an on-board data acquisition system.

Appendix B contains the dummy head, chest, and femur response data traces.


There was 100 percent windshield retention and no intrusion into the protected zone of the windshield during the event.  There was no Stoddard Solvent leakage after the event or during any phase of the static rollover.


The maximum static crush of the vehicle was 597 mm and both the driver and passenger side doors remained closed and latched during the impact event and were operable after the impact.


The driver’s head and chest contacted the airbag.  The driver’s head also contacted the headrest and headliner. The driver’s knees contacted the knee bolster.  The passenger’s head and chest contacted the airbag.  The passenger’s head also contacted the headrest and B-Pillar.  The passenger’s knees contacted the glove box. 

The occupant data is summarized below:

	ATD position
	HIC
	T1
	T2
	Clip (g)
	T1
	T2
	Chest Disp. (mm)
	Left Femur (N)
	Right Femur (N)

	Driver
	405
	60.0
	96.0
	41
	66.1
	69.1
	-28
	-412
	-3113

	Passenger
	574
	67.0
	103.0
	48
	70.9
	75.2
	-28
	-3354
	-233


The test data can be found on the NHTSA website at www.nhtsa.dot.gov. 

TEST NOTES
There was no valid data collected for:

Bottom of Engine X after 30 msec.
Right Brake Caliper X after 30 msec.

Left Brake Caliper X after 80 msec.

Left Rear Seat Crossmember Z after 50 msec.

SECTION 2

OCCUPANT AND VEHICLE INFORMATION / DATA SHEETS

DATA SHEET NO. 1

CRASH TEST SUMMARY

	Test Vehicle:
	2008 BMW X5
	NHTSA No.:
	M80504

	Test Program:
	35mph Frontal Impact
	Test Date:
	12/19/2007


DOOR OPENING AND SEAT TRACK INFORMATION

	Description
	Driver
	Passenger

	Locked/Unlocked Doors
	Doors were unlocked
	Doors were unlocked

	Front Door Opening
	Door remained closed and latched; Door opened without tools
	Door remained closed and latched; Door opened without tools

	Rear Door Opening
	Door remained closed and latched; Door opened without tools
	Door remained closed and latched; Door opened without tools

	Seat Track Shift (mm)
	0
	0

	Seat Back Failure
	None
	None

	Glazing Damage
	The windshield cracked.


VEHICLE REBOUND FROM BARRIER

	Measured Parameter
	Units
	Value

	Left Side
	mm
	574

	Center 
	mm
	543

	Right Side
	mm
	630

	Average
	mm
	582


BELT LENGTH DATA

	Measurement Description
	Units
	Driver
	Passenger

	Shoulder belt length as measured on ATD
	mm
	890
	845

	Lap belt length as measured on ATD
	mm
	640
	640

	Remainder of belt on reel
	mm
	970
	1030

	Total belt length for continuous webbing systems
	mm
	2500
	2515


DATA SHEET NO. 2

GENERAL TEST AND VEHICLE PARAMETER DATA
	Test Vehicle:
	2008 BMW X5
	NHTSA No.:
	M80504

	Test Program:
	35mph Frontal Impact
	Test Date:
	12/19/2007


	TEST VEHICLE INFORMATION
	
	TEST VEHICLE OPTIONS

	Manufacturer
	BMW
	
	Front Airbag
	Yes

	Model
	X5
	
	Driver Side Curtain Airbag
	Yes

	Body Style
	SUV
	
	Driver Side Torso Airbag
	Yes

	NHTSA No.
	M80504
	
	Rear Passenger Side Curtain Airbag
	Yes

	VIN
	5UXFE43568L002144
	
	Rear Passenger Side Torso Airbag
	No

	Color
	Black Sapphire Metallic
	
	Force Limiter
	Yes

	Delivery Date
	12/06/2007
	
	Pretensioner
	Yes

	Odometer Reading (mile)
	73
	
	Power Steering
	Yes

	Dealer
	BMW of Riverside
	
	Power Door Locks
	Yes

	Transmission
	Automatic
	
	Tilt Wheel
	Yes

	Final Drive
	AWD
	
	Air Conditioning
	Yes

	Number of Cylinders
	6
	
	Anti-lock Brakes
	Yes

	Engine Displacement (L)
	3.0
	
	Traction Control
	Yes

	Engine Placement
	Longitudinal
	
	All Wheel Drive
	Yes

	Automatic Door Lock (ADL)
	Yes
	
	Power Seats
	Yes

	Owners Manual Details Instructions on Disabling ADLs
	Yes
	
	
	

	Bucket Seats
	Yes
	
	
	


	DATA FROM CERTIFICATION LABEL

	Manufactured By
	Bayerische Motorenwerke
	
	GVWR (kg)
	2725

	Date of Manufacture
	10/07
	
	GAWR Front (kg)
	1260

	
	
	GAWR Rear (kg)
	1560


	Measured Parameter
	Front
	Rear
	Third
	Total

	Type of Seats
	Bucket
	Bench
	
	

	Number of Occupants
	2
	3
	
	5

	Capacity Wt. (VCW) (kg)
	
	
	
	524

	Cargo Wt. (RCLW) (kg)
	
	
	
	136


DATA SHEET NO. 2... (CONTINUED)

GENERAL TEST AND VEHICLE PARAMETER DATA
	Test Vehicle:
	2008 BMW X5
	NHTSA No.:
	M80504

	Test Program:
	35mph Frontal Impact
	Test Date:
	12/19/2007


TEST VEHICLE WEIGHTS

	
	Units
	As Delivered (UVW) (Axle)
	As Tested (ATW) (Axle)

	
	
	Front
	Rear 
	Total
	Front 
	Rear
	Total

	Left
	kg
	530.7
	550.2
	
	570.2
	650.4
	

	Right
	kg
	526.2
	565.6
	
	565.2
	671.8
	

	Ratio
	%
	48.6
	51.4
	
	46.2
	53.8
	

	Totals
	kg
	1056.9
	1115.8
	2172.7
	1135.4
	1322.2
	2457.6


TARGET TEST WEIGHT CALCULATION

	Measured Parameter
	Units
	Value

	Total Delivered Weight (UVW)
	kg
	2172.7

	Weight of 2 P572E ATDs
	kg
	156.0

	Rated Cargo/Luggage Weight (RCLW)
	kg
	136

	Calculated Vehicle Target Weight (TVTW)
	kg
	2464.7


TEST VEHICLE ATTITUDES AND CG

	
	Units
	LF
	RF
	LR
	RR
	CG (aft of front axle)

	As Delivered
	mm
	821
	820
	822
	818
	1505

	As Tested
	mm
	813
	808
	795
	790
	1576

	Post Test
	mm
	795
	817
	790
	813
	


Vehicle Wheelbase (mm): 






   
 2930 Weight of Ballast secured in truck box (kg): 

                      


  22.7
Vehicle Components Removed:
Jack, spare tire, rear cargo cover, and splash guard.
Ballast weight does not include instrumentation and data acquisition system.
DATA SHEET NO. 3

TEST VEHICLE TIRE INFORMATION

	Test Vehicle:
	2008 BMW X5
	NHTSA No.:
	M80504

	Test Program:
	35mph Frontal Impact
	Test Date:
	12/19/2007


[image: image18.wmf] 

Upright

 

Position

 

Adjuster

 

Inclinometer

 

Seat

 

Back

 

Angle

 

(Degrees)

 

FRONT SEAT ASSEMBLY

 

Seat Cushion

 


	DATA FROM TIRE PLACARD

	Measured Parameter
	Front
	Rear

	Maximum Tire Pressure (kPa)
	345
	345

	Cold Pressure (kPa)
	220
	220

	Recommended Tire Size
	255/55R18
	255/55R18

	Tire Size on Vehicle
	255/55R18
	255/55R18

	Tire Manufacturer
	Bridgestone
	Bridgestone

	Tire Name
	Dueler H/L
	Dueler H/L

	Tire Type
	Passenger
	Passenger

	Tire Width (mm)
	255
	255

	Ratio of Height to Width (aspect ratio)
	55
	55

	Radial
	R
	R

	Wheel Diameter
	18
	18

	Load Index & Speed Symbol
	109H
	109H

	Treadwear
	300
	300

	Traction Grade
	A
	A

	Temperature Grade
	A
	A


DATA SHEET NO. 4
TEST VEHICLE INFORMATION

	Test Vehicle:
	2008 BMW X5
	NHTSA No.:
	M80504

	Test Program:
	35mph Frontal Impact
	Test Date:
	12/19/2007


NORMAL DESIGN RIDING POSITION

The driver and passenger seat back is positioned to the manufacturer's designated angle. The procedure is as follows: Set seat back angle at 24.0 degrees measured from upright position for both the driver and passenger.  
[image: image19.wmf] 

A

 

D

 

E

 

B

 

F

 

C

 

Lower Edge of Protected Zone

 

FRONT VIEW OF WINDSHIELD

 

Zero Point

 

Y

 

X

 


Driver seat back angle:  
23.9 degrees at seatback
Passenger seat back angle:
24.0 degrees at seatback
SEAT FORE/AFT POSITIONING

	
	Total Fore/Aft Travel
	Placed in Position #

	Driver Seat
	293 mm
	146 mm

	Passenger Seat
	293 mm
	146 mm


ADJUSTABLE D-RING POSITION

The driver and passenger D-rings were non-adjustable.
[image: image20.wmf] 

I

 

J

 

K

 

C

 

F

 

D

 

E

 

A

 

B

 

B

 

H

 

M

 

G

 

L

 

N

 

O

 

P

 

Q

 



Data Sheet No. 4…(CONTINUED)

TEST VEHICLE INFORMATION
	Test Vehicle:
	2008 BMW X5
	NHTSA No.:
	M80504

	Test Program:
	35mph Frontal Impact
	Test Date:
	12/19/2007


FUEL TANK CAPACITY

	
	Liters

	Usable Capacity of “Standard Tank”
	85.2

	Usable Capacity of “Optional” Tank
	

	92-94% of Usable Capacity 
	78.3 to 80.1

	Actual Amount of Solvent used
	80.1

	1/3 of Usable Capacity
	28.4


[image: image21.wmf]Vapor Volume

Unusable Capacity

Filler Neck Volume

Filler Cap

VEHICLE FUEL TANK ASSEMBLY

Fuel Line

Usable

Capacity

The test vehicle is equipped with an electric fuel pump. When ignition is on, the fuel pump starts to run until the target pressure is reached. The pump then stops until the engine is started; the pump then runs continuously.
Steering Column AdJUSTMENT
[image: image22.jpg]


Steering wheel and column adjustments are made so that the steering wheel hub is at the geometric center of the locus it describes when moved through its full range of motion. An aluminum plate is placed across the rim of the steering wheel, an inclinometer is placed on the plate and the angle is measured. 

STEERING COLUMN POSITIONS
	
	Fore/Aft Position (mm)
	Degrees

	Lowermost position No. 1
	0
	64.7

	Geometric center position No. 2
	20 mm
	66.8

	Uppermost position No. 3
	40 mm
	69.0


DATA SHEET NO. 5

DUMMY POSITIONING IN VEHICLE

	Test Vehicle:
	2008 BMW X5
	NHTSA No.:
	M80504

	Test Program:
	35mph Frontal Impact
	Test Date:
	12/19/2007


DUMMY MEASUREMENTS FOR FRONT SEAT OCCUPANTS


[image: image2.wmf] 

PASS

 

DRVR

 

Vertical Longitudinal Planes

 

Vertical

 

Transverse

 

Plane

 

HS

 

KK

 

AD

 

HD

 

HR

 

SHY

 

STRIKER

 

HZ

 

CD

 

KD

 

CS

 

Seat Back Angle Line

 

SH

 

NR

 

SK

 

AD

 

Arm to Door

 

HD

 

H

-

Point to Door

 

HR

 

Head to Side Header

 

HS

 

Head to Side Window

 

KK

 

Knee to Knee

 

SHY

 

St

riker to H

-

Point

 

          (Y Axis)

 

CD

 

Chest to Dash

 

CS

 

Chest to Steering Wheel Hub

 

HH

 

Head to Header

 

HW

 

Head to Windshield

 

HZ

 

Head to Roof

 

KDA

 

Knee to Dash Angle

 

KDL

 

Left Knee to Dash

 

KDR

 

Right Knee to Dash

 

NA

 

Nose to Rim Angle

 

NR

 

Nose to Rim

 

PA

 

Pelvic Angle

 

RA

 

Rim to Abdomen

 

SA

 

Seat Back Angle

 

SCA

 

Steering Column Angle

 

SH

 

Striker to H

-

Point

 

SK

 

Striker t

o Knee

 

ST

 

Striker to Head

 

SWA

 

Steering Wheel Angle

 

TA

 

Tibial Angle

 

WA

 

Windshield Angle

 

 SWA

°

 

  HH

 

 WA

°

 

 HW

 

 NA

°

 

 SCA

°

 

 KDA

°

 

 TA

°

 

 SA

°

 

ST

 

 PA

°

 


DATA SHEET NO. 5... (CONTINUED)

DUMMY POSITIONING IN VEHICLE

	Test Vehicle:
	2008 BMW X5
	NHTSA No.:
	M80504

	Test Program:
	35mph Frontal Impact
	Test Date:
	12/19/2007


TEST DUMMY POSITION MEASUREMENTS

	Code
	Measurement Description
	Driver
	Passenger

	
	
	Length (mm)
	Angle (°)
	Length (mm)
	Angle (°)

	WA
	Windshield Angle
	
	31.6
	
	

	SWA
	Steering Wheel Angle
	
	66.8
	
	

	SCA
	Steering Column Angle
	
	25.1
	
	

	SA
	Seat Back Angle
	
	23.9
	
	24.0

	HZ
	Head to Roof (Z)
	212
	90
	215
	90

	HH
	Head to Header
	381
	23.3
	381
	24.0

	HW
	Head to Windshield
	693
	0
	698
	0

	HR
	Head to Side Header (Y)
	233
	
	233
	

	NR
	Nose to Rim
	431
	8.4
	
	

	CD
	Chest to Dash
	540
	
	523
	

	CS
	Chest to Steering Hub
	335
	1.2
	
	

	RA
	Rim to Abdomen
	219
	0
	
	

	KDL
	Left Knee to Dash
	196
	32.7
	130
	

	KDR
	Right Knee to Dash
	125
	
	191
	22.4

	PA
	Pelvic Angle
	
	23.2
	
	24.1

	TA
	Tibia Angle
	
	46.6
	
	48.7

	KK
	Knee to Knee (Y)
	335
	
	268
	

	SK
	Striker to Knee
	521
	98.7
	538
	99.8

	ST
	Striker to Head
	479
	2.5
	475
	1.7

	SH
	Striker to H-Point
	227
	146.6
	230
	141.1

	SHY
	Striker to H-Point (Y)
	290
	
	286
	

	HS
	Head to Side Window
	343
	
	339
	

	HD
	H-Point to Door (Y)
	174
	
	164
	

	AD
	Arm to Door (Y)
	160
	
	146
	

	AA
	Ankle to Ankle
	330
	
	257
	


DATA SHEET NO. 6

SEAT BELT POSITIONING DATA

	Test Vehicle:
	2008 BMW X5
	NHTSA No.:
	M80504

	Test Program:
	35mph Frontal Impact
	Test Date:
	12/19/2007



[image: image3.wmf] 

 

Dummy’s Centerline

 

TBI

 

'D' Ring

 

Shoulder Belt 

Portion

 

Lap Belt 

Portion

 

1/8" Thick 

Aluminum 

Plate

 

Inboard 

Anchorage

 

Flo

orpan

 

Buckle 

Assembly

 

PBL

 

PBU

 

Outboard 

Anchor

 

Locking 

Retractor

 


SEAT BELT POSITIONING MEASUREMENTS

	Measurement Description
	Units
	Driver
	Passenger

	PBU - Top surface of reference to belt upper edge
	mm
	355
	355

	PBL - To surface of reference to belt lower edge
	mm
	265
	265


DATA SHEET NO. 7

VEHICLE ACCELEROMETER LOCATIONS

	Test Vehicle:
	2008 BMW X5
	NHTSA No.:
	M80504

	Test Program:
	35mph Frontal Impact
	Test Date:
	12/19/2007


VEHICLE ACCELEROMETER PRE-TEST LOCATIONS

	No.
	 Accelerometer Location 
	Measurements (mm)

	
	
	X
	Y
	Z

	1
	 Left Rear X-Member X
	1898
	-396
	543

	2
	 Right Rear X-Member X
	1898
	362
	543

	3
	 Engine Top X
	4091
	94
	910

	4
	 Engine Bottom X
	3894
	75
	248

	5
	 Left Brake Caliper X
	3832
	-756
	315

	6
	 Right Brake Caliper X 
	3832
	756
	315

	7
	 Instrument Panel X
	
	
	

	8
	 Left Rear X-Member Z
	1898
	-396
	543

	9
	 Right Rear X-Member Z
	1898
	362
	543


Reference Points:
X - Rear Surface of Vehicle (+ forward)   




Y - Vehicle Centerline
(+ to right)






Z - Ground Plane (+ up)


[image: image4.wmf] 

Z

 

Y

 

X

 

5/6

 

4

 

3

 

7

 

1/2/8/9

 

5

 

7

 

1/8

 

2/9

 

3/4

 

6

 


DATA SHEET NO. 8

SUMMARY OF FMVSS 212 AND FMVSS 219 (Partial) DATA

	Test Vehicle:
	2008 BMW X5
	NHTSA No.:
	M80504

	Test Program:
	35mph Frontal Impact
	Test Date:
	12/19/2007


Windshield Mounting Details:

Windshield glass is secured to the vehicle frame with a rubber trim and glue.

The standard requires that the post-test retention measurement be a minimum of 75 percent of the pretest total periphery measurement for vehicles not equipped with occupant passive restraints and 50 percent for each side of the windshield for vehicles, which are equipped with occupant passive restraints.

Temperature of windshield molding during test: 21°C

WINDSHIELD PERIPHERY MEASUREMENTS

	Measurement
	Pre-Test (mm)
	Post-Test (mm)
	% of Retention

	Left Side
	2252
	2252
	100

	Right Side
	2252
	2252
	100

	Total
	4504
	4504
	100


[image: image23.emf] 

Seat Travel Area  

A  

B  

Seat  Cushion  

Seat Travel Area  

B  

A  

Seat  Cushion  


	Item
	Units 
	Value

	A
	mm
	1296

	B
	mm
	513

	C
	mm
	1660

	D
	mm
	774

	E
	mm
	533

	F
	mm
	559


AREA OF PROTECTED ZONE FAILURES - NONE
	A.  Provide coordinates of the area that the protected zone was penetrated more than 0.25 inches by a vehicle component other than one that is normally in contact with the windshield. None


	X

Y



	B.   Provide coordinates of the area beneath the protected zone that the inner surface of the windshield was penetrated by a vehicle component. None

	X

Y



DATA SHEET NO. 9

SUMMARY OF FMVSS 301 DATA

	Test Vehicle:
	2008 BMW X5
	NHTSA No.:
	M80504

	Test Program:
	35mph Frontal Impact
	Test Date:
	12/19/2007


FMVSS 301 FUEL SYSTEM INTEGRITY POST IMPACT DATA

Temperature at Time of Impact: 
21o C               Test Time:   1:39 pm

Stoddard Solvent Spillage Measurements

A. 
From impact until vehicle motion ceases: 



      0    oz.


(Maximum Allowable = 1 ounce)

B. 
For the 5 minute period after motion ceases: 


       0   oz.
(Maximum Allowable = 5 ounces)

C. 
For the following 25 minutes: 





       0   oz.
(Maximum Allowable = 1 oz. /minute)

D. 
Spillage: 
      None
                                                                                                                         
FMVSS 301 STATIC ROLLOVER DATA

	
[image: image5.wmf]Rear View

0

o

/360

o

Filler Cap

REAR BUMPER


	
[image: image6.wmf]Rear

View

90

o

Filler Cap

R

E

A

R

B

U

M

P

E

R


	
[image: image7.wmf]Rear View

180

o

Filler Cap

REAR BUMPER


	1. The specified fixture rollover rate for each 90° of rotation is 60 to 180 seconds.

2. The position hold time at each position is 300 seconds (minimum).

3. Details of Stoddard 

Solvent spillage locations:
None


	                        0° to 90° 
         90° to 180°
	

	
[image: image8.wmf]Rear View

180

o

Filler Cap

REAR BUMPER


	
[image: image9.wmf]270

o

Rear View

Filler Cap

R

E

A

R

B

U

M

P

E

R


	
[image: image10.wmf]Rear View

0

o

/360

o

Filler Cap

REAR BUMPER


	

	                    180° to 270°
        270° to 360°
	


	Test Phase
	Rotation Time (sec.)
	Hold Time (sec.)
	Spillage (oz.)

	0° to 90°
	126
	300
	0

	90° to 180°
	118
	300
	0

	180° to 270°
	113
	300
	0

	270° to 360°
	116
	300
	0


DATA SHEET NO. 10

VEHICLE MEASUREMENTS

	Test Vehicle:
	2008 BMW X5
	NHTSA No.:
	M80504

	Test Program:
	35mph Frontal Impact
	Test Date:
	12/19/2007


	No.
	Measurement Description
	Units
	Pre-Test
	Post-Test
	Difference

	1
	Total length of vehicle at centerline
	mm
	4820
	4233
	587

	2
	RSOV to front of engine
	mm
	4294
	4138
	156

	3
	RSOV to firewall centerline
	mm
	3543
	3492
	51

	4
	RSOV to leading edge of right door
	mm
	3263
	3248
	15

	5
	RSOV to leading edge of left door
	mm
	3266
	3268
	-2

	6
	RSOV to lower leading edge of right door
	mm
	3298
	3283
	15

	7
	RSOV to lower leading edge of left door
	mm
	3303
	3296
	7

	8
	RSOV to upper leading edge of right door
	mm
	2209
	2205
	4

	9
	RSOV to upper leading edge of left door
	mm
	2210
	2199
	11

	10
	RSOV to lower trailing edge of right door
	mm
	2228
	2225
	3

	11
	RSOV to lower trailing edge of left door
	mm
	2226
	2224
	2

	12
	RSOV to bottom of right 'A' pillar
	mm
	3291
	3288
	3

	13
	RSOV to bottom of left 'A' pillar
	mm
	3294
	3294
	0

	14
	RSOV to firewall on right side
	mm
	3480
	3420
	60

	15
	RSOV to firewall on left side
	mm
	3462
	3410
	52

	16
	RSOV to steering column
	mm
	2781
	2809
	-28

	17
	Center of steering column to left 'A' pillar
	mm
	349
	362
	-13

	18
	Center of steering column to headlining
	mm
	404
	415
	-11

	19
	RSOV to right side of front bumper
	mm
	4703
	4235
	468

	20
	RSOV to left side of front bumper
	mm
	4702
	4136
	566

	21
	Length of engine block
	mm
	652
	652
	0

	RD
	RSOV to right side of dash panel
	mm
	2997
	2989
	8

	CD
	RSOV to center of dash panel
	mm
	2901
	2895
	6

	LD
	RSOV to left side of dash panel
	mm
	2997
	2987
	10



DATA SHEET NO. 10... (continued)

VEHICLE MEASUREMENTS

	Test Vehicle:
	2008 BMW X5
	NHTSA No.:
	M80504

	Test Program:
	35mph Frontal Impact
	Test Date:
	12/19/2007



[image: image11.wmf]
DATA SHEET NO. 10... (continued)

VEHICLE MEASUREMENTS

	Test Vehicle:
	2008 BMW X5
	NHTSA No.:
	M80504

	Test Program:
	35mph Frontal Impact
	Test Date:
	12/19/2007


Target Vehicle Structural Measurement

	
	Elements
	Pre-Test (mm)

	1
	Total Length
	4820

	2
	Total Width
	1903

	3
	Bumper Top Height
	602

	4
	Bumper Bottom Height
	512

	5
	Longitudinal Member Top Height
	626

	6
	Distance between Longitudinal Members
	794

	7
	Longitudinal Member Width
	68

	8
	Engine Top Height
	929

	9
	Engine Bottom Height
	252

	10
	Engine and gearbox width
	574

	11
	Front bumper-engine distance
	519

	12
	Front shock absorber fixing height
	890

	13
	Bonnet leading edge height
	905

	14
	Front shock absorber fixing width
	932

	15
	Front bumper – front axle distance
	857

	16
	Front axle – a pillar distance
	676

	17
	A-pillar – B-pillar distance
	1098

	18
	B-Pillar – rear axle distance
	1182

	19
	B-pillar – C-pillar distance
	710

	20
	Roof sill bottom height
	1651

	21
	Roof sill top height
	1693

	22
	Floor sill bottom height
	267

	23
	Floor sill top height
	390


DATA SHEET NO. 11

CAMERA LOCATIONS

	Test Vehicle:
	2008 BMW X5
	NHTSA No.:
	M80504

	Test Program:
	35mph Frontal Impact
	Test Date:
	12/19/2007


	No.
	Camera View
	Location (mm) *
	Lens (mm)
	Speed (fps)

	
	
	X
	Y
	Z
	
	

	1
	Real-Time Left Side View
	
	
	
	13
	24

	2
	Left Front View
	1240
	-4520
	1220
	24
	1000

	3
	Steering Column Top
	1100
	-4900
	1300
	25
	1000

	4
	Steering Column Bottom
	1100
	-4900
	1290
	25
	1000

	5
	Driver Close-up
	1300
	-4730
	1350
	35
	1000

	6
	Driver Angle
	6800
	-5050
	2180
	50
	1000

	7
	On board Driver Side
	
	
	
	8
	1000

	8
	On board Passenger Side
	
	
	
	8
	1000

	9
	Right Overall
	1600
	6050
	1350
	24
	1000

	10
	Right Passenger Half
	1250
	4530
	1200
	24
	1000

	11
	Right Close-up
	1300
	4700
	1310
	35
	1000

	12
	Right Angle
	6710
	4980
	2190
	50
	1000

	13
	Windshield
	-285
	0
	2860
	12.5
	1000

	14
	Top Driver
	-135
	-470
	2180
	24
	1000

	15
	Top Passenger
	-110
	420
	2180
	24
	1000

	16
	Pit Front
	1190
	0
	-3150
	24
	1000

	17
	Pit Rear
	3100
	0
	-3150
	24
	1000


*COORDINATES:

+X = forward of impact plane



 

+Y = right of monorail centerline



+Z = above ground level

DATA SHEET NO. 11... (continued)

CAMERA LOCATIONS

	Test Vehicle:
	2008 BMW X5
	NHTSA No.:
	M80504

	Test Program:
	35mph Frontal Impact
	Test Date:
	12/19/2007



[image: image12.wmf]LEFT SIDE VIEW

TOP VIEW

MONORAIL

COVERED PHOTO PIT

TOW ROAD

COVERED PHOTO PIT

REAL TIME CAMERA

1

3

9

16

17

CONCRETE PAD

10

14,15

13

11

12

4

2

CAMERA POSITIONS FOR FRONTAL IMPACTS

CAMERA FRAME RATES:

#1 = 24 fps

All Others = 1,000 fps

TEST

VEH.

 15

MONORAIL

CONCRETE

BARRIER

CONCRETE

BARRIER

6

5

7

 13

 14


DATA SHEET NO. 12

PHOTOGRAPHIC REFERENCE TARGET LOCATIONS

	Test Vehicle:
	2008 BMW X5
	NHTSA No.:
	M80504

	Test Program:
	35mph Frontal Impact
	Test Date:
	12/19/2007


[image: image24.wmf] 

Upright

 

Position

 

Adjuster

 

Inclinometer

 

Seat

 

Back

 

Angle

 

(Degrees)

 

FRONT SEAT ASSEMBLY

 

Seat Cushion

 


	Item
	Value

	A
	421

	B
	952

	C
	2427

	D
	915

	E
	1478

	F
	280

	G
	

	H
	745

	I
	1406

	J
	914

	K
	911

	L
	1589

	M
	730

	N
	1579

	O
	915

	P
	914

	Q
	1412


DATA SHEET NO. 13
VEHICLE INTRUSION MEASUREMENTS

	Test Vehicle:
	2008 BMW X5
	NHTSA No.:
	M80504

	Test Program:
	35mph Frontal Impact
	Test Date:
	12/19/2007


DOOR OPENING WIDTH

	Item
	Description
	Units
	Pre-Test
	Post-Test
	Difference

	A
	Left Side Upper
	mm
	974
	971
	3

	B
	Left Side Lower
	mm
	938
	938
	0

	D
	Right Side Upper
	mm
	976
	973
	3

	E
	Right Side Lower
	mm
	939
	939
	0


WHEELBASE MEASUREMENTS

	Item
	Description
	Units
	Pre-Test
	Post-Test
	Difference

	C
	Left Side Wheelbase
	mm
	2930
	2830
	100

	F
	Right Side Wheelbase
	mm
	2930
	2824
	106



[image: image13.wmf] 

C

 

B

 

A

 

 

F

 

E

 

D

 


DATA SHEET NO. 13... (continued)

VEHICLE INTRUSION MEASUREMENTS

	Test Vehicle:
	2008 BMW X5
	NHTSA No.:
	M80504

	Test Program:
	35mph Frontal Impact
	Test Date:
	12/19/2007


DRIVER COMPARTMENT INTRUSION

	Item
	Description
	Units
	Pre-Test
	Post-Test
	Difference

	AB
	Door Opening (Inside window jam)
	mm
	777
	777
	0

	CX
	Left Knee Bolster to X
	mm
	302
	296
	6

	DX
	Right Knee Bolster to X
	mm
	281
	279
	2

	EX
	Brake Pedal to X
	mm
	569
	560
	9

	FX
	Foot Rest to X
	mm
	611
	603
	8

	GX
	Center of Steering Column Wheel Hub to X
	mm
	76
	117
	-41


X = Front of Seat Track (stationary)


[image: image14.wmf] 

 A

 

 B

 

 X

 

G

 

 FX

 

 AB

 

 CX

 

 DX

 

 EX

 

GX

 


DRIVER COMPARTMENT  

 DATA SHEET NO. 13... (continued)

VEHICLE INTRUSION MEASUREMENTS

	Test Vehicle:
	2008 BMW X5
	NHTSA No.:
	M80504

	Test Program:
	35mph Frontal Impact
	Test Date:
	12/19/2007


[image: image25.wmf] 

A

 

D

 

E

 

B

 

F

 

C

 

Lower Edge of Protected Zone

 

FRONT VIEW OF WINDSHIELD

 

Zero Point

 

Y

 

X

 


UNDERBODY FLOORBOARD DEFORMATION

	Measurement
	Pre-Test
	Post-Test
	Difference

	A
	270
	265
	5

	B
	298
	298
	0

	C
	283
	280
	3

	D
	268
	266
	2

	E
	424
	424
	0

	F
	240
	240
	0

	G
	266
	266
	0

	H
	430
	430
	0


DATA SHEET NO. 14

LOAD CELL LOCATIONS ON FIXED BARRIER 

	Test Vehicle:
	2008 BMW X5
	NHTSA No.:
	M80504

	Test Program:
	35mph Frontal Impact
	Test Date:
	12/19/2007



[image: image15.wmf] 

30 Load Cell Rigid Barrier

 

Load Cell Locations on Fixed Barrier

 

Plywood Face on Steel Frame

 

 153

 

Ground Surface

 

Group 4

 

D1

-

D5

 

6 Groups of 5 Load Cells Each

 

2143

 

Group 5

 

E1

-

E5

 

Group 6

 

F1

-

F5

 

Group 1

 

A1

-

A5

 

Group 2

 

B1

-

B5

 

Group 3

 

C1

-

C5

 

 

818

 

D5

 

D4

 

D

3

 

D2

 

D1

 

E3

 

E4

 

E1

 

E5

 

E2

 

F4

 

F3

 

F5

 

F2

 

F1

 

A1

 

A3

 

A4

 

A5

 

A2

 

B3

 

B5

 

B4

 

B1

 

B2

 

C4

 

C3

 

C5

 

C1

 

C2

 

19

 

2143

 

766

 

611

 

766

 

407

 

984

 


DATA SHEET NO. 15

ACCIDENT INVESTIGATION DIVISION DATA

	Test Vehicle:
	2008 BMW X5
	NHTSA No.:
	M80504

	Test Program:
	35mph Frontal Impact
	Test Date:
	12/19/2007


VEHICLE INFORMATION

	VIN:
	5UXFE43568L002144
	Wheelbase (mm) :
	2930

	Vehicle Size Category:
	SUV
	Test Weight (kg) :
	2457.6


ACCELEROMETER DATA

Accelerometer Locations:   As per measurements on Page 12                                                               

Cal. Procedure/Interval:     MGA procedure / 6 month
Integration Algorithm: 
       Trapezoidal

Linearity:  > 99%
Impact Velocity (km/h):      56.3
Velocity Change (km/h):    63.0


Time of Separation (msec):  
101
CRUSH PROFILE

Collision Deformation Classification:  Frontal   
Midpoint of Damage: Centerline 
Damage Region Length (mm): 
1262

Impact Mode:  Frontal
	No.
	 Measurement Description
	Units
	Pre-Test
	Post-Test
	Difference

	C1
	 Crush zone 1 at left side
	mm
	4702
	4136
	566

	C2
	 Crush zone 2 at left side
	mm
	4781
	4218
	563

	C3
	 Crush zone 3 at left side
	mm
	4814
	4217
	597

	C4
	 Crush zone 4 at right side
	mm
	4815
	4254
	561

	C5
	 Crush zone 5 at right side
	mm
	4781
	4261
	520

	C6
	 Crush zone 6 at right side
	mm
	4703
	4235
	468

	L
	 C1 TO C6
	mm
	1262
	1253
	9


DATA SHEET NO. 16

DUMMY / VEHICLE TEMPERATURE STABILIZATION CHART

	Test Vehicle:
	2008 BMW X5
	NHTSA No.:
	M80504

	Test Program:
	35mph Frontal Impact
	Test Date:
	12/19/2007


[image: image26.wmf] 

I

 

J

 

K

 

C

 

F

 

D

 

E

 

A

 

B

 

B

 

H

 

M

 

G

 

L

 

N

 

O

 

P

 

Q

 

[image: image27.wmf]Vapor Volume

Unusable Capacity

Filler Neck Volume

Filler Cap

VEHICLE FUEL TANK ASSEMBLY

Fuel Line

Usable

Capacity

[image: image16.jpg]:\\
N
%g;
§§‘\
i

:
i
,,,,,’,’"'17"’/,,'?%%
Vi L ==
3 Illlll[;,f,;"'l,’,"? ;;/of{"%;—
1 /i Il/ll’ 7% S Ses==
llllfl "Ilf"lfz 5 "?:?'%?o:;‘a E“E‘E‘E
'.'.',','.'I'"/"';%--é* »
/] /] 177% 550K, o ==
il i i /f ..'u'%o‘: s ‘:“‘=='i===== A
TH "'ln'".'-'n'f:’o‘:v‘,'5‘-..'=='=-'~ D
“““\'\‘“ﬂium-i#!ii:;:!-’::’,’,%-z:::f&?:‘?«?:‘?“-!:s5.=,=.-'.~' N
LR T et '!--:'::.-':'s’@&\\\w%'ﬂ:’:.-:i \
A ‘\ "“h ‘I NG ,.E.g..‘ﬁé*;\*\‘\ A ‘Iﬂlnl’# O K AN {
i \ ‘\‘\“\\mu“ --iﬁii"’ st 3 AW
T ““‘\\‘\“““»‘"-',"-i,%aa%%% § S
Y 5 S g =="“‘"\“‘ 0 A
S S Dt §55==x=§=====‘m‘:"p N
\ L ':3""3;5"55:_5_'_55'1?“ ] i | A J\ ¥ \v\f"\./
i =\‘}.“%\}s‘s§§§i:.-'iii:=-..' T e :
N s kgl
e ) uitlsint
0‘.: L) / / 11.1 LA
& i il }H
,—fi" ’!}i e w
[1] 3
HHT

R
Vo W

‘W\ ‘ “““z‘ "’
: \f .H= =‘l“




A = Dummies installed in vehicle at 6:00 am

B = Test conducted at 1:39 pm
� EMBED Word.Picture.8  ���








� EMBED Word.Picture.8  ���





� EMBED Word.Picture.8  ���





� EMBED Word.Picture.8  ���








� EMBED Word.Picture.8  ���





Driver





Passenger























1
iii

[image: image28.png]Treadwear, traction
and temperature grades

Trepy
compagition
used

USS. DOT satety

standard code Passenger

car tire
Width of tire
meters

Ratio of height to
‘width (aepect ratio)

Load index &
speed symbol
Diameter of

wheel ininches Rainl




[image: image29.wmf]LEFT SIDE VIEW

STEERING COLUMN ASSEMBLY

1

2

3

_1062403479.doc
[image: image1.bmp]













Z











X











3/4















4







5/6















7











3































2/9























6







1/2/8/9











5











Y















1/8















































7
















_1062412109.doc
[image: image1.bmp]

Rear View







REAR BUMPER







Filler Cap







180o












_1062412129.doc
[image: image1.bmp]

Rear View







REAR BUMPER







Filler Cap







0o/360o












_1062413578.doc














C



















































































D



























A	































F















































B







E












_1062418366.doc


30 Load Cell Rigid Barrier







Load Cell Locations on Fixed Barrier







407







2143











Plywood Face on Steel Frame







19











A1







F1







F2







F5







F3







F4







E2







E5







E1







E4







E3







D1







D2







D3







D4







D5







2143







818







984















766







611







766







C2







C1







C5







C3







C4







B2







B1







B4







B5







B3







A2







A5







A4







A3







 153







Ground Surface







Group 4



D1-D5



























Group 3



C1-C5











Group 2



B1-B5







Group 1



A1-A5







Group 6



F1-F5











Group 5



E1-E5







6 Groups of 5 Load Cells Each












_1208871312.unknown

_1062413874.doc


GX







 EX







 DX







 CX







 AB







 FX







G







 X



















 B







 A












_1062412741.unknown

_1062413357.doc
[image: image1.bmp][image: image2.bmp]













F











G











N















K







J







I



















Q



















B







C







A







O























L







E



















H







B











M















































D







P












_1062412120.doc
[image: image1.bmp]

Rear View







R



E



A



R



B



U



M



P



E



R







Filler Cap







270o












_1062412061.doc
[image: image1.bmp]

Rear View







R



E



A



R



B



U



M



P



E



R







Filler Cap







90o












_1062412071.doc
[image: image1.bmp]

Rear View







REAR BUMPER







Filler Cap







180o












_1062412043.doc
[image: image1.bmp]

Rear View







REAR BUMPER







Filler Cap







0o/360o












_1062410801.doc


D







A







E







B







F







C







Lower Edge of Protected Zone







FRONT VIEW OF WINDSHIELD







Zero Point







Y







X












_1062398800.doc
[image: image1.png]






_1062402346.doc
 





Dummy’s Centerline







TBI







'D' Ring







Shoulder Belt Portion















Lap Belt Portion











1/8" Thick Aluminum Plate















Locking Retractor















Outboard Anchor











Inboard Anchorage











Floorpan







Buckle Assembly



















PBU







PBL
















_1062397815.doc


Seat Cushion







Upright



Position







Adjuster







Inclinometer







Seat



Back







Angle



(Degrees)











FRONT SEAT ASSEMBLY












_1062398663.doc


CD	Chest to Dash



CS	Chest to Steering Wheel Hub



HH	Head to Header



HW	Head to Windshield



HZ	Head to Roof



KDA	Knee to Dash Angle



KDL	Left Knee to Dash



KDR	Right Knee to Dash



NA	Nose to Rim Angle



NR	Nose to Rim



PA	Pelvic Angle



RA	Rim to Abdomen



SA	Seat Back Angle



SCA	Steering Column Angle



SH	Striker to H-Point



SK	Striker to Knee



ST	Striker to Head



SWA	Steering Wheel Angle



TA	Tibial Angle



WA	Windshield Angle







 HW







HZ







 PA(







 SA(







 TA(







 KDA(







 SCA(







 NA(







 WA(







  HH







 SWA(







AD	Arm to Door



HD	H-Point to Door



HR	Head to Side Header



HS	Head to Side Window



KK	Knee to Knee



SHY	Striker to H-Point



          (Y Axis)











ST







SK







NR







SH







Seat Back Angle Line







CS







KD







CD







STRIKER







SHY







HR







HD







AD







KK







HS















Vertical



Transverse



Plane







Vertical Longitudinal Planes







DRVR







PASS












_984601894.doc


LEFT SIDE VIEW







STEERING COLUMN ASSEMBLY







1







2







3












_986225396.doc


Usable



Capacity











Vapor Volume







Unusable Capacity







Filler Neck Volume







Filler Cap







VEHICLE FUEL TANK ASSEMBLY







Fuel Line












