# Front Seat Modeling in Rear Impact Crashes

Development of Detailed Finite Element Model for Seat Back Strength Requirements

| Prepared For: Sanjay Patel Department of Transportation National Highway Traffic Safety Administration Structural and Restraints Research Division (NSR-210) W46-445 1200 New Jersey Avenue, S.E., Washington, DC 20590 Janella Davis National Highway Traffic Safety Administration Office of Acquisition Management (NFO-300) W51-115 1200 New Jersey Avenue, S.E., Washington, DC 20590 | Principle Investigator: Wade Bridges EDAG, Inc. 1875 Research Dr. Troy, MI 48083 Phone: 248 786 1815 Fax: 248 588 3259 wade.bridges@edag-us.com Contributing Authors: Velayudham Ganesan, EDAG, Inc. Praneshkumar Jayakumar, EDAG, Inc. James Davies; EDAG, Inc. Mahendran Paramasuwom, EDAG, Inc. |
| --- | --- |

Program Reference:	DOT Contract DTNH22-15-D-00006 / DTNH22-17-F-00118

Contract Prime:           EDAG, Inc.

Distribution:		Unlimited

Report – October 29, 2018 REVISED

Disclaimer

This report is furnished to the U.S. Department of Transportation (DOT), and under the terms of DOT contract DTNH22-15-D-00006 / DTNH22-17-F-00118 DOT and EDAG, Inc. This research was funded by the Department of Transportation, National Highway Traffic Safety Administration; Structures and Restraints Research Division. The views and conclusions contained in this document are those of the authors and should not be interpreted as representing the official policies, either expressed or implied, of the National Highway Traffic Safety Administration; Structures and Restraints Research Division, or the U.S. Government. The opinions, findings and conclusions expressed in this report are those of the authors and not necessarily those of EDAG, Inc. Publication of this report by EDAG, Inc. should not be considered an endorsement by EDAG, Inc., or the accuracy or validity of any opinions, findings or conclusions expressed herein.

In publishing this report, EDAG, Inc. make no warranty or representation, expressed or implied, with respect to the accuracy, completeness, usefulness, or fitness for purpose of the information contained herein, or that the use of any information, method, process, or apparatus disclosed in this report may not infringe on privately owned rights. EDAG, Inc. assume no liability with respect to the use of, or for damages resulting from the use of, any information method, process, or apparatus described in this report.

Contents

1	Executive Summary	6

2	Introduction and Scope of Work	7

2.1	Introduction	7

2.2	Program Tasks Summary	8

3	Baseline Seat Simulation	8

3.1	Baseline Seat Choice	8

3.2	Baseline Seat Tests	9

3.2.1	Seat Back Pull Test – Manual Seat	10

3.2.2	Seat Back Pull Test – Power Seat	12

3.2.3	Seat FE Model Development	14

3.2.4	Manual Seat – Seat Back Pull Test Correlation	16

3.2.5	Power Seat – Seat Back Pull Test Correlation	20

4	FMVSS 301 High Speed Rear Impact Sled Test	24

5	FMVSS 301 Model Development	30

5.1	FMVSS 301 FEM Development	30

5.2	FMVSS 301 Sled Test CAE, Test Comparison	32

5.2.1	Manual Seat – Sled Test Comparison	32

5.2.2	Power Seat – Sled Test Comparison	35

6	Seat Back Strength Requirements Study	38

6.1	FMVSS 301 Simulations with Rear Seat Passenger	38

6.1.1	Manual Seat – FMVSS 301 Simulation with Rear Seat Passenger	39

6.1.2	Power Seat – FMVSS 301 Simulation with Rear Seat Passenger	40

7	Seat Back Strength Improvements	42

7.1	Countermeasures and Design Changes	42

7.2	Updated Manual Seat – FMVSS 301 Simulations with Rear Seat Passenger	43

7.3	Updated Power Seat – FMVSS 301 Simulations with Rear Seat Passenger	47

7.4	Countermeasure Validation for Low Speed Rear Impact	51

7.5	Cost Estimation	56

7.5.1	Cost Estimate – Manual Seat	56

7.5.2	Cost Estimate – Power Seat	57

8	Conclusion and Recommendations	58

8.1	Summary of Project Results	59

8.2	Recommendations	61

9	Appendix A	63

LIST OF FIGURES

Figure 1: Seat Assemblies	9

Figure 2: Schematic Diagram of the Seat Back Pull Loading	10

Figure 3: Seat Back Pull Test Seat Setup – Manual Seat	11

Figure 4: Seat Back Pull Test, Post Test – Manual Seat	12

Figure 5: Seat Back Pull Test, Post Test, Seat Back Frame Deformation – Manual Seat	12

Figure 6: Seat Back Pull Test Seat up – Power Seat	13

Figure 7: Seat Back Pull Test, Post Test – Power Seat	14

Figure 8: Seat Back Pull Test, Post Test, Seat Back Frame Deformation – Power Seat	14

Figure 9: Scanned Seat Models	15

Figure 10: MY 2014 Detailed FE Models	16

Figure 11: Manual Seat FE Model Setup, Loading and Boundary Conditions	17

Figure 12: Loading Curve – Manual Seat	17

Figure 13: Global Deformation of the Seat Frame – Manual Seat	18

Figure 14: Deformation of LHS Seat Frame – Manual Seat	18

Figure 15: Deformation of RHS Seat Frame – Manual Seat	19

Figure 16: Manual Seat Deformation at Failure Area Similar to Test	19

Figure 17: Force vs. Displacement Curves – Manual Seat	20

Figure 18: Loading Curve – Power Seat.	21

Figure 19: Global Deformation the Seat Frame – Power Seat	21

Figure 20: Deformation of LHS Seat Frame – Power Seat	22

Figure 21: Deformation of RHS Seat Frame – Power Seat	22

Figure 22: Power Seat Deformation at Failure Area Similar to Test	23

Figure 23: Force vs. Displacement Curves – Power Seat	24

Figure 24: FMVSS 301 Rear Impact Vehicle Pulse (CAE Simulation)	25

Figure 25: FMVSS 301 Sled Pulse (Generic, 20G Pulse)	26

Figure 26: FMVSS 301 Sled Test Setup	27

Figure 27: FMVSS301 Sled Test – Manual Seat	28

Figure 28: FMVSS301 Sled Test – Power Seat	28

Figure 29: Seat Back Dynamic Deflection - Manual Seat	29

Figure 30: Seat Back Dynamic Deflection - Power Seat	30

Figure 31: Seat FEM –Mechanism and Cushions (Ex. Manual Seat)	31

Figure 32: Manual and Power Seat Models with Deformed Cushions	31

Figure 33: FMVSS301 Sled Test FEA and Test – Manual Seat	33

Figure 34: Post Test Seat Back Rotation	34

Figure 35: Head Acceleration	34

Figure 36: Neck Forces	35

Figure 37: FMVSS301 Sled Test FEA and Test – Power Seat	36

Figure 38: Post Test Seat Back Rotation	37

Figure 39: Head Acceleration	37

Figure 40: Neck Forces	38

Figure 41: FMVSS 301 FE Model with Rear Seat Occupant Dummy – Manual Seat	39

Figure 42: Front Seat Back and Rear Seat Occupant Interaction – Manual Seat	40

Figure 43: FMVSS 301 FE Model with Rear Seat Occupant Dummy – Power Seat	41

Figure 44: Front Seat Back and Rear Seat Occupant Interaction – Power Seat	41

Figure 45: Seat Bottom Countermeasure (Manual and Power Seats)	43

Figure 46: Seat back Rotation Comparison (Manual Seat)	44

Figure 47: Seat back Angle (Baseline – Manual Seat)	44

Figure 48: Seat back Angle (Countermeasure – Manual Seat)	45

Figure 49: Seat back to Knee clearance (Baseline and Countermeasure – Manual Seat)	45

Figure 50: Head rest clearance (Baseline and Countermeasure – Manual Seat)	46

Figure 51: Knee Impact Force Baseline and Countermeasure (Manual Seat)	46

Figure 52: Head Acceleration Baseline and Countermeasure (Manual Seat)	47

Figure 53: Seat back Rotation Comparison (Power Seat)	48

Figure 54: Seat back Angle (Baseline – Power Seat)	48

Figure 55: Seat back Angle (Countermeasure – Power Seat)	49

Figure 56: Seat back to Knee clearance (Baseline and Countermeasure – Power Seat)	49

Figure 57: Head rest clearance (Baseline and Countermeasure – Power Seat)	50

Figure 58: Knee Impact Force Baseline and Countermeasure (Power Seat)	50

Figure 59: Head Acceleration Baseline and Countermeasure (Power Seat)	51

Figure 60: IIHS Vehicle Seat Head Restraint Test – Baseline (Manual Seat)	52

Figure 61: IIHS Vehicle Seat Head Restraint Test – Baseline vs. Test (Manual Seat)	52

Figure 62: IIHS Vehicle Seat Head Restraint Test – Neck Injury Rating, Baseline vs. Test	53

Figure 63: IIHS Vehicle Seat Head Restraint Test – Baseline (Power Seat)	53

Figure 64: IIHS Vehicle Seat Head Restraint Test – Neck Injury Rating, Baseline (Power Seat)	54

Figure 65: IIHS Neck Injury Rating Chart – Countermeasure vs. Baseline (Manual Seat)	55

Figure 66: IIHS Neck Injury Rating Chart – Countermeasure vs. Baseline (Power Seat)	55

Figure 67: Countermeasure Parts and Cost Estimate (Manual Seat)	57

Figure 68: Countermeasure Parts and Cost Estimate (Power Seat)	58

Figure 69: Illustration of Front Seat Interaction on 5th %ile Rear Seat Occupant	61

Figure 70: Illustration of Front Seat Interaction on 90th %ile Rear Seat Occupant	62

Figure 71: Illustration of Front Seat Interaction on Child Rear Seat Occupant	62

LIST OF TABLES

Table 1: Seat Back Pull Test Load Profile – Manual Seat	10

Table 2: Seat Back Pull Test Seat Position – Manual Seat	11

Table 3: Seat Back Pull Test Load Profile – Power Seat	13

Table 4: FMVSS 301 Sled Test FE Simulations and Test Comparison – Manual Seat	35

Table 5: FMVSS 301 Sled Test FE Simulations and Test Comparison – Power Seat	38

Table 6: Seat back strength improvement targets	42

Table 7: Countermeasure improvement measures (Manual Seat)	47

Table 8: Countermeasure improvement measures (Power Seat)	51

Table 9: Cost Impact of Countermeasure Seats	58