DUMMY POSITIONING

CRS: Evenflo Chase – Forward Facing – LATCH

TRC Test Number: S211019-1	VRTC Test Number: SIDE_383

DUMMY POSITIONING FARO MEASUREMENTS

| Description | Units | X | Y | Z |
| --- | --- | --- | --- | --- |
| Origin Post | mm | -0.50 | -0.31 | 0.15 |
| Front of Sliding Seat | mm | -154.82 | 672.75 | 576.28 |
| Sliding Seat Corner | mm | -162.70 | 373.03 | 561.15 |
| Point S1 | mm | -148.29 | 392.29 | 622.01 |
| Point S2 | mm | -150.22 | 1050.70 | 625.55 |
| Honeycomb Corner | mm | -324.08 | 95.67 | 575.39 |
| Z point | mm | -771.28 | 1070.92 | 685.77 |
| Back of Sliding Seat | mm | -893.96 | 978.56 | 5.08 |
| CRS top of the seat | Mm | -837.34 | 671.25 | -69.64 |
| CRS top of the headrest | mm | -833.12 | 669.97 | -108.78 |
| CRS bottom Center | mm | -243.66 | 670.28 | 421.31 |
| Top of Head | mm | -654.38 | 670.47 | 29.26 |
| Front of Face | mm | -573.14 | 671.52 | 79.98 |
| Head CG | mm | -634.51 | 608.77 | 97.17 |
| Shoulder | mm | -627.09 | 565.20 | 219.88 |
| Chest Clip | mm | -555.75 | 670.47 | 197.33 |
| Buckle | mm | -452.27 | 669.10 | 287.48 |
| Right Knee | mm | -373.69 | 743.28 | 350.18 |
| Right Ankle | mm | -240.67 | 747.59 | 285.37 |
| Left Knee | mm | -368.31 | 599.28 | 353.60 |
| Left Ankle | mm | -239.64 | 604.90 | 291.63 |

DUMMY POSITIONING FARO MEASUREMENTS, CONTINUED

| Description | Units | X | Y | Z |
| --- | --- | --- | --- | --- |
| CRS 1 | mm | -766.63 | 546.71 | 105.71 |
| CRS 4 | mm | -670.02 | 541.85 | 358.02 |
| CRS 2 | mm | -524.40 | 506.26 | 419.77 |
| CRS 3 | mm | -378.16 | 454.91 | 388.50 |